3GPP TSG-RAN WG4 Meeting #84bis	R4-1711588

Dubrovnik, Croatia, 9 - 13 Oct, 2017

Agenda item:	8.14.1

Source:	Dish Network

Title:	TP for TR36.761: Extended-Band12 new E-UTRA Band Deployment scenarios

Document for:	Approval

# 1	Introduction

As part of the WI proposal agreed in RAN77 [1] it was agreed to have a Part 1 study phase included in the work item.  This document looks at the various deployment scenarios with and without the 1MHz guard band (728-729 MHz) between E block and A block.  A TP is provided for the TR in Annex A.

# 2	WI Background

The objective of the WI [1] are copied below:

The work item consists of two parts.

Part 1 would be a study phase to look at the co-existence impacts of the removal of the 1MHz guard between E block and A block (728-729MHz). Following aspects would be considered;

- Impact of interference on E Block performance from increased carriers in the A block in various deployment scenarios. Compare the performance impact with and without the 1 MHz guard band.

- Impact of interference from E block to A block receivers

- Address potential BS and UE coexistence issues

The outcome of the study phase would be captured in the TR for this band.  If needed the objectives of the WID would need to be revised based in the outcome of Part 1 study phase

Part 2 would be the reminder of the Work Item.  Those objectives dependent upon the study phase cannot begin until the completion of Part 1. The following are the Part 2 core objectives:

- Standardize a new FDD E-UTRA operating band based on the 36 MHz paired spectrum consists of E-UTRA Band 12 and the 1 MHz “unused” guard bands on the uplink and the downlink immediately below Band 12. To be specific, the uplink band for this new band is 18 MHz, 698-716 MHz; the downlink band is 18 MHz, 728-746 MHz; the duplex gap is 12 MHz, 716-728 MHz Refer to Figure 2.

  - Support for 5 and 10MHz bandwidths

Figure 2

- Specify band numbering and RF characteristics of the new band

- Address potential BS and UE coexistence issues taking into account the outcome of the Part 1 study phase

- Ensure the new band supports NB-IoT UE and category M1 UE.

- Update the related 3GPP E-UTRA technical specifications to include support for the new band

- Specify requirements for the new band such that B12 can also be supported, on a fully interoperable basis.

- Future requirements for Carrier Aggregation combinations with the new band shall guarantee all combinations with B12 can also be supported. Carrier Aggregation is not part this WI.

The Study Phase of the Work Item will begin in RAN4 #84bis. No Part 2 work may begin before RAN4 #85. The WID will not be considered complete until Part 1 and Part 2 are finished.

# General

The out-of-band spurious emissions from the adjacent channel represent an increase in interference power into the E block receiver.  Without a 1 MHz guard band, the adjacent out of bands spurious emissions or interferer is expected to be higher into the E block.

The addition of up to 4 NB-IoT BS carriers in the guard band has potential to further increase the out-of-band spurious emissions into the E block.

## 3.1	Deployment scenarios

The out-of-band spurious emissions from the adjacent channel represent an increase in interference power into the E block receiver.  Without a 1 MHz guard band, the adjacent out of bands spurious emissions or interferer is expected to be higher into the E block.

The following deployment scenarios are proposed to study the impact of the increased out-of-band emissions in the adjacent E block channel with and without the current 1 MHz guard band (728-729MHz).

Figure 3.1-1 Deployment scenarios

The three scenarios being proposed are:

- Scenario 1 is the current B12 deployment scenario and is proposed to be the reference case i.e. with 1 MHz guard band

- Scenario 2 is a possible B12+ deployment scenario where 4 NB-IoT carriers are deployed adjacent to B block

- Scenario 3 is a possible B12+ deployment scenario where up to 4 NB-IoT carriers are deployed adjacent to E block

According to TS 36.104, the maximum number of NB-IoT carriers that can be placed within 1MHz is 4 because the FOFFSET from the center of outermost carriers is 200kHz.

The E block can be occupied by a supplementary DL (SDL) or broadcast services (eMBMS/ATSC).  The impact on E block performance due to increased emissions from A block shall be evaluated with and without 1 MHz guard band as emissions difference in dB over both the first 1 MHz (727-728 MHz) and the whole 6 MHz (722-728MHz) of E block. Companies are encouraged to further evaluate the impact of emissions difference to other parameters such as coverage and throughput.

It is proposed that the above 3 scenarios are considered in the Part 1 study phase.

# References

# [1]   RP-172111 “New WI: Extended-Band12 new E-UTRA Band”, T-Mobile USA, US Cellular, and C-Spire

# Annex A  		TP for TR36.716

## 5.1	Deployment scenarios

The out-of-band spurious emissions from the adjacent channel represent an increase in interference power into the E block receiver.  Without a 1 MHz guard band, the adjacent out of bands spurious emissions or interferer is expected to be higher into the E block.

The following deployment scenarios are used to study the impact of the increased out-of-band emissions in the adjacent E block channel with and without the current 1 MHz guard band (728-729MHz).

Figure 3.1-1 Deployment scenarios

The three scenarios being are:

- Scenario 1 is the current B12 deployment scenario and is proposed to be the reference case i.e. with 1 MHz guard band

- Scenario 2 is a possible B12+ deployment scenario where 4 NB-IoT carriers are deployed adjacent to B block

- Scenario 3 is a possible B12+ deployment scenario where up to 4 NB-IoT carriers are deployed adjacent to E block

According to TS 36.104, the maximum number of NB-IoT carriers that can be placed within 1MHz is 4 because the FOFFSET from the center of outermost carriers is 200kHz.

The E block can be occupied by a supplementary DL (SDL) or broadcast services (eMBMS/ATSC).  The impact on E block performance due to increased emissions from A block shall be evaluated with and without 1 MHz guard band as emissions difference in dB over both the first 1 MHz (727-728 MHz) and the whole 6 MHz (722-728MHz) of E block. Companies are encouraged to further evaluate the impact of emissions difference to other parameters such as coverage and throughput.