TSG-RAN Working Group 4 (Radio) meeting #84bis	R4-1711739

Dubrovnik, Croatia; 9 - 13 October, 2017

Source:	Nokia, Nokia Shanghai Bell

Title:	TP to 38.817-02: Requirements for contiguous and non-contiguous spectrum

Agenda item:	9.5.1.1

Document for:	Approval

# Background

This contribution provides the text proposal on requirements for contiguous and non-contiguous spectrum.

# Text proposal

## 5.7	Requirements for contiguous and non-contiguous spectrum

Requirements for contiguous and non-contiguous spectrum are well defined in existing single-RAT and MSR specifications. Both contiguous and non-contiguous operation has been requested in various NR-LTE CA/DC combinations and is considered to be completed within Rel’15 timeframe.

The text to Technical Specification for requirements for contiguous and non-contiguous spectrum Clause is as follows:

[A spectrum allocation where an BS operates can either be contiguous or non-contiguous. Unless otherwise stated, the requirements in the present specification apply for BS configured for both contiguous spectrum operation and non-contiguous spectrum operation.

For BS operation in non-contiguous spectrum, some requirements apply both at the Base Station RF Bandwidth edges and inside the sub-block gaps. For each such requirement, it is stated how the limits apply relative to the Base Station RF Bandwidth edges and the sub-block edges respectively.]