3GPP TSG-RAN WG4 Meeting #84bis		R4-1711822

# Dubrovnik, Croatia, 9 - 13 Oct, 2017

Agenda Item:		9.8.1

Title:	An Auxiliary Observation System that Enables Measurements to be made in Non-Anechoic Environments

Source:		Fraunhofer HHI

Document for:	Discussion

# Introduction

RAN WG4 is currently defining New Radio (NR) baseline setups for UE RF, UE RRM and UE demodulation testing [1]. In addition to the definition of baselines, alternative test methods can be proposed to offer potential solutions for advanced or more sophisticated NR measurement test methods and procedures.

This paper discusses an approach that supports baseline test setups—both those that exist and those that are under development. The approach promotes measurements in non-anechoic environments [2] and thus removes any restriction normally associated with the dimensions of the device under test (DUT). Furthermore, the method also finds application in multi-station production test environments in which interference effects are reduced using advanced signal processing techniques.

Observation 1: 	The auxiliary observation system finds application in multi-station production test environments in which interference effects are reduced using advanced signal processing techniques.

# Rationale

## Measurements in non-anechoic environments

Anechoic chambers have been used for decades to perform over-the-air (OTA) or radiated RF measurements [3]. Considering a non-anechoic environment to perform the over-the-air (OTA) measurements imposes two major issues that must be addressed:

Firstly, a shielded room is needed to eliminate or at least to substantially reduce the effects of external interference. Secondly, radiation-absorbent material (RAM) is needed to eliminate or at least to substantially reduce the strength of reflected electromagnetic waves that would otherwise create multipath effects that could affect the quality of the measurement.

OTA measurements can however be made in a non-anechoic environment [2] by considering a) the measurement is preceded by a calibration phase b) unpredicted channel variations are observed and compensated for. The calibration phase is then followed by an active OTA measurement phase throughout which the calibration must remain valid and, moreover, no sporadic disturbances shall occur. Such tests made in non-anechoic and (quasi)-non-stationary environments necessitate the checking of calibration validity and the observation of environmental disturbances.

## An auxiliary observation system that enables measurements in non-anechoic environments

The auxiliary observation system—an augmentation of the ordinary OTA setup—detects channel variations and in-band disturbances during an OTA measurement. In other words, the OTA measurement setup calibrates the current multipath environment prior to performing an OTA measurement. During the OTA measurement, all changes and disturbances to the channel that fall within the measurement frequency range are detected and recorded by the auxiliary antenna array.

Figure 1 Block diagram showing the role that the auxiliary observation system plays in non-anechoic environment implementations.

In Figure 1, an auxiliary observation system detects environmental perturbations and triggers the UE (DUT) via a test interface (TI) to perform a recalibration procedure (i.e., to retrieve the channel matrix) [4]. The auxiliary observation system is supported by self-interference cancellation mechanisms [5 and 6] and can detect environmental changes. The auxiliary observation system stays active all the time to trigger the recalibration procedure based on specific criteria (i.e., significate multipath channel perturbation) and reduces the test interface communication significantly.

Observation 2: 	The auxiliary observation system allows recalibration procedures to be triggered based on defined environmental perturbation criteria.

Observation 3: 	The auxiliary observation system reduces the test interface communication significantly.

In other words, the auxiliary observation system measures and monitors the aggregated interference level to ensure that it is be below a defined threshold and thus guarantees the validity of measurement procedures.

Observation 4: 	The auxiliary observation system measures and monitors the aggregated interference level to ensure that it is be below a defined threshold and thus guarantees the validity of measurement procedures.

# Conclusion

Observation 1: 	The auxiliary observation system finds application in multi-station production test environments in which interference effects are reduced using advanced signal processing techniques.

Observation 2: 	The auxiliary observation system allows recalibration procedures to be triggered based on defined environmental perturbation criteria.

Observation 3: 	The auxiliary observation system reduces the test interface communication significantly.

Observation 4: 	The auxiliary observation system measures and monitors the aggregated interference level to ensure that it is be below a defined threshold and thus guarantees the validity of measurement procedures.

# References

- RP-171021, “New SID on Study on test methods for New Radio”, Intel Corporation, CATR, Qualcomm Incorporated, Mediatek.

- R4-1709832, “Measurements in Non-Anechoic Environments”, Fraunhofer HHI, Fraunhofer IIS.

- C. A. Balanis, Antenna Theory: Analysis and Design, 4th Ed., Wiley, 2016.

- R4-1708202, "NR UE test interface functions”, ROHDE & SCHWARZ, Azimuth Systems Incorporated, Anritsu, Keysight Technologies, CATR, Fraunhofer HHI.

- R. Askar, T. Kaiser, B. Schubert, T. Haustein und W. Keusgen, “Active Self-Interference Cancellation Mechanism for Full-Duplex Wireless Transceivers,” in Cognitive Radio Oriented Wireless Networks and Communications (CROWNCOM), 2014 9th International Conference on, 2014.

- R. Askar, B. Schubert, W. Keusgen und T. Haustein, “Agile Full-Duplex Transceiver: The Concept and Self-Interference Channel Characteristics, “ in European Wireless 2016; 22th European Wireless Conference, 2016.