3GPP TSG-RAN WG4 RAN4-84Bis	R4-1711830

Dubrovnik, Croatia, 9-13 October, 2017

Source:		Ericsson

Title:			TP to TS 38.113: Section 3 (Definitions, symbols and abbreviations)

Agenda item:		9.6.1

Document for:	Approval

# Introduction

In this contribution we provide TP to the NR EMC specification TS 38.113 for Section 3 based on the skeleton approved in [1].

# References

R4-1708861, Proposal on the skeleton of  NR BS EMC standard TS 38.113. ZTE, Huawei, Ericsson

R4-1710018, Timeline for NR BS EMC specification TS 38.113. ZTE, Huawei, Ericsson.

R4-1710019, WF on EMC radiated emission and RF spurious emission requirements for NR BS. Ericsson, Huawei, ZTE.

- --------------Start of text proposal-------------

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [x] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [X].

ancillary equipment: electrical or electronic equipment, that is intended to be used with a receiver or transmitter

NOTE: It is considered as an ancillary equipment if:

- the equipment is intended for use with a receiver or transmitter to provide additional operational and/or control features to the radio equipment, (e.g. to extend control to another position or location); and

- the equipment cannot be used on a stand alone basis to provide user functions independently of a receiver or transmitter; and

- the receiver or transmitter, to which it is connected, is capable of providing some intended operation such as transmitting and/or receiving without the ancillary equipment (i.e. it is not a sub-unit of the main equipment essential to the main equipment basic functions).

antenna port: port, for connection of an antenna used for intentional transmission and/or reception of radiated RF

energy

associated equipment: equipment needed to exercise and/or monitor the operation of the EUT

Base Station equipment: Radio and/or ancillary equipment intended for operation at a fixed location and powered directly or indirectly (e.g. via an AC/DC converter or power supply) by AC mains network, or an extended local DC mains network.

Channel bandwidth: The RF bandwidth supporting a single E-UTRA RF carrier with the transmission bandwidth configured in the uplink or downlink of a cell. The channel bandwidth is measured in MHz and is used as a reference for transmitter and receiver RF requirements.

Continuous phenomena (continuous disturbance): Electromagnetic disturbance, the effects of which on a particular device or equipment cannot be resolved into a succession of distinct effects (IEC 60050-161 [8]).

Enclosure port: physical boundary of the equipment through which electromagnetic fields may radiate or impinge

NOTE: In the case of integral antenna equipment, this port is inseparable from the antenna port.

Exclusion band: frequency range(s) not subject to test or assessment

Integral antenna: antenna designed for permanent connection to the equipment and considered part of the enclosure port

NOTE: An integral antenna may be fitted internally or externally.

Lower RF bandwidth edge:  The frequency of the lower edge of the Base Station RF bandwidth, used as a frequency reference point for transmitter and receiver requirements.

Radio communications equipment : Telecommunications equipment which includes one or more transmitters and/or receivers and/or parts thereof for use in a fixed, mobile or portable application. It can be operated with ancillary equipment but if so, is not dependent on it for basic functionality.

Radio equipment: Equipment which contains Radio digital unit and Radio unit.

Radio digital unit: Equipment which contains base band and functionality for controlling Radio unit.

Radio unit: Equipment which contains transmitter and/or receiver.

Receiver exclusion band: The receiver exclusion band is the band of frequencies over which no tests of radiated immunity of a receiver are made. The exclusion band for receivers is expressed relative to the base station receive band.

Signal and control: Port which carries information or control signals, excluding antenna ports.

Signal port: port intended for the interconnection of components of an EUT, or between an EUT and AE and used in

accordance with relevant functional specifications (for example for the maximum length of cable connected to it)

Telecommunication port: Ports which are intended to be connected to telecommunication networks (e.g. public switched telecommunication networks, integrated services digital networks), local area networks (e.g. Ethernet, Token Ring) and similar networks.

Transient phenomena: Pertaining to or designating a phenomena or a quantity which varies between two consecutive steady states during a time interval short compared with the time-scale of interest (IEC 60050-161 [X]).

Transmitter exclusion band: The transmitter exclusion band is the band of frequencies over which no tests of radiated immunity of a transmitter are made. The exclusion band for transmitters is expressed relative to the carrier frequencies used (the carrier frequencies of the base stations activated transmitter(s).)

Upper RF bandwidth edge: The frequency of the upper edge of the Base Station RF bandwidth, used as a frequency reference point for transmitter and receiver requirements

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

β 	Bandwidth

BWChannel	Channel bandwidth

Bandrx(lower) 	Lower edge, in terms of frequency, of the tuning range or allocated band of the receiver under assessment

Bandrx(upper) 	Upper edge, in terms of frequency, of the tuning range or allocated band of the receiver under assessment

BWRX 	          Bandwidth of Receiver (or receiver part of transceiver)

ChWRX 	        Channel Width of Receiver (or receiver part of transceiver)

EXband(lower)    Exclusion band lower frequency edge

EXband(upper)    Exclusion band upper frequency edge

FBW RF,high  	Upper RF bandwidth edge, where FBW RF,high  = FC,high + Foffset, RAT

FBW RF,low  	Lower RF bandwidth edge, where FBW RF,low  = FC,low - Foffset, RAT

FC,high		Center frequency of the highest transmitted/received carrier.

FC,low		Center frequency of the lowest transmitted/received carrier.

Foffset, RAT	Frequency offset from FC,high to the upper  RF bandwidth edge or FC,low to the lower RF bandwidth edge for a specific RAT

foffset	Frequency offset used for discovering narrowband response for receivers

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [X] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [X].

AC	Alternating Current

AMN	Artificial Mains Network

BC	Band Category

BWRX 	Bandwidth of Receiver (or receiver part of transceiver)

CDN	Coupling/Decoupling Network

ChWRX 	Channel Width of Receiver (or receiver part of transceiver)

CS	Capability Set

DC	Direct Current

EMC	Electromagnetic Compatibility

ESD	Electrostatic discharge

EUT	Equipment Under Test

FRC	Fixed Reference Channel

NTC	Test Configuration for Non-contiguous operation

RAT	Radio Access Technology

RF	Radio frequency

rms	root mean square

TC	Test Configuration