3GPP TSG RAN WG4 Meeting #85	R4-1712102

Reno, US, 27 November - 1 December 2017

3GPP TSG RAN WG1 Meeting #90bis			R1-1719039

Prague, CZ, 9th – 13th, October 2017

Title:	LS on UE minimum bandwidth

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN WG 1

To:	RAN WG 4

CC:

Contact Person:

#### Name:	Ren Da

E-mail Address:	renda@catt.com

1. Overall Description:

RAN1 would like to inform RAN4 that during the discussion of the bandwidth of CORESET containing PDCCH scheduling RMSI (remaining minimum system information) and NR-PDSCH carrying RMSI, RAN1 made the following agreement for UE minimum bandwidth in RAN1#90bis:

Agreements: - UE minimum bandwidth in the context of confinement of RMSI and CORESET containing PDCCH scheduling RMSI is defined as the largest bandwidth that all UEs must support regardless of UE capability, which is at least no less than the SS/PBCH bandwidth. - Bandwidth for RMSI and CORESET containing PDCCH scheduling RMSI supports at least the same bandwidth as SS/PBCH (e.g.,[24 PRBs]). - Note: The UE minimum bandwidth will be finally determined by RAN4.

2. Actions:

To RAN WG4:

RAN1 respectfully asks RAN4 whether RAN4 sees any issues with the above agreements, and if so, to provide guidance accordingly.

3. Date of Next RAN1 Meetings:

RAN1#91	27th November – 1st December	Reno, Nevada, USA

RAN1-AH-1801	22 - 26 Jan 2018		Vancouver, Canada