3GPP TSG RAN WG4 Meeting #85	R4-1712106

Reno, US, 27 November - 1 December 2017

3GPP TSG RAN WG1 Meeting #90bis		R1-1719090

Prague, Czech Republic, 9th – 13th October 2017

Title:	LS on TBS for TDD special subframe for BL/CE UE without repetition

# Response to:

# Release:	Rel-13

Work Item:	LTE_MTCe2_L1

Source:	RAN1

To:	RAN4

# Contact Person:

# Name:	Johan Bergman

E-mail Address:	johan (dot) bergman (at) ericsson (dot) com

# Attachments:

# R1-1718830, 36.213CR0980 (Rel-13, F) “TBS for TDD special subframe for BL/CE UE without repetition”

# R1-1719075, 36.213CR0987 (Rel-14, A) “TBS for TDD special subframe for BL/CE UE without repetition”

# 1. Overall Description:

RAN1 identified an issue with the TBS determination that may cause performance degradation for PDSCH transmissions without repetition in TDD special subframes to BL/CE UEs.

In legacy LTE TDD, in special subframes the TBS for PDSCH is scaled down to take the smaller number of OFDM symbols into account, but the same behaviour has not been specified for BL/CE UEs.

Therefore, RAN1 has agreed the attached Rel-13 CR and corresponding Rel-14 shadow CR, where it is specified that for BL/CE UEs in the PDSCH non-repetition case in CE mode A, the TBS in special subframes is scaled down in a similar way as in legacy LTE TDD.

# 2. Action to RAN4:

RAN1 respectfully asks RAN4 to take the above information into account.

# 3. Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting #91	27th Nov – 1st Dec 2017		Reno, NV, USA

TSG RAN WG1 Meeting #92	26th Feb – 2nd Mar 2018		Athens, Greece