3GPP TSG-RAN WG4 #85 Meeting                                   R4-1712157

Reno, Nevada, 27 November to 1 December, 2017

Agenda item:	9.3.3

Source: 	ZTE

Title: 	Guard band calculations for NR spectrum utilization in single numerology case

Document for:	Approval

Introduction

In August RAN4#84 meeting R4-1709075 WF on spectrum utilization was approved for single carrier operation and single numerology case for FR1 and FR2 as below [1]:

In October RAN4#84bis meeting R4-1711732 WF on BS channel BW set was approved and spectrum utilization of 30, 70, and 90 MHz for BS for single carrier operation and single numerology case for 3.3-3.8 GHz and 3.3-4.2 GHz bands was approved as below [7]:

The RB values in the tables above are agreed based on the assumption of symmetric guard band, and they are only meant to indicate the minimum guard band achievable for a given combination of SCS and channel BW. The RAN1 agreements below for subcarrier and RB alignment between different SCSs in a channel BW [2][3][4] could result in asymmetric guard band for some combinations of channel BW and higher order SCS [5], and their actual maximum RB allocation should be 1 RB less than the values in the tables above.

In October RAN4#84bis meeting R4-1711736 WF on NR spectrum utilization was approved and here are the open issues have been identified and the procedure for guard band calculation for finalizing the spectrum utilization RB values for single carrier operation and single numerology case [6]:

In this contribution, based on our view and understanding on the open issues and our assumption on channel raster design we calculate the guard band of each SCS and channel BW combination according to the above agreed procedure to find out which combinations’ RB values should be reduced by 1 RB.

Guard band calculation results

Here are the assumptions for the guard band calculation and RB value reduction:

- Reference SCS: Minimum SCS in a channel BW (fixed value agnostic to actual set of SCS used by network), i.e. for FR1 SCS 15 kHz for channel BWs ≤ 50 MHz, and SCS 30 kHz for channel BWs > 50 MHz, and for FR2 SCS 60 kHz for channel BWs <= 200 MHz, and 120 kHz for CBWs > 200 MHz. Since at this moment for a give channel BW, it’s not clear if other working groups could support higher order SCSs as reference SCS based on network scheduling, in this contribution we did not consider higher order SCSs as the reference SCS in a give channel BW.

- PRB grid alignment: Using the RAN1 subcarrier 0 alignment agreement. No constraints on aligning PRB #0 across different SCSs.

- Channel raster and channel raster mapping to subcarrier: the center of all RBs of the reference SCS in a channel BW will be left shifted ∆ = ½ reference SCS away from the center of the channel BW. Here’s an example for channel BW 5MHz where the center of 25 RBs is left shifted 7.5 kHz (½ reference SCS 15 kHz) away from the center of the channel BW.

Figure 1: 7.5 kHz left shift of the center of the 25 RBs for channel BW 5MHz

- PRB reduction: A shift of ½ reference SCS in a channel BW is acceptable for the reference SCS and PRB reduction will not be applied. For a given SCS and channel BW combination, RB value reduction will be applied when the guard band is more than ½ SCS less than the minimum guard band indicated in the approved R4-1709075 WF on spectrum utilization and R4-1711732 WF on BS channel BW set.

According to the above assumptions and the guard band calculation procedure in [6] the left guard band and right guard band of each SCS and channel BW combination have been calculated (see Appendix A1.1 and A1.2). Table 1 and Table 2 summarize the tentative agreed RB values (numbers in black in the first row of a cell) in [1], the minimum left guard band and the minimum right guard band (numbers in green in the 2nd row of a cell) derived from [1], and the left guard band and right guard band (numbers in red in the 3rd row of a cell) calculated according to [6] for FR1 and FR2 respectively. The highly asymmetric guard bands are highlighted with yellow color in the tables, and the corresponding RB values of the SCS and channel BW combinations should be reduced with 1 RB since the left or right guard band is much smaller than the minimum left or right guard band.

Table 1: Reference SCS for FR1 is the minimum SCS in each channel BW (i.e. 15kHz for CBWs <= 50MHz, and 30kHz for CBWs > 50MHz ). Tentative agreed RB values (numbers in black in the first row of a cell) in [1], the minimum left guard band and minimum right guard band (numbers in green in the 2nd row of a cell), and the left guard band and right guard band (numbers in red in the 3rd row of a cell)

| CBW (MHz) |  | 5 | 10 | 15 | 20 | 25 | 30(BS) | 40 | 50 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | 15 | 25 250,250 242.5,257.5 | 52 320,320 312.5,327.5 | 79 390,390 382.5,397.5 | 106 460,460 452.5,467.5 | 133 530,530 522.5,537.5 | 160 600,600 592.5,607.5 | 216 560,560 552.5,567.5 | 270 700,700 692.5,707.5 |
|  | 30 | 11 520,520 595,445 | 24 680,680 665,695 | 38 660,660 735,585 | 51 820,820 805,835 | 65 800,800 875,725 | 78 960,960 945,975 | 106 920,920 905,935 | 133 1060,1060 1045,1075 |
|  | 60 | N.A | 11 1040,1040 1010,1070 | 18 1020,1020 1080,960 | 24 1360,1360 1510,1210 | 31 1340,1340 1580,1100 | 38 1320,1320 1290,1350 | 51 1640,1640 1610,1670 | 65 1600,1600 1750,1450 |
| CBW (MHz) |  | 60 | 70(BS) | 80 | 90(BS) | 100 |  |  |  |
| SCS | 15 | N.A | N.A | N.A | N.A | N.A |  |  |  |
|  | 30 | 162 840,840 825,855 | 189 980,980 965,995 | 217 940,940 925,955 | [245] [900,900] [885,915] | 273 860,860 845,875 |  |  |  |
|  | 60 | 79 1560,1560 1530,1590 | [93] [1520,1520] [1670,1370] | 107 [1480,1480] [1630,1330] | [121] [1440,1440] [1590,1290] | 135 1400,1400 1550,1250 |  |  |  |

Table 2: Reference SCS for FR2 is the minimum SCS in each channel BW (i.e. 60kHz for CBWs <= 200MHz, and 120kHz for CBWs > 200MHz ). Tentative agreed RB values (numbers in black in the first row of a cell) in [1], the minimum left guard band and minimum right guard band (numbers in green in the 2nd row of a cell), and the left guard band and right guard band (numbers in red in the 3rd row of a cell)

| CBW (MHz) |  | 50 | 100 | 200 | 400 |
| --- | --- | --- | --- | --- | --- |
| SCS | 60 | 66 1240,1240 1210,1270 | 132 2480,2480 2450,2510 | 264 4960,4960 4930,4990 | N.A |
|  | 120 | 32 1960,1960 1900,2020 | 66 2480,2480 2420,2540 | 132 4960,4960 4900,5020 | 264 9920,9920 9860,9980 |

Based on the discussions and calculations above, we have the following two proposals:

Proposal 1: For a given SCS and channel BW combination, RB value reduction will be applied when the guard band is more than ½ SCS less than the minimum guard band indicated in the approved R4-1709075 WF on spectrum utilization and R4-1711732 WF on BS channel BW set.

Proposal 2: For the NR spectrum utilization of single carrier operation and single numerology case for FR1, the tentative agreed RB values in R4-1709075 and R4-1711732 of the following SCS and channel BW combinations (highlighted with yellow colour) should be reduced to the values shown in the table below in order to meet the minimum guard band requirement.

| CBW [MHz] |  | 5 | 10 | 15 | 20 | 25 | 30(BS) | 40 | 50 | 60 | 70(BS) | 80 | 90(BS) | 100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | 15 | 25 | 52 | 79 | 106 | 133 | 160 | 216 | 270 | N.A | N.A | N.A | N.A | N.A |
|  | 30 | 10 | 24 | 37 | 51 | 64 | 78 | 106 | 133 | 162 | 189 | 217 | [245] | 273 |
|  | 60 | N.A | 11 | 18 | 23 | 30 | 38 | 51 | 64 | 79 | [92] | 106 | [120] | 134 |

Note 1: Assume the reference SCS is 15 kHz for channel BWs ≤ 50 MHz, and 30 kHz for channel BWs > 50 MHz.

Note 2: Assume the center of all RBs of the reference SCS will be left shifted ∆ ≤ ½ reference SCS away from the center of the channel BW.

Note 3: Assume RAN1 subcarrier 0 alignment agreement across different SCSs in a channel BW. No constraints on aligning PRB #0 across different SCSs.

Conclusions

Based on the discussions in this contribution, we have the following Observation and Proposal:

Proposal 1: For a given SCS and channel BW combination, RB value reduction will be applied when the guard band is more than ½ SCS less than the minimum guard band indicated in the approved R4-1709075 WF on spectrum utilization and R4-1711732 WF on BS channel BW set.

Proposal 2: For the NR spectrum utilization of single carrier operation and single numerology case for FR1, the tentative agreed RB values in R4-1709075 and R4-1711732 of the following SCS and channel BW combinations (highlighted with yellow colour) should be reduced to the values shown in the table below in order to meet the minimum guard band requirement.

| CBW [MHz] |  | 5 | 10 | 15 | 20 | 25 | 30(BS) | 40 | 50 | 60 | 70(BS) | 80 | 90(BS) | 100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | 15 | 25 | 52 | 79 | 106 | 133 | 160 | 216 | 270 | N.A | N.A | N.A | N.A | N.A |
|  | 30 | 10 | 24 | 37 | 51 | 64 | 78 | 106 | 133 | 162 | 189 | 217 | [245] | 273 |
|  | 60 | N.A | 11 | 18 | 23 | 30 | 38 | 51 | 64 | 79 | [92] | 106 | [120] | 134 |

Note 1: Assume the reference SCS is 15 kHz for channel BWs ≤ 50 MHz, and 30 kHz for channel BWs > 50 MHz.

Note 2: Assume the center of all RBs of the reference SCS will be left shifted ∆ ≤ ½ reference SCS away from the center of the channel BW.

Note 3: Assume RAN1 subcarrier 0 alignment agreement across different SCSs in a channel BW. No constraints on aligning PRB #0 across different SCSs.

# References

- R4-1709075 WF on spectrum utilization, August 2017.

- R1-1706173 WF on subcarrier grid in NR, April 2017.

- R1-1706756 LS on subcarrier grid in NR, April 2017.

- R1-1708890 Final Report of 3GPP TSG RAN WG1 #88bis v1.0.0, May 2017.

- R4-1711972 LS on PRB grid in the NR, October 2017.

- R4-1711736 WF on NR spectrum utilization, ZTE, October 2017.

- R4-1711732 WF on BS channel BW set, October 2017.

# Appendix

#### A1.1 Subcarrier 0 alignment for FR1

#### A1.2 Subcarrier 0 alignment for FR2

(Zoomed)

(Zoomed)

(Zoomed)