3GPP TSG-RAN4 Meeting #85	R4-1712176

# Reno, Nevada, United States, 27th Nov 2017 - 1st Dec 2017

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 4746 | rev | - | Current version: | 14.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for EIRP based requirements in V2X |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_V2X-Core |  |  |  |  |  | Date: |  |  | 2017-11-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | ETSI harmonise standard EN 302 571 mandate a 23dBm/MHz EIRP Power Spectral Density. This requirement also affects power class 3 UE with positive effective antenna gain. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add additonal clause for power class 3 UE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Current spec will not be compliance with EN 302 571 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2G |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 36.512-1... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Begin of change>

### 6.2.2G	UE maximum output power for V2X Commincation

When UE is configured for E-UTRA V2X sidelink transmissions non-concurrent with E-UTRA uplink transmissions for E-UTRA V2X operating bands specified in Table 5.5G-1, the allowed V2X UE maximum output power for shall be as applied in Table 6.2.2-1 in subclause 6.2.2.

For V2X UE is configured for simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions for inter-band E-UTRA V2X / E-UTRA bands specified in Table 5.5G-2, the UE maximum output power shall be as specified in Table 6.2.2G-1 in subclause 6.2.2G for the corresponding inter-band con-current operation with uplink assigned to two bands.

Table 6.2.2G-1: Inter-band con-current V2X UE Power Class (two bands)

| V2X con-current band Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V2X_3A-47A |  |  |  |  | 23 | +2/-34 |  |  |
| V2X_7A-47A |  |  |  |  | 23 | +2/-34 |  |  |
| V2X_8A-47A |  |  |  |  | 23 | +2/-34 |  |  |
| V2X_39A-47A |  |  |  |  | 23 | +2/-34 |  |  |
| V2X_41A-47A |  |  |  |  | 23 | +2/-34 |  |  |
| NOTE 1: The con-current band combinations is used for V2X Service. NOTE 2: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 3: For inter-band con-current aggregation the maximum power requirement apply to the total transmitted power over all component carriers (per UE). NOTE 4: 4 refers to the transmission bandwidths (Figure 5.6-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |  |  |  |  |  |  |

For intra-band contiguous multi-carrier operation, the maximum output power is defined in Table 6.2.2G-2.

Table 6.2.2G-2: V2X UE Power Class for intra-band contiguous multi-carrier operation

| V2X multi-carrier Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V2X_47B |  |  |  |  | 23 | ±2 |  |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: For intra-band multi-carrier operation, the maximum power requirement should apply to the total transmitted power over all component carriers (per UE). |  |  |  |  |  |  |  |  |

When aUE is configured for E-UTRA V2X sidelink transmissions in Band 47, the UE shall meet the following additional requirements for transmission within the frequency ranges 5855-5925 MHz:

-	The maximum mean power spectral density shall be restricted to 23 dBm/MHz EIRP when the network signaling value NS_34 is indicated for Power Class 2.

-     The maximum mean power spectral density shall be restricted to 23 dBm/MHz EIRP when the network signaling value NS_33 is indicated for Power Class 3.

where the network signaling values are specified in clause 6.2.4G.

<End of change>