3GPP TSG-RAN WG4 Meeting #85                                                             R4-1712297

Reno Nevada, USA, 27 Nov-1 Dec, 2017

Agenda Item:	9.7.7.1

Source:	Intel Corporation

Title:	Link level simulation results for NR RLM based on SS block

Document for:	Discussion

# Introduction

In this contribution, we provide our initial simulation results for NR RLM SINR estimation.

# Simulation results

The agreed simulation parameters can be found in [1]. The detailed simulation parameters and assumptions are given in Table 1.

Table 1: RLM simulation parameters for SS block based on RLM

| Parameters | Value |  |
| --- | --- | --- |
|  | Sub-6GHz | Above 6GHz |
| Carrier frequency | 4GHz | 30GHz |
| Subcarrier Spacing for SS block | 15KHz, 30KHz | 120KHz, 240KHz |
| Bandwidth of SS block | 24 RBs |  |
| Number of transmitted SS block within a SS burst set period (K) | K=1 |  |
| RB utilization | 100% |  |
| Reference signals | NR-SSS |  |
| Transmission period of SS block | 20ms |  |
| Number of samples in L1 evaluation period | 5 samples, 10 samples, 20 samples |  |
| Transmit antennas | 1 |  |
| Receive antennas | 2, 4 |  |
| Propagation conditions | AWGN, EPA 5Hz, ETU 30Hz, ETU 70HZ, TDL-A/B/C/D 70Hz | AWGN, EPA5, TDL-A/B/C/D 83Hz |
| CP type | Normal CP |  |
| SNR | [-14, -13, ., 1, 0] dB |  |

Simulation results for EPA5 15KHz and 30KHz are shown in Fig.1 and Fig.2. SNR estimation delta is corresponding to the max SNR estimation error of 5% or 95%.

Fig.1 SNR estimation delta for EPA 15KHz

Fig.2 SNR estimation delta for EPA 30KHz

Simulation results for ETU30 15KHz and 30KHz are shown in Fig.3 and Fig.4.

Fig.3 SNR estimation delta for ETU 15KHz

Fig.4 SNR estimation delta for ETU 30KHz

Simulation results for TDL-B 120K and 240K Hz are shown in Fig.5 and Fig.6.

Fig.5 SNR estimation delta for TDL-B 120KHz

Fig.6 SNR estimation delta for TDL-B 240KHz

Observation 1: in low SNR region (SNR=-14dB), SNR estimation error is around 3.5~4.5dB with 5 samples.

Observation 2: in high SNR region (SNR=0dB), SNR estimation error can be reduced to 1dB with 5 samples.

# Conclusion

In this contribution some NR link level simulation results for NR SINR estimation was provided. The following observation can be drawn:

Observation 1: in low SNR region (SNR=-14dB), SNR estimation error is around 3.5~4.5dB with 5 samples.

Observation 2: in high SNR region (SNR=0dB), SNR estimation error can be reduced to 1dB with 5 samples.

# References

[1] R4-1709896, “Simulation assumptions on radio link monitoring in NR”, Huawei, HiSilicon