3GPP TSG-RAN WG4 Meeting #85	R4-1712448

Reno, US, 27th Nov – 1st Dec 2017

Source: 	Nokia, Nokia Shanghai Bell, Skyworks Solutions, Inc.

Title: 					Band n41 A-MPR

Agenda Item: 			9.4.3.3.1

Document for:	Discussion

# 1	Introduction

# 2	Discussion

## 2.1	Emission requirements

In addition to general spurious emission requirements following requirement apply.

### 2.1.1	Spectrum emission mask

Band 41 has special emission mask for LTE due to FCC requirements, see below Table 1.

Table 1: Band 41 LTE SEM

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -15 | -18 | -20 | -21 | 30 kHz |
| 1-2.5 | -10 | -10 | -10 | -10 | 1 MHz |
| 2.5-2.8 | -10 | -10 | -10 | -10 | 1 MHz |
| 2.8-5 | -10 | -10 | -10 | -10 | 1 MHz |
| 5-6 | -13 | -13 | -13 | -13 | 1 MHz |
| 6-9 | -25 | -13 | -13 | -13 | 1 MHz |
| 9-10 | -25 | -25 | -13 | -13 | 1 MHz |
| 10-13.5 |  | -25 | -13 | -13 | 1 MHz |
| 13.5-15 |  | -25 | -25 | -13 | 1 MHz |
| 15-18 |  |  | -25 | -13 | 1 MHz |
| 18-20 |  |  | -25 | -25 | 1 MHz |
| 20-25 |  |  |  | -25 | 1 MHz |

What is noticeable in band 41 SEM is that the breakpoint from -13 dBm/1 MHz to -25 dBm / 1 MHz is

- 6 MHz for 5 MHz Channel

- 9 MHz for 10 MHz Channel

- 13.5 MHz for 15 MHz Channel

- 18 MHz for 20 MHz Channel

Apart from 5 MHz channel this break point corresponds to LTE transmission bandwidth and comes from the way FCC has defined the mask requirement.

Band 41 emission mask for NR needs to be defined in similar manner by taking into account the actual transmission bandwidths of the signals when setting the breakpoint form -13 dBm/1 MHz to -25 dBm / 1 MHz. Actual NR transmission bandwidths are presented in Table 2. Also RAN4 need to consider what is the requirement for first 1 MHz outside than channel bandwidth and one possibility is to use same requirement as for NR general emission mask. Hence the proposed n41 SEM is presented in Table 3.

Table 2: NR transmission bandwidths

| SCS [kHz] | Channel bandwidths [MHz] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 | 25 | 40 | 50 | 60 | 80 | 100 |
| 15 | 4.5 | 9.36 | 14.22 | 19.08 | 23.94 | 38.88 | 48.6 | N.A | N.A | N.A |
| 30 | 3.96 | 8.64 | 13.68 | 18.36 | 23.40 | 38.16 | 47.88 | 58.32 | 78.12 | 98.28 |
| 60 | N.A | 7.92 | 12.96 | 17.28 | 22.32 | 36.72 | 46.8 | 56.88 | 77.04 | 97.20 |

Table 3: n41 SEM

| ΔfOOB MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ± 0 - 1 | -15 | -18 | -20 | -21 | -22 | -24 | -24 | -24 | -24 | -24 | 30 kHz |
| ± 1 - 5 | -10 |  |  |  |  |  |  |  |  |  | 1 MHz |
| ± 5 – 6 | -13 | -10 |  |  |  |  |  |  |  |  |  |
| ± 6 - X | -13 |  |  |  |  |  |  |  |  |  |  |
| ± X - (BWChannel + 5 MHz) | -25 |  |  |  |  |  |  |  |  |  |  |
| Note 1: X is defined in Table 2 |  |  |  |  |  |  |  |  |  |  |  |

### 2.1.2	Additional spurious emissions

In addition to band 41 specific emission mask UE needs to full fill additional spurious emission requirement as presented in Table 4.

Table 4: Additional spurious emission requirement

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | All channel bandwidths |  |
| 2490.5 ≤ f < 2495 | -13 | 1 MHz |
| 0 < f < 2490.5 | -25 | 1 MHz |

### 2.1.2	ACLR

Standard NR ACLR requirement applies as in Table 5 and 6.

Table 5: NR ACLR measurement bandwidth

| NR channel bandwidth / NR ACLR measurement bandwidth |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| NR ACLR measurement bandwidth | 4.515 | 9.375 | 14.235 | 19.095 | 23.955 | 38.895 | 48.615 | 58.35 | 78.15 | 98.31 |

Table 6: NR ACLR requirement

|  | Power class 1 | Power class 2 | Power class 3 |
| --- | --- | --- | --- |
| NRACLR |  | 31 dB | 30 dB |

UTRAACLR does not apply for band 41.

## 2.2. Simulation assumptions

The channel was placed at the lower edge of band n41. This maximizes the required A-MPR due to the additional spurious limit.

Simulation assumptions were as follows.

• PA calibration point: QPSK 100 RB DFT-s-OFDM signal with 0 dB MPR

• LO leakage and IQ-Image = 28 dBc

• NR ALCR = 30 dBc

• n41 SEM according to Table 3

• n41 additional spurious emission limit according to Table 4

## 2.3. A-MPR Simulation results

The result files in appendix provide total back-off triangles for several channel bandwidths as well as A-MPR triangles. A-MPR = total back off - tentave MPR, where tentative MPR refers (Table 1) to MPR results averaged between contributing companies in previous meeting [1], supplemented with our results on 256-QAM.

As an example, we provide Figure 1 below.

Figure 1: Total backoff (left) and A-MPR (right) triangles.

From right hand side figure the need for A-MPR can be easily seen as it is indicated in red color. We believe that A-MPR should be set with similar approach as MPR and not to do LTE like tables. A-MPR can also be additive on top of MPR atleast for contiguous allocationss. We will provide more concrete proposal once MPR values are agreed.

# 3	Conclusion

In this contribution we have provided A-MPR results for n41. Based on these results we believe that A-MPR should be set with similar approach as MPR and not to do LTE like tables.

# 4	References

[1] R4-1711559 WF on MPR for Sub-6 NR, RAN4#84bis, October 2017, Dubrovnik

# APPENDIX

Table 1. Tentative MPR used as reference in delta plots

|  |  | Tentative MPR [dB] |  |
| --- | --- | --- | --- |
| WF type | Modulation | Outer allocation | Inner allocation |
| DFT-S-OFDM | pi/2-BPSK | 0 | 0 |
|  | QPSK | 1 | 0 |
|  | 16-QAM | 1.5 | 0.5 |
|  | 64-QAM | 2 | 2 |
|  | 256-QAM | 4 | 4 |
| CP-OFDM | QPSK | 3 | 0.5 |
|  | 16-QAM | 3 | 1.5 |
|  | 64-QAM | 3 | 3 |
|  | 256-QAM | 6 | 6 |