3GPP TSG-RAN WG4 Meeting #85	R4-1712694

Reno, Nevada, USA, 27 November – 1 December 2017

Agenda Item:	9.5.3.7.1

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Simulation Results for FRC for BS receiver in-channel sensitivity requirements

Document for:	Discussion

# 1.	Introduction

Fixed Reference Channels (FRC) for BS receiver requirements and simulation assumptions were discussed in RAN4#84Bis, and the way forward was agreed in [1]. Further discussions have been held via RAN4 reflector, and some simulation parameters had been revised accordingly [2].

This contribution provides the simulation results for FRC for BS receiver in-channel sensitivity requirements using QPSK, according to the agreed simulation parameters in [1] as well as those revised by email discussions [2].

# 2.	Discussion

The simulation results for FRC without FDM (i.e. the 6REs not used for DMRS in each DMRS symbol are not used for PUSCH transmission) for BS receiver in-channel sensitivity requirements using QPSK are summarized in Table 1 below. Again, practical channel estimation is used but no other imperfection is applied in the simulation.

Table 1: Simulation results for in-channel sensitivity requirements without FDM using QPSK

|  | Channel Bandwidth (MHz) | SCS (kHz) | TBS (bits) | LDPC Code Rate | Number of PRBs | 95% Throughput SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| FR1-A1-1 | 5 | 15 | 1616 | 1/3 | 15 | 0.6 |
| FR1-A1-2 | 5 | 30 | 648 | 1/3 | 6 | 1.2 |
| FR1-A1-3 | 10 | 60 | 648 | 1/3 | 6 | 1.2 |
| FR2-A1-1 | 50 | 60 | 3560 | 1/3 | 33 | 0.4 |
| FR2-A1-2 | 50 | 120 | 1728 | 1/3 | 16 | 0.7 |

# 3.	Conclusion

This contribution has provided the simulation results for FRC for BS receiver in-channel sensitivity requirements using QPSK.

# In-channels

[1]		R4-1711926, “WF for FRC for BS receiver requirements and simulation assumption”, Ericsson, NTT DoCoMo, Huawei, ZTE, CATT, Nokia.

[2]	R4-1712630, “TP to TR 38.817-02: A.4 In-channel Channels”, Ericsson.