3GPP TSG-RAN WG4 Meeting #85	R4-1712701

Reno, USA, 27 Nov - 1 Dec, 2017

Agenda item:	9.4.3.1.3

Source:	Dish Network

Title:	Discussion on mmWave spherical coverage

Document for:	Discussion

# 1	Introduction

This contribution provides our thoughts about mmWave UE spherical coverage.

# 2	Discussion

Spherical coverage in mmWave devices has been discussed quite extensively in 3GPP. A couple of most recent WF’s are listed in references [1] [2]. Basically, RAN4 needs to agree on CDF percentile point and respective dBm-number.

On high level, the most controversial discussion so far has been around the spherical coverage CDF percentile point. Our understanding is that most but not all UE/chipset vendors claim 50% percentile is enough to characterise spherical coverage and that 20% percentile burdens the UE design too much. On the other hand, operators and some UE/chipset vendors are in favour of choosing 20% percentile.

We look at the issue from a couple of different aspects.

- Differences in 20% and 50% percentiles due to variation of dBm-values depending how design constraints are accounted in spherical analysis

- Network planning

- Device operation in the field

- Uncertainly in device design due to new technology

It is natural that the lower the CDF percentile is the more it’s value varies depending on the assumed design and associated constraints. For instance, if vendor A assumes antenna group in each corner and doesn’t account any impacts due to full glass screen the dBm-value is much higher compared with vendor B who assumes two antenna groups and full glass screen to have a significant spherical impact.

Observation 1: Variation of dBm-value associated with certain CDF percentile point increases the more the more design options/constraints are included.

Devices operate in networks that are planned according to certain general and operator specific criteria. While Sub-6GHz UE’s have rather omnidirectional antennas, the RF performance of mmWave UE’s is much relying on antenna gain that is achieved by beamforming. From network planning perspective, it is very important to have understanding on how the UE’s behave. It is beneficial for everyone to build an ecosystem where the mmWave connectivity is reliable enough.

One of the most essential questions is what kind of performance can be assumed below the CDF spherical coverage requirement? Logically, this question is much more important for 50% point than it is for 20% point. The required amount of mmWave base stations in network is the higher the lower the spherical coverages of the devices running in the network are. Operator has to have certain confidence to spherical coverage when they plan mmWave networks. Due to high frequencies, the network is dense anyway, so increasing the amount of base stations that makes the network more complex and increases difficulties in finding possible base station installation locations just to guarantee good performance within the areas of spherical coverage that is unspecified in 3GPP should be minimized.

Observation 2: Operators account the spherical coverage aspects in mmWave network planning. The spherical coverage percentile and associated requirement have an impact to network planning.

Naturally, in different use cases the device is hold for instance in hand blocking some antenna group(s) and so on impacting heavily to radiation patterns. However, without detailed analysis 3GPP shouldn’t conclude that because of the spherical coverage is heavily impaired anyways in some uses cases it hence suffices to specify the spherical coverage only for half of the sphere for free space.

In practice, no matter what kind of spherical coverage requirement 3GPP defines, the UE in any case will be subject to positions where it’s spherical coverage is not defined by specification. From UE perspective, it does not help in UE design if the spherical coverage is defined for 50% only because the UE operation cannot be restricted within the best 50% of the sphere only. We note that if the spherical coverage suffers for 50% percentile due to screen and antenna placement challenges as has been shown by some vendors, the even more degraded performance is present anyways for UE positions outside 50% percentile even if not specified by 3GPP.

Observation 3: In real operation, UE will be in such positions with respect to base station where it’s spherical coverage is not defined.

Observation 4: Defining spherical coverage for 20% percentile instead of 50% percentile does not impact UE design if the UE targets generally good spherical coverage.

Today, LTE UE’s are already very complex due to 4RX, CA, large number of supported bands and so on. Current trend in User Interface (UI) development is screen to cover more and more area of UE leaving fewer feasible places for antennas. It’s self-evident that in handheld devices Sub-6GHz bands will have the priority over mmWave antennas in antenna placement. We understand device vendors concerns and reluctance to agree “stringent requirements” for mmWave spherical coverage. Technology is new and some of the practical aspects such as screen impact might be a bit unclear, at least in level that can be publicly disclosed. Another side of the coin is that the large-scale deployments are still at least some years away making it possible to design against a challenging target.

Observation 5: mmWave technology is new and some aspects of it are unclear. While relaxed requirement is easy to meet, the time until large-scale deployments still allows to set a requirement to design for.

Our intention is to have enough analysis results with precisely defined assumptions for both 20% and 50% spherical coverage points. As observed earlier, greater delta amongst different analysis exists for 20% percentile compared with 50% percentile. However, that is not a valid reason to say 50% should be chosen because of that. If 50% percentile is chosen, operators are likely much more reluctant to agree say an average value amongst all the analysis conducted for 50% because the value at 50% percentile implies a bit what could be expected performance just below 50%.

The biggest concern from our side is, so far, if it really is so that from UE perspective the spherical coverage can only be guaranteed over 50% percentile. If that holds, then how dramatically the performance would drop outside this percentile? On high level, without going into dBm-values, we’d be more ok to accept 20% percentile with a fairly relaxed requirement compared with 50% and a challenging requirement

Observation 6: It is essential to have enough analysis and detailed assumptions provided both for 20% and 50% spherical coverage. At least some operators might be willing to accept more relaxed spherical coverage requirement associated with 20% percentile compared with the requirement associated with 50% percentile.

Finally, we bring up an idea for consideration. Would 20% percentile be more agreeable if RAN4 agrees to allow [x]dB relaxation for R15 and apply normal requirement from R16 onwards. X would be agreed at the same time with the 20% dBm-value.

Question: Would RAN4 be more comfortable with specifying 20% percentile if the requirement in R15 would be [x]dB relaxed compared with R16 to give companies more time to build confidence in designs?

# 3	Conclusions

mmWave UE spherical coverage was discussed with the following observations and a general question.

Observation 1: Variation of dBm-value associated with certain CDF percentile point increases the more the more design options/constraints are included.

Observation 2: Operators account the spherical coverage aspects in mmWave network planning. The spherical coverage percentile and associated requirement have an impact to network planning.

Observation 3: In real operation, UE will be in such positions with respect to base station where it’s spherical coverage is not defined.

Observation 4: Defining spherical coverage for 20% percentile instead of 50% percentile does not impact UE design if the UE targets generally good spherical coverage.

Observation 5: mmWave technology is new and some aspects of it are unclear. While relaxed requirement is easy to meet, the time until large-scale deployments still allows to set a requirement to design for.

Observation 6: It is essential to have enough analysis and detailed assumptions provided both for 20% and 50% spherical coverage. At least some operators might be willing to accept more relaxed spherical coverage requirement associated with 20% percentile compared with the requirement associated with 50% percentile.

Question: Would RAN4 be more comfortable with specifying 20% percentile if the requirement in R15 would be [x]dB relaxed compared with R16 to give companies more time to build confidence in designs?

# 4	References

[1] R4-1710081, “WF on Power class for mm Wave”, Qualcomm, RAN4#NR-AH3

[2] R4-1711895, “WF on power class in FR2“, Intel Corporation, Apple, Samsung, LGE, Huawei, MediaTek, Xiaomi, vivo, OPPO, Motorola, RAN4#84Bis