3GPP TSG-RAN WG4 Meeting #85	R4-1712703

Reno, USA, 27 Nov - 1 Dec, 2017

Agenda item:	9.4.4.4.1

Source:	Dish Network

Title:	In-band blocker for n71

Document for:	Approval

# 1	Introduction

This contribution proposes to specify n71 specific in-band blocker in a similar way as B71 specific in-band blocker is specified.

# 2	Discussion

High power TV broadcast channels exist below n71 DL. In LTE (B71) a study was conducted to evaluate the power level of CH36 TV blocker. According to [1] the CH36 power level at the UE can be up to -15dBm. The general in-band blocker at this offset is at -44dBm that is met without help from RF filter because it can be inside the band. In LTE, it is assumed that RX filter provides at least 29dB attenuation for CH36 blocker to be on part with the baseline -44dBm blocker. We note that this 29dB attenuation is feasible according to filter studies in [1] and elsewhere.

The IBB for B71 is as follows.

Table 7.6.1.1-2: In-band blocking

| E-UTRA band | Parameter | Unit |  | Case 1 | Case 2 | Case 3 | Case 4 | Case 5 | Case 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -56 |  | -44 | Void | Void | -38 | -15 |
|  | FInterferer (offset) | MHz | =-BW/2 – FIoffset,case 1 & =+BW/2 + FIoffset,case 1 |  | ≤-BW/2 – FIoffset,case 2 & ≥+BW/2 + FIoffset,case 2 |  |  | -BW/2 - 11 |  |
| 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 17, 18, 19, 20, 21, 22, 23, 25, 26, 27, 28, 31, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42, 43, 44, 45, 48, 65, 66, 68, 70, 72, 74 | FInterferer | MHz |  | (NOTE 2) | FDL_low – 15 to FDL_high + 15 |  |  |  |  |
| 30 | FInterferer | MHz |  | (NOTE 2) | FDL_low – 15 to FDL_high + 15 |  |  | FDL_low – 11 |  |
| 71 | FInterferer | MHz |  | (NOTE 2) | FDL_low – 12 to FDL_high + 15 |  |  |  | FDL-low - 12 |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band NOTE 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -BW/2 - FIoffset, case 1 and b. the carrier frequency +BW/2 + FIoffset, case 1 NOTE 3: FInterferer range values for unwanted modulated interfering signal are interferer center frequencies |  |  |  |  |  |  |  |  |  |

Because the same CH36 blockers is present close to n71 and because n71 RF filters have same characteristics as those designed for B71, the same IBB for n71 should be re-used.

Proposal1: Use the same band-specific in-band blocking for n71 as is specified for B71

# 3	References

[1] R4-1705684, “TP for TR 36.755: Rx blocking”, Qualcomm Incorporated, T-Mobile USA, Skyworks Solutions, Inc., Intel, Samsung, Nokia, Ericsson, RAN4#83, Hangzhou, China, May 2017