3GPP TSG-RAN WG4 Meeting #85	R4-1712773

Reno, NV, USA, 27 november – 1 december 2017

Agenda Item:	7.2.5.1

Source:	Ericsson

Title:	Remaining issues for FeMTC Cat-M2 UE PDSCH demodulation requirements

Document for:	Discussion

# 1	Introduction

RAN4#84bis agreed with the CR for PDSCH demodulation requirements for UE DL Cat-M2 [1], but some parameters and requirements are TBD. This contribution discusses the remaining issues on Cat-M2 UE demodulation requirements.

# 2	Simulation results

## 2.1	Cat-M2 (BL/CE UE)

Table 1 summarizes the simulation results for Cat-M2 PDSCH [2]. This table also provides the impairment results used to specify the Cat-M2 PDSCH demodulation requirements.

Table 1	Summary of simulation results for Cat-M2 PDSCH.

| CE Mode | Duplex mode | Ideal results | Impairment results |
| --- | --- | --- | --- |
| CE Mode A | FDD | -8.0 | -6.0 |
|  | TDD | -8.4 | -6.4 |
| CE Mode B | FDD | -15.5 | -13.5 |
|  | TDD | -15.5 | -13.5 |

# 3	MPDCCH parameter for PDSCH test

## 3.1	MPDCCH search space scheduling

The first subframe of MPDCCH search space is scheduled every (mdcch-NumRepetition * mpdcch-startSF-UESS), where mpdcch-startSF-UESS = {1, 1.5, 2, 2.5, 4, 5, 8, 10} for FDD and mpdcch-startSF-UESS = {1, 2, 4, 5, 8, 10, 20} for TDD.

Table 2 proposes the MPDCCH search space parameters for Cat-M2 UE PDSCH demodulation requirements. Figure 1 illustrates the scheduling pattern for the test cases. For TDD, note that RAN4 set the special subframes are set to non-BL/CE subframe; this mean no DL channel is scheduled in the special subframe.

Table 2	Proposed MPDCCH search space parameters for Cat-M2 UE PDSCH demodulation requirements.

| Duplex mode | CE Mode | PDSCH repetition number | mpdcc-NumRepetition | mpdcch-startSF-UESS | MPDCCH search space scheduling period |
| --- | --- | --- | --- | --- | --- |
| FDD/Half-duplex FDD | CE Mode A | 8 | 16 | 2 | 32 |
|  | CE Mode B | 32 | 64 | 2 | 128 |
| TDD | CE Mode A | 8 | 16 | 5 | 80 |
|  | CE Mode B | 32 | 32 | 8 | 256 |

Figure 1	Scheduling pattern for Cat-M2 UE PDSCH demodulation requirements.

# 4	Conclusion

Proposal: Set the MPDCCH search space scheduling parameters as follows:

| Duplex mode | CE Mode | mpdcc-NumRepetition | mpdcch-startSF-UESS |
| --- | --- | --- | --- |
| FDD/Half-duplex FDD | CE Mode A | 16 | 2 |
|  | CE Mode B | 64 | 2 |
| TDD | CE Mode A | 16 | 5 |
|  | CE Mode B | 32 | 8 |

# 5	References

R4-1711704, “Introduction of UE demodulation and CQI requirements for FeMTC (Rel-14)”, Ericsson.

R4-1710793, “Simulation results of PDSCH for Rel-14 BL/CE UE”, Ericsson.