3GPP TSG-RAN WG4 Meeting #85	R4-1712777

Reno, NV, USA, 27 November – 1 December 2017

Agenda Item:	7.2.2.2

Source:	Ericsson

Title:	Simulation result of enhanced RLM for CEModeB

Document for:	Discussion

# 1	Introduction

RAN4#84bis agreed with several CRs on enhanced RLM test case for CE Mode A by reusing the existing RLM CE Mode A test cases from Release 13. Since the enhanced RLM uses the event reporting, it is possible to introduce the test case for CE Mode B UE. However, there is no RLM test cases for CE Mode B in release 13 which makes it not possible to reuse release 13 test cases. We therefore provide the MPDCCH simulation results assuming CE Mode B for the enhanced RLM test cases.

It is noteworthy that RAN4 has studied the possibility to introduce RLM test case for CE Mode B in release 13, and MPDDCH simulations studies were conducted based on agreed simulations as shown in Table 1. The reason no RLM test case was introduced in release 13 RLM for CEModeB was that there is no support for reporting such as CQI reporting. Release 14 enhanced RLM is based on new event reporting, and since it based on the MPDDCH it is possible to reuse the release 13 CEModeB MPDCCH simulation work.

Table 1	MPDCCH simulation assumption for CE Mode B [1].

| Parameter | MPDCCH (CE Mode B) |
| --- | --- |
| DCI format | DCI Format 6-1B |
| System Bandwidth | 10 MHz |
| Channel model | AWGN, ETU30 |
| Antenna configuration | 2x1 |
| Number of information bits (incl. 16 bits CRC) | 18 |
| Antenna correlation | Low |
| Aggregation level and Repetition level | (16,256), (4,128), (16,128), (4,64) |
| Starting OFDM symbols (CFI) | 2 |
| Frequency hopping | OFF |
| Number of PRB | 4 for Aggregation level = 4, 8, 16 2+4 for Aggregation level =24 |
| Transmission type configured to UE | Distributed |
| DMRS scrambling sequence initialization parameter for UE-SS | PCID = 1 |
| Channel Estimation | DMRS based |
| UE residual frequency error | 50 Hz |

# 2	Simulation result

## 2.1	MPDCCH BLER with CEModeB

Table 2 shows the required SNR values to achieve 2%/10% MPDCCH BLER for each (AL, Rmax) combination for FDD. In Rel-13 discussion, it was discussed to derive the Qin/Qout from the two (AL, Rmax) sets below:

- Set 1: (24, 256) for Qout and (8,128) for Qin for OoS test cases

- Set 2: (16, 128) for Qout and (4,64) for Qin for IS test cases

Table 2	Required SNR [dB] to achieve CE Mode B UE MPDCCH BLER=2% and BLER=10% for each (AL, Rmax) pair with 2x1 antenna configuration.

|  | AWGN |  | ETU30 |  |
| --- | --- | --- | --- | --- |
| (AL, Rmax) | SNR@BLER=10% | SNR@BLER=2% | SNR@BLER=10% | SNR@BLER=2% |
| (16, 256) | -22.3 | -21.6 | -21.4 | -20.5 |
| (4, 128) | -16.8 | -16.6 | -16.6 | -14.7 |
| (16, 128) | -20.3 | -20.1 | -19.6 | -18.6 |
| (4, 64) | -15.4 | -14.6 | -13.4 | -12.5 |

Table 3 and Table 4 summarize the Qout/Qin points for set 1 and set 2.

Table 3	SNR difference between IS and OOS for Set 1.

| Channel | SNR for OOS (AL,Rmax)=(16,256) | SNR for IS (AL,Rmax)=(4,128) | Difference between OOS and IS |
| --- | --- | --- | --- |
| AWGN | -22.3dB | -16.6dB | 5.7dB |
| ETU30 | -21.4dB | -14.7dB | 6.7dB |

Table 4	SNR difference between IS and OOS for Set 2.

| Channel | SNR for OOS (AL,Rmax)=(16,128) | SNR for IS (AL,Rmax)=(4,64) | Difference between OOS and IS |
| --- | --- | --- | --- |
| AWGN | -20.3dB | -14.6dB | 5.7dB |
| ETU30 | -19.6dB | -12.5dB | 7.1dB |

## 2.2	Parameter setup

From the simulation results, it is observed that the SNR levels for Qin is around -15dB; this value is the assumed lowest operating SNR test point for CE Mode B UE. The early Qout event and early Qin event shall be triggered earlier than these points. The relation between transmission parameters used for early Qout and early Qin events are as in Table 5.

Table 5	Relation between the transmission parameters used in legacy RLM events and enhanced RLM events

| Event type | Aggregation level | Repetition level |
| --- | --- | --- |
| Legacy Qout | L’max | Rmax |
| Early Qout | L’max-[1] | Rmax/[2] |
| Legacy Qin | L’max-2 | Rmax/2 |
| Early Qin | L’max-[2] | Rmax/[8] |

As per the relation above, in our view it is possible to use the set as in Table 3 and Table 4 as reference to derive the SNR points for early Qin and early Qout events by scaling the aggregation level and repetition level according to relation Table 5. This gives:

(AL, Rmax) = (X1, Y1) assuming set 1 and (AL, Rmax) = (X2, Y2) assuming set 2.

The results show that set 1 is more aggressive at it allows the UE to operate in lower coverage compared to set 2 as it allows the UE to operate in lower SNR levels. Therefore our preference is to use set 2 for which the aggregation and repletion levels are (AL, Rmax)=(16, 128) and (AL, Rmax)=(4, 64) for ‘legacy’ out-of-sync and ‘legacy’ in-sync respectively. Using the relation between the aggregation/repetition level in Table 5, we derive the following set for early Qout and early Qin in CEModeB:

(AL, RL) = (8, 64) for Early Qout

(AL, RL) = (4, 8) for Early Qin

This means, if we design the test case that UE trigger the early Qout at -19.6 dB SNR and early Qin at -12.5 dB SNR assuming ETU30 channel model, the transmission parameters can be set to (AL, RL) = (8, 64) and (AL, RL) = (4, 8) respectively.

We think the parameter setups for Set1/Set2 are feasible to derive the SNR points for detecting early Qin and early Qout event for defining the enhanced RLM test cases for CE Mode B.

## 2.3	Test points

Based on the simulation results, we discuss the SNR levels to be set in the enhanced RLM requirements. The test points for the existing RLM out-of-synch and in-synch requirements are traditionally derived from the following formula:

- SNR2 = Qout + margin1 [dB],

- SNR3 = Qout - margin1 [dB],

- SNR4 = Qin – margin2 [dB],

- SNR5 = Qin + margin2 [dB],

- SNR1 = SNR5,

where margin1 = 3 dB and margin2 = 2.5 dB for ETU30 for UE Category 0. If we the same margin, SNR1 to SNR5 are derived based on Qin=-12.5 and Qout=-19.6 as follows:

|  | SNR1 | SNR2 | SNR3 | SNR4 | SNR5 |
| --- | --- | --- | --- | --- | --- |
|  | = SNR5 | Qout + 3 | Qout - 3 | Qin - 2.5 | Qin + 2.5 |
| Set 2 | -10.0dB | -16.6dB | -22.6dB | -15.0dB | -10.0dB |

# 3	References

R4-1609697, “Simulation result of RLM with Cat-M1 CE Mode B”, Ericsson.