3GPP TSG-RAN WG4 meeting #85	      R4-1713061

Reno, Nevada, USA, 27th November – 1st December, 2017

Source:	ZTE Corporation

Title:	TP for TS 38.113: Section 4 (Exclusion bands)

Agenda item:	9.6.1

Document for:	Approval

Introduction

In this contribution we provide TP to the NR EMC specification in TS 38.113[1] for Section 4, capturing exclusion bands for the EMC radiated immunity test for the NR BS, based on the in-band blocking limit defined in the approved TS 38.104 V0.4.0 [2]. Exclusion bands for NR BS type 1-O and 2-O will be aligned to TS 38.104 subclause 10.5 OTA in-band selectivity and blocking requirements.

References

- R4-1711984, Draft TS 38.113 v0.0.2, ZTE

- R4-1711971, TS 38.104 V0.4.0, Ericsson

--------------Start of text proposal-------------

- 4.4	Exclusion bands

An exclusion band is a band of frequencies over which no tests of radiated immunity are made.

4.4.1	Transmitter exclusion band

For testing of radiated immunity there shall be no transmitter exclusion band.

4.4.2	Receiver exclusion band

The receiver exclusion band for base stations extends from the lower frequency of the Base Station receive band minus 20 MHz to the upper frequency of the Base Station receive band plus 20 MHz  for 1-C type NR BS in an operating band less than 200 MHz wide or 1-H type NR BS in an operating band less than 100 MHz wide, or minus 60 MHz to the upper frequency of the Base Station receive band plus 60 MHz for 1-C type NR BS in an operating band more than or equal to 200 MHz wide or 1-H type NR BS in an operating band more than or equal to 100 MHz wide. The exclusion bands for NR operating bands are as set out in Tables 4.4.2 respectively.

<For NR type 1-O and 2-O, receiver exclusion band requirement will be aligned to TS 38.104 subclause 10.5 OTA In -band selectivity and blocking requirements>

<table will be added>

-------------End of text proposal-------------