3GPP TSG RAN WG4 Meeting #85	R4-1713514

Reno, Nevada, USA, 27 November-1 December, 2017

Agenda Item:	9.7.6.2

Source: 	Ericsson

Title:	Further discussions on synchronous and asynchronous Dual connectivity in Rel-15 LTE-NR combinations

Document for:	Approval

# Introduction

In RAN4#84bis, RAN4 agreed to a way forward in R4-1711964 [7], where a number of agreements related to LTE-NR DC combinations. In this way forward, a number of cases have been agreed, while there are other open issues. We summarize our understanding on open issues as listed in the table below.

|  | LTE-NR Inter-band DC | LTE-NR Intra-band DC |
| --- | --- | --- |
| Asynchronous DC | Asynchronous requirements are defined in [7]. | For FDD-FDD case, asynchronous case can be supported and defined in [7]. |
| Synchronous DC | Synchronous requirements are not defined in [7]. | MRTD for synchronous requirement is not finalized yet. |

In this contribution, we provide our proposals for the two open issues as listed in the above table, namely:

- Inter-band synchronous requirements

- Intra-band synchronous requirements

# Inter-band synchronous LTE-NR dual connectivity

In [7], we have agreed on asynchronous requirements for inter-band LTE-NR dual connectivity. In this section, we provide our proposals for synchronous inter-band dual connectivity requirements.

In LTE, following are defined in [1]:

- Synch requirement for maximum receive timing difference (MRTD) is 33µs

- For maximum transmission timing difference (MTTD) is defined as 35.21 µs for synchronous case

The MRTD definition in LTE considers up to 9km relative propagation distance difference, which provides excellent flexibilities for LTE deployment. Since LTE-NR dual connectivity will require same deployment flexibilities, we propose to adopt LTE-LTE DC synchronous requirements also for LTE-NR DC. As an example, when a below 1Ghz LTE carrier is aggregated via DC with a mmWave NR carrier, then this flexibility will be very beneficial.

Based on the discussions in the above discussions, we propose the following:

Proposal 1: For synchronous operation of inter-band LTE-NR dual connectivity,

- Maximum receive timing difference (MRTD) is defined as 33µs

- Maximum transmission timing difference (MTTD) is defined as 35.21µs.

# Intra-band LTE-NR dual connectivity

In [7], agreement on TDD-TDD case is as follows:

- For TDD-TDD case,

  - Synchronous DC operation as well as NR DL/UL resource assignment to avoid DL/UL interference with LTE TDD is needed for intra-band LTE-NR DC.

  - It is proposed to only consider collocation scenario for intra-band LTE-NR DC in Rel-15.

The exact MRTD value is not agreed. As we have explained in our previous contribution [8], the requirements for intra-band will largely depend on UE implementation. Any of the following are possible:

- The LTE and NR bands are implemented in the UE using a common FFT.

- The LTE and NR bands are implemented in the UE using totally separated FFTs.

Regarding the MRTD in intra-band combinations when regardless the UE implementation uses common FFT or separate FTTs for both LTE and NR bands, the cyclic prefix determines the maximum timing difference between two different reference signals at the UE since the combinations only operate in collocated manner, thus, the MRTD will scale according to CP length as assumed for different SCS. Regarding the MTTD, the UE has single TAG for both the UL (since they are collocated) and also the UE uses the same path for both LTE and NR, there is no MTTD requirement in this case.

It is also worth noting here that, the same is also valid for FDD-FDD case since the same conditions are applicable for FDD intra-band combinations as it is for TDD intra-band case.

Thus we propose the following:

Proposal 2: UE shall support the synchronous LTE-NR intra-band DC provided that the MRTD at the UE does not exceed values shown in the table below:

| LTE SCS (kHz) | NR SCS (kHz) | Maximum receive timing difference, MRTD (µs) for synchronous operation |
| --- | --- | --- |
| 15 | 15 | 4.76 |
| 15 | 30 | 2.38 |
| 15 | 60 | 1.19 |

For obvious spectrum arrangement reasons, TDD-FDD intra-band combination are not possible, thus, no consideration for TDD-FDD intra-band combinations are needed.

# Conclusions

We proposed the following in this paper:

Proposal 1: For synchronous operation of inter-band LTE-NR dual connectivity,

- Maximum receive timing difference (MRTD) is defined as 33µs

- Maximum transmission timing difference (MTTD) is defined as 35.21µs.

Proposal 2: UE shall support the synchronous LTE-NR intra-band DC provided that the MRTD at the UE does not exceed values shown in the table below:

| LTE SCS (kHz) | NR SCS (kHz) | Maximum receive timing difference, MRTD (µs) for synchronous operation |
| --- | --- | --- |
| 15 | 15 | 4.76 |
| 15 | 30 | 2.38 |
| 15 | 60 | 1.19 |

Based on these proposals, we proposed to send an LS to RAN1 and RAN2 in [6].

# References

- 3GPP TS 36.133

- R4-135611, “Way forward on methodology for calculating max UL time difference between TAGs”, Huawei, HiSilicon, Ericsson, Intel

- R4-151998, ”TDD+FDD 2 UL interband”, Ericsson.

- R4-100340, “Suggestion of UE Rx-Tx time difference measurement requirement”, CATT

- R4-151854, “UL Tx timing difference in DC”, Ericsson.

- R4-1711479, “LS to RAN1 and RAN2 on Definition of synchronous and asynchronous Dual connectivity in Rel-15 LTE-NR combinations”, Ericsson.

- R4-1711964, “WF on synchronous and asynchronous definitions for LTE-NR DC in Rel-15”, Ericsson, MediaTek

- R4-1711478, “Definition of synchronous and asynchronous Dual connectivity in Rel-15 LTE-NR combinations”, Ericsson.