3GPP TSG-RAN WG4 #85	R4-1713536

Reno, Nevada, USA, 27th November – 1 December 2017

Source: 	Ericsson

Title:  	TP on UE requirements for Rel-15 TR 36.790: LAA/eLAA for the “CBRS” 3.5GHz band in the United States

Agenda Item:	8.11.2

Document for:	Approval

# Introduction

In RAN#75, a new work item was agreed for licence assisted access (LAA) and enhanced LAA (eLAA) operation for the CBRS 3.5GHz band in US [1]. A new TR is agreed as TR 36.790 and a skeleton in agreed in [2].

In this contribution, we propose a text for this TR which includes texts related to scope of the report.

# Text proposal

The following text proposal is intended for TR 36.790.

<<<<< start of TP <<<<<<

# E-UTRA RF requirements for UE

## UE Transmitter Characteristics

## <Editor’s note: this section will be completed when eLAA will be worked on.>

### Transmit power

## UE Receiver Characteristics

### Reference sensitivity power level

Table 6.2.1-1: Reference sensitivity QPSK PREFSENS (CA with band 49)

| CA_2A-49A1 | 2 |  |  | -97.8 | -94.8 | -93.0 | -91.7 | FDD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 49 |  |  |  | -95.5 |  | -92.5 | TDD |
| NOTE 1: The requirement for B49 does not apply when there is at least one individual RE within the B49 downlink transmission bandwidth which falls into the reference sensitivity exclusion region as specified in Table 6.2.1-2. |  |  |  |  |  |  |  |  |

Table 6.2.1-2: Band 49 Reference sensitivity measurement exclusion region in MHz.

| Licensed Component Carriers / E-UTRA Band / Harmonic order / Channel BW in UL |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Licensed Component Carriers | Harmonic order | 5MHz | 10MHz | 15MHz | 20MHz |
| 21 | 2 | +/- 12.5 | +/- 25 | +/- 37.5 | +/- 50 |
| NOTE 1: Even though UL harmonic does not fall directly into Band 49 the exclusion region still applies. NOTE 2: The center of the exclusion region is obtained by multiplying the UL channel center frequency by the harmonic order. |  |  |  |  |  |

### Adjacent channel selectivity

### Blocking characteristics

### In band blocking

For CA configurations including Band 49, in-band blocking in Band 49 is defined for a 10 MHz unwanted interfering signal falling into the UE receive band or into the first 60 MHz below or above the UE receive band (Table 7.6.1.1A-0a and Table 7.6.1.1A-0b in TS 36.101).

<<<<< end of TP <<<<<<

# Conclusion

We propose to adopt the above mentioned text proposal for TR 36.790.

# References

- RP- 170804, “New WI: LAA/eLAA for the CBRS 3.5GHz band in the United States”, AT&T, RAN#75, March 2017.

- R4-1709823, “Template skeleton for TR 36.790: LAA/eLAA for the “CBRS” 3.5GHz band in the United States”, Ericsson.