3GPP TSG-RAN WG4 Meeting #85	  R4-1713545

Reno, Nevada, US, 27 Nov – 1 Dec 2017

Source:	Nokia, Nokia Shanghai Bell

Title:	Consideration on EVM window length reduction for NR

Agenda item:	9.5.2.4.1.1

Document for:	Approval

# 1.	Introduction

In this contribution, we would like to discuss the possible reduction of EVM window length for NR with different BW and SCS combinations with high bandwidth utilization.

# 2.	Discussion

The EVM shall be measured at the point after the FFT and a zero-forcing (ZF) equalizer in the receiver, according to TS 36.104 [1], Annex E. The same reference point will be also selected for NR. It should be noted that before measuring the EVM, time and frequency synchronization must be carried out. The test equipment then computes the EVM for two extreme values of sample timing difference between the FFT processing window and the nominal timing of the ideal signal; these two extreme values correspond to the beginning and end of the EVM window, the length of which is expressed as a percentage of the Cyclic Prefix (CP) length.

Figure 1 illustrates the time window positions for FFT and the inter-symbol interference, coming from the delay spread of adjacent symbols. Typically, the filters have a symmetric impulse response; hence the optimum FFT window with lowest impact of inter-symbol interference is located at the centre position between the symbol boundaries.

Specification TS 36.104 defines an offset of WEVM/2 from that optimum position in both directions. The values of WEVM are copied from 3GPP TS36.104 and provided in Table 1 for E-UTRA and in Table 2 for NB-IoT.

OFDM symbol

n-1

OFDM symbol

n

FFT window 1

OFDM symbol

n+1

CPn+1

FFT window 2

Optimum FFT window

CPn

WEVM

Inter-symbol

interference

Figure 1: FFT time window position in OFDM symbol n, which is suffering from inter-symbol interference from adjacent symbols n+1 and n-1. For symmetric delay spread, the centered green FFT window constitutes the optimum. Both 3GPP defined FFT windows (cyan) experience higher inter-symbol interference. Cyclic prefix (CPn and CPn+1) boundaries are marked with dashed lines.

Table 1: EVM window length for LTE with normal CP as copied from 3GPP TS36.104 [1]

| Channel Bandwidth MHz | FFT size |  | Cyclic prefix length for symbols 0 in FFT samples | Cyclic prefix length for symbols 1-6 in FFT samples | EVM window length W | Ratio of W to total CP for symbols 1-6* [%] |
| --- | --- | --- | --- | --- | --- | --- |
| 1.4 | 128 |  | 10 | 9 | 5 | 55.6 |
| 3 | 256 |  | 20 | 18 | 12 | 66.7 |
| 5 | 512 |  | 40 | 36 | 32 | 88.9 |
| 10 | 1024 |  | 80 | 72 | 66 | 91.7 |
| 15 | 1536 |  | 120 | 108 | 102 | 94.4 |
| 20 | 2048 |  | 160 | 144 | 136 | 94.4 |
| * Note: These percentages are informative and apply to symbols 1 through 6. Symbol 0 has a longer CP and therefore a lower percentage. |  |  |  |  |  |  |

Table 2: EVM window length for NB-IoT with normal CP as copied from 3GPP TS36.104 [1]

| FFT size | Cyclic prefix length for symbols 0 in FFT samples | Cyclic prefix length for symbols 1-6 in FFT samples | EVM window length W | Ratio of W to total CP for symbols 1-6(Note 1) [%] |
| --- | --- | --- | --- | --- |
| 128 | 10 | 9 | 3 | 33.3 |
| Note 1: These percentages are informative and apply to symbols 1 through 6. Symbol 0 has a longer CP and therefore a lower percentage. |  |  |  |  |

Please note that the EVM window length of NB-IoT (see Table 2) is 60% of the LTE 1.4 MHz EVM window length, though NB-IoT has the same sample rate, FFT size and CP length as LTE 1.4 MHz. However, NB-IoT has a narrower guard band than LTE 1.4 MHz.

For LTE 10 MHz, WEVM is 66 out of 72 cyclic prefix samples. This leaves just 3 samples on either side to protect from inter-symbol interference. Consequently, some inter-symbol interference will leak into the FFT window and generate EVM.

While this was not that much of an issue at the time the EVM definition was established for LTE, it creates severe problems in NR standard specification:

- Around 1 percent of EVM contribution have not been a problem for modulation schemes up to 64QAM. However, for modulation schemes as high as 256QAM or 1024 QAM, this constitutes a problem due to more stringent requirements for EVM.

- Bandwidth efficiency above 90% [2] requires steeper filters with longer filter impulse response. As a result, more inter-symbol interference is expected to leak from adjacent OFDM symbols.

For these reasons, a shorter EVM window length is recommended.

Observation 1: The reuse of current EVM window based on 36.104 specification in NR creates several problems for high modulation schemes (256 and 1024 QAM) and high spectral utilization (above 90%).

The drawback of shorter EVM window length could be that more of the CP is utilized for the base station and less is remaining for channel delay spread or UE synchronization offset. This applies even more for larger subcarrier spacing (i.e. shorter CP). Therefore, the reduction of EVM window could take into account also this adverse effect.

Taking into account above considerations we propose to reduce EVM window length to be reduced to 60% of its original value (based on LTE) only for BW/SCS combination in NR [2] where spectral utilization is above 90%. This could be applied based on the highest order modulation scheme for given base station class because the modulation scheme is assigned dynamically and potentially different for each resource block, so we cannot consider definition of EVM window length depending on modulation scheme.

Proposal 1: EVM window length should be reduced to 60% of its original value (based on LTE) for BW/SCS combinations with spectral utilization above 90%.

# 3.	Conclusion

In this contribution, we discussed the possible reduction of EVM window length for NR with different BW and SCS combinations with high bandwidth utilization. We have made following observation and proposal:

Observation 1: The reuse of current EVM window based on 36.104 specification in NR creates several problems for high modulation schemes (256 and 1024 QAM) and high spectral utilization (above 90%).

Proposal 1: EVM window length should be reduced to 60% of its original value (based on LTE) for BW/SCS combinations with spectral utilization above 90%.

# References

- 3GPP TS 36.104, 	Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception

- R4-1711736 “WF on NR spectrum utilization”, ZTE