3GPP TSG-RAN WG4 #85	R4- 1713556

Reno, Nevada, USA 27th November –1st December 2017

Source:	Ericsson

Title:	TP for draft CR for TS 37.105: Updating sub-clause 10.6 OTA RX OOB Blocking

Agenda item:	8.27.2

Document for:	Approval

# 1.	Introduction

In this contribution a text proposal is presented for OTA RX blocking section of TS 37.105 and we would like to encourage feedback from other companies on the proposed text.

# 2.	Discussion

Based on the time line proposed timeline OTA blocking sections of the TS shall be drafted.  A revised version of the draft text for TS 37.105 was approved [3].  Based on other submissions on the concept [4,5] issues such as the lower limit and the polarization of the wanted signal and the interferrer is discussed.

Some of the open issues are:

- Fixed distance value of the interfering signal

- Interfering signal level

- Wanted signal level

- Polarization considerations for wanted and intering signal

# 3.	References

3GPP TS 37.105 v14.0.0

R4-1706126, “WF on Spec drafting”, Huawei

R4-1711858, “DRAFT TS 37.105 Specification text for Section 10.6 OTA RX Blocking”, Ericsson

R4- 1713277, “TP for TR 37.843 Lower limit on OOB Blocking”, Ericsson

R4-1713560, “OOB Blocking Polarization”, Ericsson

[Start of Text Proposal]

## 10.6	OTA Blocking

### 10.6.1	General

The blocking characteristics are a measure of the receiver unit ability to receive a wanted signal at the RIB at its assigned channel in the presence of an unwanted interferer.

The requirement applies at the RIB when the AoA of the incident wave of the received signal and the interfering signal are the same direction and are within the RoAoA associated with minSENS

The wanted signal applies to all supported polarizations, under the assumption of polarization matching. The interferer shall be mechanically and polarization between the interferer and AAS BS shall be polarization aligned.

### 10.6.2	Minimum requirement for MSR operation

10.6.2.1	General minimum requirement

#### The interfering signal mean power for all operating bands is [x] dBm and a constant fixed distance of [x] m.

For a wanted and an interfering signal coupled to the RIB using the parameters in table 10.6.2.1-1, the following requirements shall be met:

-	For any E-UTRA carrier, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channel defined in 3GPP TS 36.104 [8], subclause 7.2.1.

-	For any UTRA FDD carrier, the BER shall not exceed 0,001 for the reference measurement channel defined in 3GPP TS 25.104 [6], subclause 7.2.1.

-

For multi-band RIB, the requirement applies for each supported operating band. The in-band blocking frequency ranges of all supported operating bands according to Table 10.6.2.1-1 shall be excluded from the requirement.

Table 10.6.2.1-1: Blocking performance requirement

| Operating Band Number | Centre Frequency of Interfering Signal (MHz) |  |  |  |  |  | Wanted Signal mean power (dBm) | Type of Interfering Signal |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1-7, 9-11, 13, 14, 18, 19, 21-23, 24, 27, 30, 33-45, 65, 66, 68 | 30 (FUL_high +20) |  | to to | (FUL_low -20) 12750 |  |  | EISminSENS + x dB (NOTE) | CW carrier |  |
| 8, 26, 28 | 30 (FUL_high +10) |  | to to | (FUL_low -20) 12750 |  |  | EISminSENS + x dB (NOTE) | CW carrier |  |
| 12 | 30 (FUL_high +13) |  | to to | (FUL_low -20) 12750 |  |  | EIS minSENS + x dB (NOTE) | CW carrier |  |
| 17 | 30 (FUL_high +18) |  | to to | (FUL_low -20) 12750 |  |  | EISminSENS + x dB (NOTE) | CW carrier |  |
| 20 | 30 (FUL_high +20) |  | to to | (FUL_low -11) 12750 |  |  | EISminSENS + x dB (NOTE) | CW carrier |  |
| 25 | 30 (FUL_high +15) |  | to to | (FUL_low -20) 12750 |  |  | EISminSENS + x dB (NOTE) | CW carrier |  |
| 31 | 30 (FUL_high +5) |  | to to | (FUL_low -20) 12750 |  |  | EISminSENS + x dB (NOTE) | CW carrier |  |
| 40-455,48 | 30 (FUL_high +5) |  | to to | (FUL_low -20) 12750 |  |  | EISminSENS + x dB (NOTE) | CW carrier |  |
|  |  | NOTE: EISREFSENS depends on the RAT, the BS class and the channel bandwidth, see subclause 10.3. "x" is equal to 6 dB in case of E-UTRA or UTRA wanted signals. |  |  |  |  |  |  |  |

[End of Text Proposal]