3GPP TSG-RAN WG4 Meeting #85	R4-1713727

# Reno, United States, 27 November – 1 December 2018

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 5481 | rev |  | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 36.133 on Interruptions to LTE cells in EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2017-11-17 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Interruption requirements for NR EN-DC are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Structure for interruption requirements where the victim cell is LTE PCell or LTE SCell are added for EN-DC. Interruptions to NR cells should be covered in 38.133. Brackets are added around requirements including NR SCells, as it is not yet clear whether NR SCells will be included in Rel-15. Asynchronous/synchronous requirements are left as FFS in this draft, as no numbers are proposed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No NR interruption requirements exist. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section 7.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  | TS 36.521-3 |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 7.x	Interruptions with E-UTRAN-NR Dual Connectivity

### 7.x.1	Introduction

This section contains the requirements related to the interruptions on PCell and SCell(s) in MCG, when

NR PSCell is added or released.

transitions between active and non-active during DRX, or

transitions from non-DRX to DRX, or

SCell in MCG [or NR SCell in SCG] is added or released, or

SCell in MCG [or NR SCell in SCG] is activated or deactivated, or

measurements on SCC with deactivated SCell in MCG.

This section contains interruptions where victim cell is PCell or SCell belonging to MCG. Requirements for interruptions requirements when the victim cell is NR PSCell or NR SCell belonging to SCG are specified in [TS 38.133].

### 7.x.2	Requirements

#### 7.x.2.1	Interruptions at NR PSCell addition/release

When an NR PSCell is added or released as defined in [2] the UE is allowed an interruption of up to [TBD] subframe on PCell and the activated SCell in MCG if configured during the RRC reconfiguration procedure [2]. This interruption is for both uplink and downlink of PCell and SCell(s).

Editor’s note: Whether different requirement is needed for synchronous and asynchronous E-UTRA-NR dual connectivity is FFS.

#### 7.x.2.2	Interruptions at NR transitions from non-DRX to DRX

Interruption on PCell and the activated SCell in MCG if configured due to NR PSCell transitions from non-DRX to DRX when PCell is in non-DRX shall not exceed [TBD] subframe.

#### 7.x.2.3	Interruptions at NR SCell addition/release

When one SCell is added or released in MCG as defined in [2] the UE is allowed:

-	an interruption on PCell as defined in clause 7.8.2.7

-    an interruption on SCell in MCG as defined in clause 7.8.2.7

[When one NR SCell is added or released in SCG, the UE is allowed:

-   an interruption on PCell

-	of up to [TBD] subframes, if the PCell is not in the same band as the NR SCell being added or released, or

-	of up to [TBD] subframes, if the PCell is in the same band as the NR SCell being added or released;

-   an interruption on any activated SCell in MCG:

-	of up to [TBD] subframes, if the PCell is not in the same band as the NR SCell being added or released, or

-	of up to [TBD] subframes, if the PCell is in the same band as the NR SCell being added or released;]

Editor’s note: Whether different requirement is needed for synchronous and asynchronous E-UTRA-NR dual connectivity is FFS.

#### 7.x.2.4	Interruptions at NR SCell activation/deactivation

When one SCell is activated or deactivated in MCG, the UE is allowed:

-	an interruption on PCell as defined in clause 7.8.2.8

-	an interruption on any activated SCell in MCG as defined clause 7.8.2.8

[When one SCell is activated or deactivated in SCG, the UE is allowed:

an interruption on PCell:

-	of up to [TBD] subframes, if the PCell is not in the same band as the SCell being activated or deactivated, or

-	  of up to [TBD] subframes, if the PCell is in the same band as the SCell being activated or deactivated]

Editor’s note: Whether different requirement is needed for synchronous and asynchronous E-UTRA-NR dual connectivity is FFS.

#### 7.x.2.5	Interruptions during measurements on LTE SCC

When one SCell in MCG is deactivated, the UE is allowed due to measurements on the SCC with the deactivated SCell:

-    an interruption on PCell as defined in clause 7.8.2.9

-    an interruption on any activated SCell in MCG as defined in clause 7.8.2.9

Editor’s note: Whether different requirement is needed for synchronous and asynchronous E-UTRA-NR dual connectivity is FFS.