3GPPRAN4#85	R4-1713775

Reno, US, Nov 27th- Dec 1st, 2017

Title: 	Interruptions in NSA operation

Source: 	Qualcomm Incorporated

Agenda item:	9.7.8 - Interruption and related requirements (38.133/36.133) [NR_newRAT]

Document for:	Discussion

# Introduction

In Nagoya RAN4 meeting a WF was agreed on interruptions on NSA operation [1]. In the contribution, we provide a proposal on interruptions to be taken on PCell/SCell in MCG and PSCell/SCell in SCG.

# Discussion

## Interruptions

In the contribution, we provide times on interruptions in various scenarios for both LTE and NR in NSA operation. For NSA, we propose to keep the interruptions to be the same between LTE and NR. For reconfiguration scenarios, we propose a 5 ms interruption on all active cells when another cell is added or released. For activation scenarios, we propose a 1 ms interruption on all active cells when another cell is being activated or deactivated.

We explicitly provide the interruption time for the following scenarios which were covered in [1]

| Type of NSA operation | Victim | Aggressive | Interruption Time |
| --- | --- | --- | --- |
| PSCell is added or released | PCell SCell in MCG (No SCell in SCG in this case) | PSCell | 5 ms |
| SCell in either MCG or SCG is added or released | PCell and activated SCell in MCG PSCell and activated SCell in SCG | SCell in MCG SCell in SCG | 5 ms |
| SCell in either MCG or SCG is activated or deactivated | PCell and activated SCell in MCG PSCell and activated SCell in SCG | SCell in MCG SCell in SCG | 1 ms |
| Measurements on SCC with deactivated SCell in either MCG or SCG | PCell and activated SCell in MCG PSCell and activated SCell in SCG | SCell in MCG SCell in SCG | 1 ms |
| Transitions between active and non-active during DRX | PCell and activated SCell in MCG PSCell and activated SCell in SCG | PSCell PCell | 1 ms |
| Transitions from non-DRX to DRX | PCell and activated SCell in MCG PSCell and activated SCell in SCG | PSCell PCell | 1ms |

# Conclusions

In this contribution, we propose interruptions of 5ms when RF (re)configuration needs to happen and 1 ms when RF activation/deactivation needs to happen. These interruptions can be taken on all active cells of both NR and LTE.

# References

[1] R4-1709901, “WF on interruption on NSA operation”, Huawei, HiSilicon