3GPP TSG-RAN WG4 Meeting #85	R4-1713785

Reno, NV, The US, 27th November – 1st December 2017

Source:	Nokia, Nokia Shanghai Bell

Title:	eAAS BS transmit OFF power requirements - scaling or fixed

Agenda item:	8.27.3.2

Document for:	Approval

# 1	Introduction

During the last 3GPP RAN4 #84bis meeting, a way forward on transmit ON/OFF power and transient period was approved [1]. Even though agreements are made in the way forward, one open issue remains as to whether scaling should be applied to the requirement.

In this document, we provide our choice to the open issue.

# 2	Discussion

From [1], the agreements are as follows:

- OTA Transmit OFF power is a co-location requirement

  - The requirement is defined at the co-location reference antenna conductive output side.

- For UTRA, the output power at the co-location reference antenna shall be less than

-85 dBm/MHz - 30 dB + X dB

- For E-UTRA, the output power at the co-location reference antenna shall be less than

-85 dBm/MHz - 30 dB + X dB

where X is a scaling factor. There are two options for how to determine X:

- X = 9 dB for E-UTRA and 6 dB for UTRA

- X = 10log10(M) where M is the number of transceivers in the AAS BS.

As the same issue has been addressed in BS Tx rated output power limit and unwanted emission requirements, we propose to adopt the same solution for the Tx OFF power requirement, which uses a fixed scaling factor.

Proposal 1: For UTRA, -85 dBm/MHz – 30 dB + 6 dB

For E-UTRA, -85 dBm/MHz – 30 dB + 9 dB

# 3	Conclusions

In this document, we propose the following

Proposal 1: UTRA -85 dBm/MHz – 30 dB + 6 dB

E-UTRA -85 dBm/MHz – 30 dB + 9 dB

# References

- R4-1711799, Way forward on transmit ON/OFF power and transient period, CATT, Huawei.