3GPP TSG-RAN WG4 Meeting #85	R4-1713984

Reno, Nevada, USA, 27 November – 1 December 2017

Source:	Ericsson, Telia

Title:	TP to TR 36.715-03-01: CA_3DL_8A-20A-28A_1UL_BCS0

Agenda item:	8.3.2

Document for:	Approval

# 1. Introduction

This contribution provides a TP for TR 36.715-03-01 to complete the requirements for the 3DL/1UL CA combination CA_3DL_8A-20A-28A_1UL_BCS0 as defined in RP-171749 [1].

# 2. Text Proposal for TR 36.715-03-01

##### ---Start of changes---

## 5.x	CA_3DL_8A-20A-28A_1UL_BCS0

### 5.x.1	Operating bands for CA

Table 5.x.1-1: Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_8-20-28 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | 28 | 703 MHz | – | 733 MHz | 758 MHz | – | 788 MHz | FDD |

When included in Table 5.5A-2a, Note 1 need to be applied for this band combination as shown in table 5.x.1_2.

Table 5.x.1-2: Inter-band CA operating bands (three bands)

| E-UTRA CA Band | E-UTRA Band (Table 5.5.1) |
| --- | --- |
| CA_8-20-281 | 8, 20, 28 |
| NOTE 1: The frequency range in band 28 is restricted for this CA band combination to 703-733 MHz for the UL and 758-788 MHz for the DL |  |

### 5.x.2	Channel bandwidths per operating band for CA

Table 5.x.2-1: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz |  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_8A-20A-28A1 | 8 |  |  |  | Yes |  | Yes |  |  | 50 | 0 |
|  | 20 |  |  |  |  |  | Yes | Yes | Yes |  |  |
|  | 28 |  |  |  |  | Yes | Yes | Yes | Yes |  |  |
| NOTE 1: Power imbalance between downlink carriers on Band 20 and Band 28 is assumed to be within [6dB]. |  |  |  |  |  |  |  |  |  |  |  |

### 5.x.3	Co-existence studies

Table 5.x.3-3 summarizes frequency ranges where harmonics occur due to Band 8, Band 20 and Band 28 CA with 1 UL.

Table 5.x.3-3: Impact of 1 UL Harmonic Interference

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 8 | 880 | 915 | 925 | 960 | 1760 | 1830 | 2640 | 2745 |
| 20 | 832 | 862 | 791 | 821 | 1664 | 1724 | 2496 | 2586 |
| 28 | 703 | 733 | 758 | 803 | 1406 | 1466 | 2109 | 2199 |

It can be seen from Table 5.x.3-3 for CA_8A-20A-28A that there is no harmonic interference from uplink bands towards downlink bands.

### 5.x.4	ΔTIB,c and ΔRIB,c values

Examples of possible reference architectures using 2 antennas are shown below. Insertion loss values are applied to all bands to give freedom when choosing which bands to combine in a quadplexer or a triplexer. 1-antenna solutions have also been investigated, but to avoid high insertion loss values with current technologies, a 2-antenna solution seems like the best approach for the time being.

B20+B28

Quadplexer/
Triplexer

B20

B28

B8

B8+B20

Quadplexer/
Triplexer

B8

B20

B28

Above diagrams are not meant as full reference architectures. Diversity paths also need to be considered which will lead to additional 1 or 2 antennas. Which diversity paths chosen are not reflected in above diagrams.

ΔTIB,c values are derived from existing combination CA_8A-20A and CA_20A-28A and are shown in table 5.x.2-1.

Table 5.x.2-1: ΔTIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_8A-20A-28A | 8 | 0.6 |
|  | 20 | 0.5 |
|  | 28 | 0.5 |

ΔRIB,c values are derived from existing combination CA_8A-20A and CA_20A-28A and are shown in table 5.x.2-2.

Table 5.x.2-2: ΔRIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_8A-20A-28A | 8 | 0 |
|  | 20 | 0 |
|  | 28 | 0 |

### 5.x.5	MSD requirements

No MSD requirements is needed for CA_8A-20A-28A.

---End of changes---

# Reference

[1]	RP-171749, “Revised WID: LTE Advanced inter-band CA Rel-15 for 3DL/1UL”, RAN#77, Huawei, HiSilicon