3GPP TSG-RAN WG4 meeting #85	R4-1714001
Reno, NV, USA, 27 Nov – 1 Dec, 2017

Source:	T-Mobile USA

Title:	TP to 38.101-1: N71 specific changes to section 5

Agenda item:	9.4.1.1

Document for:	Approval

## Background

This contribution contains the N71 specific changes for 38.101-1 section 5.  The N71 changes in this section mirror the band 71 requirements found in TS36.101.

## 2. Proposal

It is proposed that the attached text proposal is included in TS 38.101-1.

## References

- RP-172115 Revision of WI: New Radio Access Technology

- R4-1709075, WF on NR spectrum utilization

TEXT PROPOSAL:

< start of text proposal >

# 5	Operating bands and channel arrangement

## 5.2	Operating bands

New Radio is designed to operate in the operating bands defined in Table 5.2-1

Table 5.2-1 NR operating bands

| NR Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |

## 5.3	Channel bandwidth

### 5.3.1	Channel bandwidths per operating band

Table 5.3.1-1: NR channel bandwidth

| NR Band |  | SCS |  | 5 MHz |  | 10 MHz |  | 15 MHz |  | 20 MHz |  | 25 MHz |  | 40 MHz |  | 50 MHz |  | 60 MHz |  | 80 MHz |  | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n71 |  | 15 KHz |  | Yes |  | Yes1 |  | Yes1 |  | Yes1,6 |  | - |  | - |  | - |  | - |  | - |  | - |
| n71 | 30 KHz |  | - |  | - |  | - |  | - |  | - |  | - |  | - |  | - |  | - |  | - |
| n71 | 60 Khz |  | - |  | - |  | - |  | - |  | - |  | - |  | - |  | - |  | - |  | - |
| NOTE 1: Refers to the bandwidth for which a relaxation of the specified UE receiver sensitivity requirement (subclause 7.3) is allowed. NOTE 6: For the 20MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 673-678 MHz or 683-688MHz. |  |  |  |  |  |  |  |  |  |  |  |

[Editors note: the numbering on the notes needs to be revised per the agreements made during RAN4#85]

## 5.4	Channel Arrangement

## [Editors note: the band 71 values in this section need to be revised per the agreements made during RAN4#85]

Table 5.4.3-1: E-UTRA channel numbers

| NR Operating Band | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | FDL_low (MHz) | NOffs-DL | Range of NDL | FUL_low (MHz) | NOffs-UL | Range of NUL |
| n71 | 617 | XXXX | XXXX | 663 | XXXX | XXXX |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. This implies that the first 25, 50, 75 and 100 channel numbers at the lower operating band edge and the last 24, 49, 74 and 99 channel numbers at the upper operating band edge shall not be used for channel bandwidths of 5, 10, 15 and 20 MHz respectively. |  |  |  |  |  |  |

### 5.4.4	TX–RX frequency separation

Table 5.7.4-1: Default UE TX-RX frequency separation

| NR Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| n71 | -46 MHz |
| NOTE 1: Default TX-RX carrier centre frequency separation. |  |

< end of text proposal >