3GPP TSG-RAN WG4 RAN4-85	R4-1714232

Reno, United States, 27 November - 1 December, 2017

Source:		Ericsson

Title:			TP to draft TS 37.114: Introduction of section 8.1 on Emission Test Configurations

Agenda item:		 8.27.3.6

Document for:	Approval

# Introduction

In this contribution we provide TP to section 8 in draft TS 37.114 [1].

# Conclusion

Proposal 1: approve the attached TP to draft TS 37.114 [1].

# References

3GPP Draft TS 37.114 V0.6.0. DRAFT 3GPP TS 37.114 V15.x.x (2017-12)

# TP to the TS 37.114

In this section, TP to the TS 37.114 is proposed for approval.

## ------------------------------ Proposed Text ------------------------------

# 8	Emission

## 8.1	Test configurations

This subclause defines the configurations for emission tests as follows:

-	The equipment shall be tested under normal test conditions as specified in the functional standards;

-	The test configuration shall be as close to normal intended use as possible;

-	If the equipment is part of a system, or can be connected to ancillary equipment, then it shall be acceptable to test the equipment while connected to the minimum configuration of ancillary equipment necessary to exercise the ports;

-	If the equipment has a large number of ports, then a sufficient number shall be selected to simulate actual operation conditions and to ensure that all the different types of termination are tested;

-	The test conditions, test configuration and mode of operation shall be recorded in the test report;

-	Ports which in normal operation are connected shall be connected to an ancillary equipment or to a representative piece of cable correctly terminated to simulate the input/output characteristics of the ancillary equipment, In case of hybrid AAS BS, Radio Frequency (RF) input/output ports shall be correctly terminated;

-	For OTA AAS BS without Radio Frequency (RF) input/output ports but intentionally radiating through the antenna array, the equipment shall be placed in a test setup suitable for the radiated power;

-	Ports which are not connected to cables during normal operation, e.g. service connectors, programming connectors, temporary connectors etc. shall not be connected to any cables for the purpose of EMC testing. Where cables have to be connected to these ports, or interconnecting cables have to be extended in length in order to exercise the EUT, precautions shall be taken to ensure that the evaluation of the EUT is not affected by the addition or extension of these cables;

-	The test arrangements for transmitter and receiver sections of the transceiver are described separately for the sake of clarity. However, where possible the test of the transmitter section and receiver section of the EUT may be carried out simultaneously to reduce test time.

## ------------------------------ End of Proposed Text ------------------------------