3GPP TSG-RAN WG4 Meeting #85                           	         R4-1714333

Reno, US, 27 November – 1 December, 2017

Source: 	Apple

Title:           TP on TR 37.863-01-01 for Single Tx Switched UL

Agenda Item:	  9.4.2

Document for:  Approval

# Introduction

In RAN4#84bis meeting, some agreements were made in [1] for single Tx switch UL. The major points comprise of:

- Inter-band combinations that have IM2 are categorized difficult and inter-band combinations that have IM3 for L-L and H-H combinations are categorized difficult.

- Formula’s are used to define which channel combinations within difficult inter-band combinations are not affected by IM2 and IM3.

  - Concurrent 2 UL is mandatory for these channel combinations.

Besides of the agreements, one issue is still FFS as following:

- RAN4 shall discuss the definition of UL and DL channels in the next meeting

•Alt.1. spectrum holdings of a operator in a band

•Alt.2. aggregated UE channel bandwidth configured by the network

•Alt.3. transmission bandwidth

In order to support this feature, the text proposal for TR37.863-01-01[2] is presented.

# Text Proposal

------------ < First Change > -------------

# 1	Scope

The present document is a technical report for Dual connectivity (DC) band combinations of LTE 1DL/1UL + one NR band under Rel-15 time frame. The purpose is to gather the relevant background information and studies in order to address Dual connectivity (DC) band combinations of LTE 1DL/1UL + one NR band for the Rel-15 band combinations in Table 1-1.

Table 1-1: Release 15 DC band combination of LTE 1DL/1UL + one NR band

| DC combination | REL independent from | Difficult for simultaneous UL(1) |
| --- | --- | --- |
| DC_1A-n77A | Rel-15 | Yes |
| DC_1A-n78A | Rel-15 | No |
| DC_1A-n79A | Rel-15 | No |
| DC_3A-n77A | Rel-15 | Yes |
| DC_3A-n78A | Rel-15 | Yes |
| DC_3A-n79A | Rel-15 | No |
| DC_11A-n77A | Rel-15 | No |
| DC_11A-n78A | Rel-15 | No |
| DC_11A-n79A | Rel-15 | No |
| DC_11A-n257A | Rel-15 | No |
| DC_18A-n77A | Rel-15 | No |
| DC_18A-n78A | Rel-15 | No |
| DC_18A-n79A | Rel-15 | No |
| DC_18A-n257A | Rel-15 | No |
| DC_19A-n77A | Rel-15 | No |
| DC_19A-n78A | Rel-15 | No |
| DC_19A-n79A | Rel-15 | No |
| DC_21A-n77A | Rel-15 | No |
| DC_21A-n78A | Rel-15 | No |
| DC_21A-n79A | Rel-15 | No |
| DC_26A-n77A | Rel-15 | No |
| DC_26A-n78A | Rel-15 | No |
| DC_26A-n79A | Rel-15 | No |
| DC_26A-n257A | Rel-15 | No |
| DC_28A-n77A | Rel-15 | No |
| DC_28A-n78A | Rel-15 | No |
| DC_28A-n79A | Rel-15 | No |
| DC_41A-n257A | Rel-15 | No |
| DC_42A-n77A | Rel-15 | No |
| DC_42A-n78A | Rel-15 | No |
| DC_42A-n79A | Rel-15 | No |
| DC_20A-n28A | Rel-15 | No |
| DC_39A-n78A | Rel-15 | No |
| DC_39A-n258A | Rel-15 | No |
| DC_1A_n257 | Rel-15 | No |
| DC_3A_n257 | Rel-15 | No |
| DC_3A-n258A | Rel-15 | No |
| DC_5A-n257A | Rel-15 | No |
| DC_7A-n78 | Rel-15 | No |
| DC_7A-n258A | Rel-15 | No |
| DC_8A-n78A | Rel-15 | No |
| DC_8A-n79A | Rel-15 | No |
| DC_8A-n258A | Rel-15 | No |
| DC_19A-n257 | Rel-15 | No |
| DC_20A-n78A | Rel-15 | No |
| DC_21A-n257 | Rel-15 | No |
| DC_28A-n257 | Rel-15 | No |
| DC_28A-n258A | Rel-15 | No |
| DC_38A-n78A | Rel-15 | No |
| DC_39A-n79A | Rel-15 | No |
| DC_41A-n77A | Rel-15 | No |
| DC_41A-n78A | Rel-15 | No |
| DC_41A-n79A | Rel-15 | No |
| DC_41A-n258A | Rel-15 | No |
| DC_42A-n257 | Rel-15 | No |
| DC_1A-n28 | Rel-15 | No |
| DC_3A-n7 | Rel-15 | No |
| DC_3A-n28 | Rel-15 | No |
| DC_5A-n78A | Rel-15 | No |
| DC_7A_n28 | Rel-15 | No |
| DC_7A-n257A | Rel-15 | No |
| DC_66A-n257A | Rel-15 | No |

Note(1): Details on how to support it can be found in Section 5.X.

This TR contains a general part and band specific combination part. The actual requirements are added to the corresponding technical specifications.

---------- < Second Change >----------

5  DC band combination of LTE 1DL/1UL + one NR band: General Part

--------<Unchanged texts are omitted>---------

5. X  Single Tx Switched UL

In some LTE-NR inter-band combinations, as indicated in the “Difficult for simultaneous UL” column in Table 1-1, it is difficult for UE to perform simultaneous LTE+NR UL transmission. The band combinations in which the 2nd order IMD of the simultaneous UL bands fall into its own Rx band are categorized as difficult. And the band combinations for L-L (<1GHz) and H-H (1.695GHz-2.695GHz) in which the 3rd order IMD of the simultaneous UL bands fall into its own Rx band are categorized as difficult. For non-difficult band combinations, simultaneous UL transmission is mandatory.

The following formulas define which channel combinations within difficult band combinations are not affected by IM2 and IM3. Simultaneous UL transmission is mandatory for the unaffected channel combinations in difficult band combinations.

Interference bandwidth: IBW = |a| x CBW1 + |b| x CBW2

|a| + |b| = 2 (or 3)

CBW1 and CBW2 are UL channels

Center frequency of IBW:  fIBW = a x f1 + b x f2

f1 and f2 are center frequency of each UL channel

The range of IMD 2 (or 3):  [fIBW – IBW/2, fIBW + IBW/2]

Note1: The DL channel and UL channels are the transmission bandwidth, on which UE shall be able to apply operations which are configured by RRC reconfiguration and corresponding HARQ timing.

Note 2: For identified difficult band combination, during two adjacent RRC reconfiguration, the changing of transmission bandwidth should not introduce IM2 and IM3, which will result in UE changing from 2Tx to 1Tx. Otherwise, UE behavior is not specified.

---------- < End of Change >----------

# Reference

- R4-1711618,   WF on single Tx switched UL,        Apple,     RAN4#84bis meeting

- TR 37.863-01-01_V0.3.0_DC band combinations of LTE 1DL1UL+one NR band