3GPP TSG-RAN WG4 #85	R4-1714337

November 27st – December 1st, 2017

Reno, Neveda, USA

Agenda item:	9.4.4.4.2

Source: 	Qualcomm Incorporated

Title: 	TP to TS 38.101-2: ACS requirement for mmW (section 7.5)

Document for:	Approval

# Introduction

Adjacent channel selectivity (ACS) requirement for range 2 NR were already agreed in case of single carrier transmission [1][2].

In this contribution, we propose ACS requirements for carrier aggregation (CA) and we make a text proposal for section 7.5 of 38.101-2.

# Discussion

In the following, we will make specific proposal to finalize the ACS requirements including tests specifications for the CA cases. The following sections are organized as follows:

- Summary of single carrier requirements

- Proposals for CA requirements

- Text Proposals for 38.801-2 specifications

## ACS: single carrier requirement

ACS single carrier requirement was agreed long time ago [1], with the two key aspects:

- ACS = 23dB

- Jammer bandwidth is equal to the wanted signal bandwidth, as shown in Figure 1. Pictorial representation of wanted and ACS test signals for the single carrier case.

Figure 1. Pictorial representation of wanted and ACS test signals for the single carrier case.

The ACS requirement and tests has been recently specified in the version of TS 38.101 approved through email discussion [3].

## ACS: Proposals for CA requirements

Proposals for CA requirements

Three different cases need to be taken into account when defining ACS requirements:

- Intra-band contiguous CA

- Intra-band non-contiguous CA

- Inter band CA

### Intra-band contiguous CA

In this case, we want to apply the same principle already discussed for IBB test cases, i.e. that requirement is based on the aggregated channel BW. In other words, the jammer bandwidth would be equal to the total aggregated bandwidth of the wanted signal and the ACS requirement is same as the single carrier case, as depicted in Figure 1.

Proposal 1: for the intra-band contiguous case, the ACS signal bandwidth is equal to the aggregated wanted signal bandwidth.

Figure 2. Pictorial representation of wanted and ACS test signals for the intra-band contiguous CA case.

Regarding the values of wanted and unwanted signals, we propose the following:

Proposal 2: for the intra-band contiguous CA case, the ACS test should be specified as follows:

- Power in transmission bandwidth configuration per CC = REFSENS + 14dB

- Pinterfer = aggregated power + [21.5] dB for band n257, n258

- Pinterfer = aggregated power + [20.5] dB for band n260

Note that the principle of the above proposals is to keep the same PSD difference between wanted and unwanted signal, as for the single carrier case.

### Intra-band non-contiguous CA

For intra-band non-contiguous CA there are two approaches that can be adopted. One possibility would be to define the ACS test case following the principle adopted for the contiguous CA case, i.e. defining the test based on the aggregated end-to-end bandwidth. In this case, the test will not apply in the gap between carriers. One other approach is to define the test based on the single component carrier bandwidth and thus the test can apply within the gap.

Since in mmW bands are very wide, the gap could be very large. As a consequence, to avoid a situation in which requirement is not defined in a large chunk of spectrum, and to also avoid having very large test signal BW, our preference is to define the ACS test case for intra-band non-contiguous CA based on the component carriers channel BW. In particular, one option is to consider the largest component carrier as reference for the test definition. In this case, a restriction on the minimum gap needs to be in place. Let us consider two component carriers, namely CC1 and CC2, with bandwidth equal to BW1 and BW2, respectively. Considering the largest channel BW, the minimum frequency separation between the two carriers needed to define the ACS test is given by ∆fACS ≥ BW1/2 + BW2/2 + max(BW1, BW2). Our proposal is the following

Proposal 3: for the intra-band non-contiguous CA case, the ACS test should be specified as follows

- Within the gap the test is defined only if ∆fACS ≥ BW1/2 + BW2/2 + max(BW1, BW2) and the tests parameters for the single carrier applies considering the largest channel bandwidth between BW1 and BW2

- Out of the gap same parameters as the single carrier case applies

An illustration of the proposed test cases for intra-band non-contiguous CA is depicted in Figure 3. Note that signal levels in the picture are not scaled according to the different PSD.

Figure 3. Pictorial representation of wanted and ACS test signals for the intra-band non-contiguous CA case.

## Inter-band CA

For the inter-band CA case, we propose to use the same test as the single carrier case apply to each component carrier

Proposal 4: for the intra-band non-contiguous CA ACS test case, the single carrier test applies to each component carrier.

# Text Proposal for TS 38.801-2

In this section we provide text proposal to 38.801-2.

Proposal 5: to approve the following Text Proposal to TS 38.801-2.

<<<<<<<<  Begin Text Proposal to TS 38.101-2   >>>>>>>>

## 7.5	Adjacent Channel Selectivity

Adjacent Channel Selectivity (ACS) is a measure of a receiver's ability to receive a NR signal at its assigned channel frequency in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel. ACS is the ratio of the receive filter attenuation on the assigned channel frequency to the receive filter attenuation on the adjacent channel(s).

### 7.5.1	Minimum requirements

The UE shall fulfil the minimum requirement specified in Table 7.5.1-1 for all values of an adjacent channel interferer up to –25 dBm. However it is not possible to directly measure the ACS, instead the lower and upper range of test parameters are chosen in Table 7.5.1-2 and Table 7.5.1-3 where the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.X.X, A.X.X and A.X.X (with QPSK, R=X/X and one sided dynamic OCNG Pattern OP.1 TDD for the DL-signal as described in Annex A.X.X.X). For operating bands with an unpaired DL part (as noted in Table 5.5-1), the requirements only apply for carriers assigned in the paired part.

Table 7.5.1-1: Adjacent channel selectivity

|  |  | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Rx Parameter | Units | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| ACS for band n257, n258 | dB | 23 | 23 | 23 | 23 |
| ACS for band n260 | dB | 22 | 22 | 22 | 22 |

Table 7.5.1-2: Test parameters for Adjacent channel selectivity, Case 1

| Rx Parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + 14 dB |  |  |  |
| PInterferer for band n257, n258 | dBm | REFSENS + 35.5 dB | REFSENS +35.5dB | REFSENS +35.5dB | REFSENS +35.5dB |
| PInterferer for band n260 | dBm | REFSENS + 34.5 dB | REFSENS +34.5dB | REFSENS +34.5dB | REFSENS +34.5dB |
| BWInterferer | MHz | 50 | 100 | 200 | 400 |
| FInterferer (offset) | MHz | 50 / -50 NOTE 3 | 100 / -100 NOTE 3 | 200 / -200 NOTE 3 | 400 / -400 NOTE 3 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex A.X.X with one sided dynamic OCNG Pattern as described in Annex A.X.X.X and set-up according to Annex C.3.1. NOTE 2: The REFSENS power level is specified in Table XXXX. NOTE 3: The absolute value of the interferer offset FInterferer (offset) shall be further adjusted to FInterferer/SCS+0.5SCS MHz with SCS the sub-carrier spacing of the wanted signal in MHz. Wanted and interferer signal have same SCS. |  |  |  |  |  |

Table 7.5.1-3: Test parameters for Adjacent channel selectivity, Case 2

| Rx Parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Power in Transmission Bandwidth Configuration for band n257, n258 | dBm | -46.5 | -46.5 | -46.5 | -46.5 |
| Power in Transmission Bandwidth Configuration for band n260 | dBm | -45.5 | -45.5 | -45.5 | -45.5 |
| PInterferer | dBm | -25 |  |  |  |
| BWInterferer | MHz | 50 | 100 | 200 | 400 |
| FInterferer (offset) | MHz | 50 / -50 NOTE 2 | 100 / -100 NOTE 2 | 200 / -200 NOTE 2 | 400 / -400 NOTE 2 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex X.X with one sided dynamic OCNG Pattern TDD as described in Annex A.X.X.X and set-up according to Annex C.X.X. NOTE 2: The absolute value of the interferer offset FInterferer (offset) shall be further adjusted to FInterferer/SCS+0.5SCS MHz with SCS the sub-carrier spacing of the wanted signal in MHz. Wanted and interferer signal have same SCS. |  |  |  |  |  |

### 7.5.1A	Minimum requirements for CA

For intra-band non-contiguous carrier aggregation with two component carriers and aggregated bandwidth BWChannel_CA up to 400MHz, two different requirements apply for out-of-gap and in-gap. For out-of-gap, the UE shall meet the requirements for each component carrier as specified in subclauses 7.5.1. For in-gap, the requirement shall apply if the following minimum gap condition is met: ∆fACS ≥ BW1/2 + BW2/2 + max(BW1, BW2), where ∆fACS is the frequency separation between the center frequencies of the component carriers and BWk/2 are the channel bandwidths of carrier k, k = 1,2. If the minimum gap condition is met, for each component carriers the UE shall meet the in-gap requirements specified in in subclauses 7.5.1 considering parameters associated to the largest channel bandwidth between the two component carriers, i.e. max(BW1, BW2). For both out-of-gap and in-gap, the requirements shall apply while all downlink carriers are active.

For intra-band non-contiguous carrier aggregation with more than two component carriers or aggregated bandwidth BWChannel_CA larger than 400MHz the requirement is FFS.

For intra-band contiguous carrier aggregation with two component carriers and aggregated bandwidth BWChannel_CA up to 400MHz, the SCC(s) shall be configured at nominal channel spacing tothe PCC. The UE shall fulfil the minimum requirement specified in Table 7.5.1A-1 for an adjacent channel interferer on either side of the aggregated downlink signal at a specified frequency offset and for an interferer power up to -25 dBm.

The throughput of each carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.X.X  (with one sided dynamic OCNG Pattern for the DL-signal as

described in Annex A.X.X.X) with parameters specified in Tables 7.5.1A-2 and 7.5.1A-3.

For intra-band contiguous carrier aggregation with more than two component carriers or aggregated bandwidth BWChannel_CA larger 400MHz the requirement is FFS.

Table 7.5.1-A-1: Adjacent channel selectivity for CA

|  |  | CA Bandwidth Class |
| --- | --- | --- |
| Rx Parameter | Units | Any CA bandwidth Classes |
| ACS for band n257, n258 | dB | 23 |
| ACS for band n260 | dB | 22 |

Table 7.5.1A-2: Adjacent channel selectivity test parameters for CA, Case 1

| Rx Parameter | Units | CA Bandwidth Class |
| --- | --- | --- |
|  |  | All CA bandwidth Classes |
| Pw in Transmission Bandwidth Configuration, per CC |  | REFSENS + 14 dB |
| Pinterferer for band n257, n258 | dBm | Aggregated power + 21.5 |
| Pinterferer for band n260 | dBm | Aggregated power + 20.5 |
| BWInterferer | MHz | BWChannel_CA |
| FInterferer (offset) | MHz | BWChannel_CA NOTE 3 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex X.X with one sided dynamic OCNG Pattern as described in Annex A.X.X.X and set-up according to Annex C.X.X. NOTE 2: The Finterferer (offset) is the frequency separation between the center of the aggregated CA bandwidth and the center frequency of the Interferer signal NOTE 3: The absolute value of the interferer offset FInterferer (offset) shall be further adjusted to FInterferer/SCS+0.5SCS MHz with SCS the sub-carrier spacing of the wanted signal in MHz. Wanted and interferer signal have same SCS. |  |  |

Table 7.5.1A-3: Adjacent channel selectivity test parameters for CA, Case 2

| Rx Parameter | Units | CA Bandwidth Class |
| --- | --- | --- |
|  |  | All CA bandwidth Classes |
| Pw in Transmission Bandwidth Configuration, aggregated power for band n257, n258 | dBm | -46.5 |
| Pw in Transmission Bandwidth Configuration, aggregated power for band n260 | dBm | -45.5 |
| Pinterferer for band n260 | dBm | -25 |
| BWInterferer | MHz | BWChannel_CA |
| FInterferer (offset) | MHz | BWChannel_CA NOTE 3 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex X.X with one sided dynamic OCNG Pattern as described in Annex A.X.X.X and set-up according to Annex C.X.X. NOTE 2: The Finterferer (offset) is the frequency separation between the center of the aggregated CA bandwidth and the center frequency of the Interferer signal NOTE 3: The absolute value of the interferer offset FInterferer (offset) shall be further adjusted to FInterferer/SCS+0.5SCS MHz with SCS the sub-carrier spacing of the wanted signal in MHz. Wanted and interferer signal have same SCS. |  |  |

<<<<<<<<  End Text Proposal to TS 38.101-2   >>>>>>>>

# Reference

- R4-1706063, “WF on mmW ACLR and ACS”, Qualcomm Incorporated, Huawei.

- R4-1706526, “UE ACS for mmWave”, Intel Corporation.

- R4-1711979, “3GPP TS 38.101-2 v0.1.0 (2017-10)”.