3GPP TSG-RAN WG4 Meeting #85	R4-1714374

Reno, USA, 27th November – 1st December 2017

Agenda item:	9.2.1.2

Source: 	CMCC

Title: 	TP to TR 38.813 NR band for n77 and n78

Document for:	Approval

# 1. Introduction

RAN4 has approved some WF on NR frequency band, band numbering, channel bandwidth, ACLR and SEM of Band n77 and n78 [1].

This paper is a text proposal to capture the agreements, background and updated spurious emission band UE co-existence for Band n77 and n78 for TR 38.813.

# 2.	References

[1]	R4-1706893, “WF on 3.5GHz NR band definition”, CMCC, Vodafone,  Ericsson,  Qualcomm, Skyworks, Huawei, HiSilicon, CATT, ZTE, Telecom Italia, Orange, Deutsche Telekom, BT, Broadcom, China Telecom, China Unicom

[2]	R4-1709181, “WF on band numbering”, NTT DOCOMO

[3] R4-1708845, “WF on UE mandatory channel bandwidth”, Nokia

[4] R4-1711732, “WF on BS channel BW set”, Huawei, Hisilicon, Vodafone, Ericsson

[5]	R4-1710957, “TP to TR 38.817-01: Futher ACLR agreements”, Nokia

[6]	R4-1710962, “TP to TS 38.101-1 Output RF spectrum emissions”, Nokia

# Text proposal for TR 38.813

----- Start of TP -----

<Unchanged sections omitted>

- Scope

The present document is a technical report for New frequency range for NR (3.3-4.2 GHz). The purpose of this technical report is to provide specification support for NR bands 3.3-3.8 GHz and 3.3-4.2 GHz.

4	Background

In 3GPP RAN4 #AH2 meeting, it is proposed that new bands for NR are assigned band numbers on a “first come first served” basis in reserved ranges regardless of duplex mode or RAT. Then, in RAN4 #84 meeting, band 3.3-3.8 GHz and band 3.3-4.2GHz are assigned band number n78 and n77 respectively.

5	NR Frequency band definition

In 3GPP, RAN 4 specified two NR operating bands n77 and n78 in 3.3GHz-4.2GHz. A UE supporting n78 band is not required to support n77 band [1].

6	Band numbering and channel bandwidth

6.1	Band numbering

For 3.3-4.2GHz frequency range the NR frequency bands are defined as in Table 6.1-1.

Table 6.1-1: NR frequency bands

| NR Operating Band | Uplink (UL) operating band BS receive UE transmit | Downlink (DL) operating band BS transmit UE receive | Duplex Mode |
| --- | --- | --- | --- |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n77 | 3300MHz-4200MHz | 3300MHz-4200MHz | TDD |
| n78 | 3300MHz-3800MHz | 3300MHz-3800MHz | TDD |

6.2	channel bandwidth

6.2.1	Channel bandwidth Set [NR_newRAT]

The superset of channel bandwidth for UE was agreed in Table 6.2.1-1:

Table 6.2.1-1 UE channel bandwidth set

| NR Band | Data SCS = 15kHz |  |  |  |  | Data SCS = 30kHz |  |  |  |  |  |  |  | Data SCS = 60kHz (for more than 1GHz bands) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 MHZ | 15 MHz | 20 MHZ | 40 MHz | 50 MHz | 10 MHZ | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | 10 MHz (NOTE) | 15 MHz | 20 MHz | 40 MH z | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| n77 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |
| n78 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |
| NOTE: 90% spectrum utilization may not be achieved |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

For band n77 and band n78, addition to the UE channel BWs for the band, 30 MHz, 70 MHz and 90 MHz are supported for BS. These new BS CBWs have low priority compared to the UE channel BWs.

7	NR band 3.3GHz - 3.8GHz specific issues

7.1	UE specific

7.1.1	Transmitter characteristics

7.1.1.4	Adjacent Channel Leakage Ratio (ACLR)

NR adjacent channel leakage power ratio (NRACLR) is the ratio of the filtered mean power centred on the assigned NR channel frequency to the filtered mean power centred on an adjacent NR channel frequency at nominal channel spacing.

The assigned NR channel power and adjacent NR channel power are measured with rectangular filters with measurement bandwidths specified in Table 7.1.1.4-1.

If the measured adjacent channel power is greater than [–50dBm] then the NRACLR shall be higher than the value specified in Table 7.1.1.4-2.

Table 7.1.1.4-1: NR ACLR measurement bandwidth

|  | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR ACLR measurement bandwidth | 9.375 | 14.235 | 19.095 | 38.895 | 48.615 | 58.35 | 78.15 | 98.31 |

Table 7.1.1.4-2: NR ACLR requirement

|  | Power class 1 | Power class 2 | Power class 3 |
| --- | --- | --- | --- |
| NRACLR |  | 31 dB | 30 dB |

7.1.1.5	Spectrum emission mask

The power of any UE emission shall not exceed the levels specified in Table 7.1.1.5-1 for the specified channel bandwidth.

Table 7.1.1.5-1: NR General spectrum emission mask

| ΔfOOB (MHz) | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ± 0-1 | -18 | -20 | -21 | -24 | -24 | -24 | -24 | -24 | 30 kHz |
| ± 1-5 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| ± 5-6 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 |  |
| ± 6-10 |  |  |  |  |  |  |  |  |  |
| ± 10-15 | -25 |  |  |  |  |  |  |  |  |
| ± 15-20 |  | -25 |  |  |  |  |  |  |  |
| ± 20-25 |  |  | -25 |  |  |  |  |  |  |
| ± 25-30 |  |  |  |  |  |  |  |  |  |
| ± 30-40 |  |  |  |  |  |  |  |  |  |
| ± 40-45 |  |  |  | -25 |  |  |  |  |  |
| ± 45-50 |  |  |  |  |  |  |  |  |  |
| ± 50-55 |  |  |  |  | -25 |  |  |  |  |
| ± 55-60 |  |  |  |  |  |  |  |  |  |
| ± 60-65 |  |  |  |  |  | -25 |  |  |  |
| ± 65-80 |  |  |  |  |  |  |  |  |  |
| ± 80-85 |  |  |  |  |  |  | -25 |  |  |
| ± 85-100 |  |  |  |  |  |  |  |  |  |
| ± 100-105 |  |  |  |  |  |  |  | -25 |  |

7.1.1.6	Spurious emissions

Since Band n77 and/or n78 will be used at least in Japan, bands operated in Japan need to be protected. In addition, protected bands required in Europe, Korea and China regions (i.e., operating bands specified for Band n78 of 3.3-3.8 GHz) should also be added for potential use of these bands. As a consequence, required protected bands for Band n77 and n78 will be the same. Note that it was already agreed not to specify the co-existence requirement between Band n77 and Band n79, which means that the co-existence will be guaranteed by the NR general emission requirements.

Table 7.1.1.6-1: Spurious emission band UE co-existence for Band n77 and n78

| NR band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | PHS |
|  | NR Band n257 | 26500 | - | 29500 | [-5] | 100 |  |

Agreement: Protected bands required for Band n77 are to be the same as those for Band n78 (i.e., not only Japanese bands but also ones required for other regions such as Europe, Korea and China)

7.1.2	Receiver characteristics

7.1.2.2	Adjacent Channel Selectivity (ACS)

Adjacent Channel Selectivity (ACS) is a measure of a receiver's ability to receive a NR signal at its assigned channel frequency in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel. ACS is the ratio of the receive filter attenuation on the assigned channel frequency to the receive filter attenuation on the adjacent channel(s).

It is not possible to directly measure the ACS, instead the lower and upper range of test parameters are chosen in Table 7.1.2.2-2 and Table 7.1.2.2-3 where the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in TS 38.101 Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in TS 38.101 Annex A.5.1.1/A.5.2.1).

Table 7.1.2.2-1: Adjacent channel selectivity

| NR band | Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| n78 | ACS | dB | [33.0] | [33.0] | [33.0] | [33.0] | [33.0] | [33.0] | [33.0] | [33.0] |

Table 7.1.2.2-2: Test parameters for Adjacent channel selectivity, Case 1

| NR band | Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| n78 | Power in Transmission Bandwidth Configuration | dBm | REFSENS + 14 dB |  |  |  |  |  |  |  |
|  | PInterferer | dBm | REFSENS [+45.5]dB | REFSENS [+45.5] dB | REFSENS [+45.5]dB | REFSENS [+45.5]dB | REFSENS [+45.5]dB | REFSENS [+45.5]dB | REFSENS [+45.5]dB | REFSENS [+45.5]dB |
|  | BWInterferer | MHz | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 |
|  | FInterferer (offset) For SCS of 15kHz | MHz | 10+0.0125 / -10-0.0125 | 15+0.0075 / -15-0.0075 | 20+0.0025 / -20-0.0025 | 40+0.0125 / -40-0.0125 | 50+0.0025 / -50-0.0025 | N/A | N/A | N/A |
|  | FInterferer (offset) For SCS of 30kHz | MHz | 10+0.005 / -10-0.005 | 15+0.015 / -15-0.015 | 20+0.025 / -20-0.025 | 40+0.005 / -40-0.005 | 50+0.025 / -50-0.025 | 60+0.015 / -60-0.015 | 80+0.025 / -80-0.025 | 100+0.005 / -100-0.005 |
|  | FInterferer (offset) For SCS of 60kHz | MHz | 10+0.050 / -10-0.050 | 15+0.030 / -15-0.030 | 20+0.010 / -20-0.010 | 40+0.050 / -40-0.050 | 50+0.010 / -50-0.010 | 60+0.030 / -60-0.030 | 80+0.010 / -80-0.010 | 100+0.050 / -100-0.050 |

Table 7.1.2.2-3: Test parameters for Adjacent channel selectivity, Case 2

| NR band | Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 10 MHz | 15 MHz |  | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| n78 | Power in Transmission Bandwidth Configuration | dBm | [-56.5] | [-56.5] |  | [-56.5] | [-56.5] | [-56.5] | [-56.5] | [-56.5] | [-56.5] |
|  | PInterferer | dBm | -25 |  |  |  |  |  |  |  |  |
|  | BWInterferer | MHz | 10 | 15 | 20 |  | 40 | 50 | 60 | 80 | 100 |
|  | FInterferer (offset) For SCS of 15kHz | MHz | 10+0.0125 / -10-0.0125 | 15+0.0075 / -15-0.0075 | 20+0.0025 / -20-0.0025 |  | 40+0.0125 / -40-0.0125 | 50+0.0025 / -50-0.0025 | NA | NA | NA |
|  | FInterferer (offset) For SCS of 30kHz | MHz | 10+0.005 / -10-0.005 | 15+0.015 / -15-0.015 | 20+0.025 / -20-0.025 |  | 40+0.005 / -40-0.005 | 50+0.025 / -50-0.025 | 60+0.015 / -60-0.015 | 80+0.025 / -80-0.025 | 100+0.005 / -100-0.005 |
|  | FInterferer (offset) For SCS of 60kHz | MHz | 10+0.050 / -10-0.050 | 15+0.030 / -15-0.030 | 20+0.010 / -20-0.010 |  | 40+0.050 / -40-0.050 | 50+0.010 / -50-0.010 | 60+0.030 / -60-0.030 | 80+0.010 / -80-0.010 | 100+0.050 / -100-0.050 |

8	NR band 3.3GHz - 4.2GHz specific issues

8.1	UE specifc

8.1.1	Transmitter characteristics

8.1.1.4	Adjacent Channel Leakage Ratio (ACLR)

NR adjacent channel leakage power ratio (NRACLR) is the ratio of the filtered mean power centred on the assigned NR channel frequency to the filtered mean power centred on an adjacent NR channel frequency at nominal channel spacing.

The assigned NR channel power and adjacent NR channel power are measured with rectangular filters with measurement bandwidths specified in Table 8.1.1.4-1.

If the measured adjacent channel power is greater than [–50dBm] then the NRACLR shall be higher than the value specified in Table 8.1.1.4-2.

Table 8.1.1.4-1: NR ACLR measurement bandwidth

|  | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR ACLR measurement bandwidth | 9.375 | 14.235 | 19.095 | 38.895 | 48.615 | 58.35 | 78.15 | 98.31 |

Table 8.1.1.4-2: NR ACLR requirement

|  | Power class 1 | Power class 2 | Power class 3 |
| --- | --- | --- | --- |
| NRACLR |  | 31 dB | 30 dB |

8.1.1.5	Spectrum emission mask

The power of any UE emission shall not exceed the levels specified in Table 8.1.1.5-1 for the specified channel bandwidth.

Table 8.1.1.5-1: NR General spectrum emission mask

| ΔfOOB (MHz) | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ± 0-1 | -18 | -20 | -21 | -24 | -24 | -24 | -24 | -24 | 30 kHz |
| ± 1-5 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| ± 5-6 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 |  |
| ± 6-10 |  |  |  |  |  |  |  |  |  |
| ± 10-15 | -25 |  |  |  |  |  |  |  |  |
| ± 15-20 |  | -25 |  |  |  |  |  |  |  |
| ± 20-25 |  |  | -25 |  |  |  |  |  |  |
| ± 25-30 |  |  |  |  |  |  |  |  |  |
| ± 30-40 |  |  |  |  |  |  |  |  |  |
| ± 40-45 |  |  |  | -25 |  |  |  |  |  |
| ± 45-50 |  |  |  |  |  |  |  |  |  |
| ± 50-55 |  |  |  |  | -25 |  |  |  |  |
| ± 55-60 |  |  |  |  |  |  |  |  |  |
| ± 60-65 |  |  |  |  |  | -25 |  |  |  |
| ± 65-80 |  |  |  |  |  |  |  |  |  |
| ± 80-85 |  |  |  |  |  |  | -25 |  |  |
| ± 85-100 |  |  |  |  |  |  |  |  |  |
| ± 100-105 |  |  |  |  |  |  |  | -25 |  |

8.1.1.6	Spurious emissions

Based on the same discussion as Spurious emissions of Band n78 described in clause 8.1.1.6, the following was agreed.

Table 8.1.1.6-1: Spurious emission band UE co-existence for Band n77 and n78

| NR band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| n77 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | PHS |
|  | NR Band n257 | 26500 | - | 29500 | [-5] | 100 |  |

Agreement: Protected bands required for Band n77 are to be the same as those for Band n78 (i.e., not only Japanese bands but also ones required for other regions such as Europe, Korea and China)

8.1.2	Receiver characteristics

8.1.2.2	Adjacent Channel Selectivity (ACS)

Adjacent Channel Selectivity (ACS) is a measure of a receiver's ability to receive a NR signal at its assigned channel frequency in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel. ACS is the ratio of the receive filter attenuation on the assigned channel frequency to the receive filter attenuation on the adjacent channel(s).

It is not possible to directly measure the ACS, instead the lower and upper range of test parameters are chosen in Table 8.1.2.2-2 and Table 8.1.2.2-3 where the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in TS 38.101 Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in TS 38.101 Annex A.5.1.1/A.5.2.1).

Table 8.1.2.2-1: Adjacent channel selectivity

| NR band | Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| n77 | ACS | dB | [33.0] | [33.0] | [33.0] | [33.0] | [33.0] | [33.0] | [33.0] | [33.0] |

Table 8.1.2.2-2: Test parameters for Adjacent channel selectivity, Case 1

| NR band | Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 10 MHz | 15 MHz | 20 MHz | 40MHz | 50 MHz | 60MHz | 80MHz | 100 MHz |
| n77 | Power in Transmission Bandwidth Configuration | dBm | REFSENS + 14 dB |  |  |  |  |  |  |  |
|  | PInterferer | dBm | REFSENS [+45.5]dB | REFSENS [+45.5]dB | REFSENS [+45.5]dB | REFSENS [+45.5]dB | REFSENS [+45.5]dB | REFSENS [+45.5]dB | REFSENS [+45.5]dB | REFSENS [+45.5]dB |
|  | BWInterferer | MHz | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 |
|  | FInterferer (offset) For SCS of 15kHz | MHz | 10+0.0125 / -10-0.0125 | 15+0.0075 / -15-0.0075 | 20+0.0025 / -20-0.0025 | 40+0.0125 / -40-0.0125 | 50+0.0025 / -50-0.0025 | N/A | N/A | N/A |
|  | FInterferer (offset) For SCS of 30kHz | MHz | 10+0.005 / -10-0.005 | 15+0.015 / -15-0.015 | 20+0.025 / -20-0.025 | 40+0.005 / -40-0.005 | 50+0.025 / -50-0.025 | 60+0.015 / -60-0.015 | 80+0.025 / -80-0.025 | 100+0.005 / -100-0.005 |
|  | FInterferer (offset) For SCS of 60kHz | MHz | 10+0.050 / -10-0.050 | 15+0.030 / -15-0.030 | 20+0.010 / -20-0.010 | 40+0.050 / -40-0.050 | 50+0.010 / -50-0.010 | 60+0.030 / -60-0.030 | 80+0.010 / -80-0.010 | 100+0.050 / -100-0.050 |

Table 8.1.2.2-3: Test parameters for Adjacent channel selectivity, Case 2

| NR band | Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 10 MHz | 15 MHz | 20 MHz | 40MHz | 50 MHz | 60MHz | 80MHz | 100 MHz |
| n77 | Power in Transmission Bandwidth Configuration | dBm | [-56.5] | [-56.5] | [-56.5] | [-56.5] | [-56.5] | [-56.5] | [-56.5] | [-56.5] |
|  | PInterferer | dBm | -25 |  |  |  |  |  |  |  |
|  | BWInterferer | MHz | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 |
|  | FInterferer (offset) For SCS of 15kHz | MHz | 10+0.0125 / -10-0.0125 | 15+0.0075 / -15-0.0075 | 20+0.0025 / -20-0.0025 | 40+0.0125 / -40-0.0125 | 50+0.0025 / -50-0.0025 | N/A | N/A | N/A |
|  | FInterferer (offset) For SCS of 30kHz | MHz | 10+0.005 / -10-0.005 | 15+0.015 / -15-0.015 | 20+0.025 / -20-0.025 | 40+0.005 / -40-0.005 | 50+0.025 / -50-0.025 | 60+0.015 / -60-0.015 | 80+0.025 / -80-0.025 | 100+0.005 / -100-0.005 |
|  | FInterferer (offset) For SCS of 60kHz | MHz | 10+0.050 / -10-0.050 | 15+0.030 / -15-0.030 | 20+0.010 / -20-0.010 | 40+0.050 / -40-0.050 | 50+0.010 / -50-0.010 | 60+0.030 / -60-0.030 | 80+0.010 / -80-0.010 | 100+0.050 / -100-0.050 |

<Unchanged sections omitted>

----- End of TP -----