3GPP TSG-RAN WG4 Meeting #85                 	R4-1714491

Reno, USA, 27 November – 1 December 2017

Source:	Nokia, Nokia Shanghai Bell

Title:	TP for TS 38.133 on inter-frequency measurement requirements

Agenda item:	9.7.1.2

Document for:	Approval

# 1	Introduction

A text proposal to specify the inter-frequency measurements Section 9.4 inter-frequency measurement without measurement gaps for TS 38.133.

# 2	TP to TS on Inter-frequency measurements

Section 9.4 inter-frequency measurement without measurement gaps in TS 38.133 is updated as follows:

<<<Start of Changes>>>

## 9.4	NR inter-frequency measurements

Editor’s note: DRX and non DRX requirement might be separately defined in this section. The numerology and BW combinations might be reflected in the requirement table.

A measurement is defined as a SSB based inter-frequency measurement provided it is not defined as in intra-frequency measurement according to section 9.2.

SSB based measurements are configured along with a measurement timing configuration (SMTC) per carrier, which provides periodicity, duration and offset information on a window of up to 5ms where the measurements on the configured inter-frequency carrier are to be performed. For inter-frequency connected mode measurements, one measurement window periodicities may be configured per inter-frequency measurement object.

When measurement gaps are needed, the UE is not expected to detect SSB on an inter-frequency measurement object which start earlier than the gap starting time + switching time, nor detect SSB which end later than the gap end – switching time. When the inter-frequency cells are in FR2 and the per-FR gap is configured to the UE, the switching time is 0,25ms Otherwise the switching time is 0.5ms.

The UE shall be able to identify new inter-frequency cells and perform SS-RSRP, SS-RSRQ, and SS-SINR measurements of identified inter-frequency cells if carrier frequency information is provided by [the PSCell], even if no explicit neighbour list with physical layer cell identities is provided.

A cell shall be considered detectable when:

- [SS-RSRP] related side conditions given in Section [TBD] are fulfilled for a corresponding Band,

- [SS-RSRQ] related side conditions given in Clause [TBD] are fulfilled for a corresponding Band,

- [SS-SINR] related side conditions given in Section [TBD] are fulfilled for a corresponding Band,

- [SCH_RP] and [SCH Ês/Iot] according to Annex [TBD] for a corresponding Band.

Editors note: requirements for both per UE measurement gap and per-FR measurement gap will be captured in this section.

### 9.4.1	NR Inter frequency cell identification

When measurement gaps are provided, or the UE supports capability of conducting such measurements without gaps, the UE shall be able to identify a new detectable inter-frequency cell within [TIdentify_Inter] according to the following expression:

TBD [Editor’s note: Requirements for both non-DRX and DRX would be captured here]

### 9.4.2		NR Inter frequency SS-Block detection of a newly detected cell

The UE shall be able to identify a SS-Block on a newly identified inter-frequency cell according to the requirements in this section.

9.4.2.1	NR Inter-frequency SS-Block identification on a newly detected cell for FR1

The UE shall be able to identify the SS-Block of a newly identified inter-frequency cell on a FR1 carrier within [TIdentify_Inter_SSB1] according to the following expression:

TBD [Editor’s note: Requirements for both non-DRX and DRX would be captured here]

9.4.2.2	NR Inter-frequency SS-Block identification on a newly detected cell for FR2

The UE shall be able to identify the SS-Block of a newly identified inter-frequency cell on an FR2 carrier within [TIdentify_Inter_SSB2] according to the following expression:

TBD [Editor’s note: Requirements for both non-DRX and DRX would be captured here]

9.4.3		NR Inter frequency SS-Block identification on a detected cell

The UE shall be able to identify a new SS-Block on an already identified inter-frequency cell according to the requirements in this section.

9.4.3.1	NR Inter-frequency SS-Block identification on a detected cell for FR1

The UE shall be able to identify a new SS-Block of an already identified inter-frequency cell on a FR1 carrier within [TIdentify_Inter_SSB3] according to the following expression:

TBD [Editor’s note: Requirements for both non-DRX and DRX would be captured here]

9.4.3.2	NR Inter-frequency SS-Block identification on a detected cell for FR2

The UE shall be able to identify a new SS-Block of an already identified inter-frequency cell on an FR2 carrier within [TIdentify_Inter_SSB4] according to the following expression:

TBD [Editor’s note: Requirements for both non-DRX and DRX would be captured here]

### 9.4.4	NR Inter frequency measurements

When measurement gaps are provided for inter frequency measurements, or the UE supports capability of conducting such measurements without gaps, the UE physical layer shall be capable of reporting SS-RSRP, SS-RSRQ and SS-SINR measurements to higher layers with measurement accuracy as specified in sub-clauses [TBD], [TBD], and [TBD], respectively, with a measurement period given by:

TBD [Editor’s note: Physical layer measurement period for both non-DRX and DRX]

9.4.4.1	NR Inter-frequency measurements on a detected cell for FR1

Per FR1 inter-frequency layer the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for

- [FFS] identified-inter-frequency cells,

- [FFS] identified SSB with distinct PCID and/or time index

The UE physical layer shall be capable of reporting SS-RSRP, SS-RSRQ and SS-SINR measurements to higher layers with measurement accuracy as specified in sub-clauses [TBD], [TBD], and [TBD], respectively with the measurement period according to TBD.

9.4.4.2	NR Inter-frequency measurements on a detected cell for FR2

Per FR2 inter-frequency layer the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for

- [FFS] identified-inter-frequency cells,

- [FFS] identified SSB with distinct PCID and/or time index

The UE physical layer shall be capable of reporting SS-RSRP, SS-RSRQ and SS-SINR measurements to higher layers with measurement accuracy as specified in sub-clauses [TBD], [TBD], and [TBD], respectively with the measurement period according to TBD.

Editor’s Note: UE requirements on number of cells to measure per inter-frequency carrier, number of SS blocks to measure per inter-frequency carrier, or number of cells and number of SS blocks per inter-frequency carrier needs to be captured here.

### 9.4.5	NR Inter frequency measurements reporting requirements

#### 9.4.5.1	Periodic Reporting

Reported SS-RSRP, SS-RSRQ, and SS-SINR measurements contained in periodically triggered measurement reports shall meet the requirements in sections [TBD], respectively.

#### 9.4.5.2	Event-triggered Periodic Reporting

Reported SS-RSRP, SS-RSRQ, and SS-SINR measurements contained in event triggered periodic measurement reports shall meet the requirements in sections [TBD], respectively.

The first report in event triggered periodic measurement reporting shall meet the requirements specified in clause [TBD].

#### 9.4.5.3	Event-triggered Reporting

Reported SS-RSRP, SS-RSRQ, and SS-SINR measurements contained in event triggered measurement reports shall meet the requirements in sections [TBD], respectively.

The UE shall not send any event triggered measurement reports, as long as no reporting criteria are fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that that the measurement report is not delayed by other RRC signalling on the [DCCH]. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the [TTI] of the uplink [DCCH]. The delay uncertainty is: [2 x TTIDCCH.] This measurement reporting delay excludes a delay which caused by no UL resources for UE to send the measurement report.

The event triggered measurement reporting delay, measured without L3 filtering shall be less than [T identify -inter ] defined in clause [TBD]. When L3 filtering is used an additional delay can be expected.

If a cell which has been detectable at least for the time period [Tidentify_inter ] defined in clause [TBD] and then triggers the measurement report as per TS 38.331 [TBD], the event triggered measurement reporting delay shall be less than [TMeasurement_Period_Inter ]defined in clause [TBD] provided the timing to that cell has not changed more than [ 50 Ts] while measurement gap has not been available and the L3 filter has not been used. When L3 filtering is used an additional delay can be expected.

Editor’s note: To be captured once the RAN2 work progresses.