3GPP TSG-RAN WG4 Meeting #85	R4-1714119

## Reno, Nevada, USA, 27 Nov - 1 Dec, 2017

Source: 	Huawei

Title:	TP to TS 38.104: OTA base station output power, 2-O (9.3.3)

Agenda Item:	9.5.2.1.2 (Radiated transmit power)

Document for:	Approval

# 1 Introduction

In this contribution we provide TP to TS 38.104 on the OTA base station output power (TRP) for BS type O-2. Value of the requirement itself is discussed in a separate contribution.

# 3 Conclusion

As the TRP requirement is needed for the FR2 (as the equivalent of the conducted Tx power requirement in case of FR1), the following proposal is formulated:

Proposal 1: Similar to the TRP OTA base station output power requirement for FR1, the FR2 requirement is specified only for Normal conditions.

The TRP power accuracy value of the requirement itself is discussed in a separate contribution.

Please note that the regional requirements for all the NR BS requirements are discussed in a separate contribution in [3].

The attached TP is proposed to be approved for the TS 38.104:

Proposal 2: approve the attached TP to draft TS 38.104 on the TRP requirement for the radiated transmit power for BS type 2-O.

TRP measurement methodology will be discussed in the Performance part of the NR WI.

# 4 References

Draft TS 38.104, v. 0.4.0

R4-1712622		FR2 output power accuracy

R4-1712641		TP to TS 38.104: consideration of regional requirements

# TP to the draft TS 38.104

In this section, TP to the TS 38.104 [1] is proposed for approval.

- ------------------------------ Modified section ------------------------------

## 9.3	OTA base station output power

### 9.3.1		General

OTA BS output power is declared as the TRP radiated requirement, with the output power accuracy requirement defined at the RIB during the transmitter ON period for BS type 1-O and BS type 2-O.

The NR BS rated carrier TRP output power shall be within limits as specified in table 9.3.1-1.

Table 9.3.1-1: NR BS rated carrier TRP output power limits for BS type 1-O

| BS class | Prated,c,TRP |
| --- | --- |
| Wide Area BS | (note) |
| Medium Range BS | ≤ + 47 dBm |
| Local Area BS | ≤ + 33 dBm |
| NOTE: There is no upper limit for the Prated,c,TRP of the Wide Area Base Station. |  |

Despite the general requirements for the NR BS output power described in subclauses 9.3.2 – 9.3.3, additional regional requirements might be applicable.

NOTE:	In certain regions, power limits corresponding to BS classes may apply for BS type 2-O.

### 9.3.2	Minimum requirement for BS type 1-O

In normal conditions, the BS type 1-O maximum carrier TRP output power, Pmax,c, TRP measured at the RIB shall remain within ±2 dB of the rated carrier TRP output power Prated,c,TRB, as declared by the manufacturer.

Normal conditions are defined in TS 38.141-1, annex B [6].

#### 9.3.2.1		Additional requirements (regional)

In certain regions, additional regional requirements may apply.

### 9.3.3	Minimum requirement for BS type 2-O

In normal conditions, the BS type 2-O maximum carrier TRP output power, Pmax,c, TRP measured at the RIB shall remain within ±3 dB of the rated carrier TRP output power Prated,c,TRB, as declared by the manufacturer.

Normal conditions are defined in TS 38.141-1, annex B [6].

------------------------------ End of modified sections ------------------------------