3GPP TSG RAN WG4 Meeting #86	R4-1801414

Athens, Greece, February 26th – March 2nd, 2018

3GPP TSG RAN WG1 Meeting # AH1801			R1-1801246

Vancouver, Canada, January 22nd – 26th, 2018

Title:	LS on RMSI CORESET configurations

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN WG1

To:	RAN WG4

Contact Person:

#### Name:	Ren Da

E-mail Address:	renda@catt.com

1. Overall Description:

In a previous LS to RAN4 (R1-1721727), RAN1 informed RAN4 about RAN1’s working assumption on RMSI CORESET configurations based on the information of the number of PRBs for each subcarrier spacing for a given system bandwidth provided in R4-1709075, and the UE minimum bandwidth in the context of RMSI and COREST containing PDCCH scheduling RMSI for FR1 and for FR2 given in RAN4’s LS R4-1714392.

RAN1 would like to inform RAN4 that in RAN1#AH1801, based on the information provided in RAN4’s LS R1-1800008 on LS on channel raster and SS raster, the applicability of the RMSI CORESET configurations in R1-1721727 for the bands with the minimum channel bandwidths of 5 MHz and 10 MHz is clarified. In addition, new RMSI CORESET configurations are added for supporting the bands with the minimum channel bands of 40 MHz, as shown in the Appendix A. The methodology used for deriving the configuration is shown in the R1-1801168.

2. Actions:

To RAN WG4:

RAN1 respectfully asks RAN4 to check if there is any issue with the RMSI CORESET configurations provided in this LS.

3. Date of Next RAN1 Meetings:

RAN1#92 	26 Feb - 2 Mar 2018   	Athens 	GR

3GPPRAN1#92-Bis	16 - 20 Apr 2018   	Sanya 	CN

Appendix A: updated and new RMSI CORESET configuration tables in TS 38.213

Table 13-1: Set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space when {SS/PBCH block, PDCCH} subcarrier spacing is {15, 15} kHz for bands with minimum channel bandwidth 5MHz

| Index | SS/PBCH block and control resource set multiplexing pattern | Number of RBs | Number of Symbols | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 24 | 2 | 0 |
| 1 | 1 | 24 | 2 | 2 |
| 2 | 1 | 24 | 2 | 4 |
| 3 | 1 | 24 | 3 | 0 |
| 4 | 1 | 24 | 3 | 2 |
| 5 | 1 | 24 | 3 | 4 |
| 6 | 1 | 48 | 1 | 12 |
| 7 | 1 | 48 | 1 | 16 |
| 8 | 1 | 48 | 2 | 12 |
| 9 | 1 | 48 | 2 | 16 |
| 10 | 1 | 48 | 3 | 12 |
| 11 | 1 | 48 | 3 | 16 |
| 12 | 1 | 96 | 1 | 38 |
| 13 | 1 | 96 | 2 | 38 |
| 14 | 1 | 96 | 3 | 38 |
| 15 | Reserved |  |  |  |

Table 13-2: Set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space when {SS/PBCH block, PDCCH} subcarrier spacing is {15, 30} kHz for bands with minimum channel bandwidth 5MHz

| Index | SS/PBCH block and control resource set multiplexing pattern | Number of RBs | Number of Symbols | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 24 | 2 | 6 |
| 1 | 1 | 24 | 2 | 7 |
| 2 | 1 | 24 | 2 | 8 |
| 3 | 1 | 24 | 3 | 6 |
| 4 | 1 | 24 | 3 | 7 |
| 5 | 1 | 24 | 3 | 8 |
| 6 | 1 | 48 | 1 | 18 |
| 7 | 1 | 48 | 1 | 20 |
| 8 | 1 | 48 | 2 | 18 |
| 9 | 1 | 48 | 2 | 20 |
| 10 | 1 | 48 | 3 | 18 |
| 11 | 1 | 48 | 3 | 20 |
| 12 | Reserved |  |  |  |
| 13 | Reserved |  |  |  |
| 14 | Reserved |  |  |  |
| 15 | Reserved |  |  |  |

Table 13-3: Set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space when {SS/PBCH block, PDCCH} subcarrier spacing is {30, 15} kHz for bands with minimum channel bandwidth 5MHz and 10MHz

| Index | SS/PBCH block and control resource set multiplexing pattern | Number of RBs | Number of Symbols | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 48 | 1 | 2 |
| 1 | 1 | 48 | 1 | 6 |
| 2 | 1 | 48 | 2 | 2 |
| 3 | 1 | 48 | 2 | 6 |
| 4 | 1 | 48 | 3 | 2 |
| 5 | 1 | 48 | 3 | 6 |
| 6 | 1 | 96 | 1 | 28 |
| 7 | 1 | 96 | 2 | 28 |
| 8 | 1 | 96 | 3 | 28 |
| 9 | Reserved |  |  |  |
| 10 | Reserved |  |  |  |
| 11 | Reserved |  |  |  |
| 12 | Reserved |  |  |  |
| 13 | Reserved |  |  |  |
| 14 | Reserved |  |  |  |
| 15 | Reserved |  |  |  |

Table 13-4: Set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space when {SS/PBCH block, PDCCH} subcarrier spacing is {30, 30} kHz for bands with minimum channel bandwidth 5MHz and 10MHz

| Index | SS/PBCH block and control resource set multiplexing pattern | Number of RBs | Number of Symbols | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 24 | 2 | 0 |
| 1 | 1 | 24 | 2 | 1 |
| 2 | 1 | 24 | 2 | 2 |
| 3 | 1 | 24 | 2 | 3 |
| 4 | 1 | 24 | 2 | 4 |
| 5 | 1 | 24 | 3 | 0 |
| 6 | 1 | 24 | 3 | 1 |
| 7 | 1 | 24 | 3 | 2 |
| 8 | 1 | 24 | 3 | 3 |
| 9 | 1 | 24 | 3 | 4 |
| 10 | 1 | 48 | 1 | 12 |
| 11 | 1 | 48 | 1 | 14 |
| 12 | 1 | 48 | 1 | 16 |
| 13 | 1 | 48 | 2 | 12 |
| 14 | 1 | 48 | 2 | 14 |
| 15 | 1 | 48 | 2 | 16 |

Table 13-x1: Set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space when {SS/PBCH block, PDCCH} subcarrier spacing is {30, 15} kHz for bands with minimum channel bandwidth 40MHz

| Index | SS/PBCH block and control resource set multiplexing pattern | Number of RBs | Number of Symbols | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 48 | 1 | 4 |
| 1 | 1 | 48 | 2 | 4 |
| 2 | 1 | 48 | 3 | 4 |
| 3 | 1 | 96 | 1 | 0 |
| 4 | 1 | 96 | 1 | 56 |
| 5 | 1 | 96 | 2 | 0 |
| 6 | 1 | 96 | 2 | 56 |
| 7 | 1 | 96 | 3 | 0 |
| 8 | 1 | 96 | 3 | 56 |
| 9 | Reserved |  |  |  |
| 10 | Reserved |  |  |  |
| 11 | Reserved |  |  |  |
| 12 | Reserved |  |  |  |
| 13 | Reserved |  |  |  |
| 14 | Reserved |  |  |  |
| 15 | Reserved |  |  |  |

Table 13-x2: Set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space when {SS/PBCH block, PDCCH} subcarrier spacing is {30, 30} kHz for bands with minimum channel bandwidth 40MHz

| Index | SS/PBCH block and control resource set multiplexing pattern | Number of RBs | Number of Symbols | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 24 | 2 | 0 |
| 1 | 1 | 24 | 2 | 4 |
| 2 | 1 | 24 | 3 | 0 |
| 3 | 1 | 24 | 3 | 4 |
| 4 | 1 | 48 | 1 | 0 |
| 5 | 1 | 48 | 1 | 28 |
| 6 | 1 | 48 | 2 | 0 |
| 7 | 1 | 48 | 2 | 28 |
| 8 | Reserved |  |  |  |
| 9 | Reserved |  |  |  |
| 10 | Reserved |  |  |  |
| 11 | Reserved |  |  |  |
| 12 | Reserved |  |  |  |
| 13 | Reserved |  |  |  |
| 14 | Reserved |  |  |  |
| 15 | Reserved |  |  |  |