3GPP TSG-RAN WG4 Meeting #86	R4-1801431

Athens, Greece, 26 Feb – 2 March 2018

Agenda item:	7.4.7.8

Source: 	CMCC

Title: 	TP for TR 38.817-01 NR UE power classes

Document for:	Approval

Introduction

This paper is a text proposal to capture the agreements of NR UE power classes for TR 38.817-01 based on [1][2].

Reference

- R4-1801430, “Proposal on NR HPUE definition for PC2”, CMCC

- R4-1801428, “TP for TR 38.813 HPUE of n78 and n77”, CMCC

- Text Proposal for NR TR 38.817-01

- <Start of Text Proposal>

Transmit power

5.2.1		Power Class

The following NR UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth for non-CA configuration unless otherwise stated.

# Table 5.2.1-1: NR UE power classes

| NR Operating band | Class 2 | Class 3 | Comments |
| --- | --- | --- | --- |
| n1 |  | 23 dBm ± 2 dB | LTE refarming band |
| n2 |  | 23 dBm ± 2 dB | LTE refarming band |
| n3 |  | 23 dBm ± 2 dB | LTE refarming band |
| n5 |  | 23 dBm ± 2 dB | LTE refarming band |
| n7 |  | 23 dBm ± 2 dB | LTE refarming band |
| n8 |  | 23 dBm ± 2 dB | LTE refarming band |
| n20 |  | 23 dBm ± 2 dB | LTE refarming band |
| n25 |  | 23 dBm ± 2 dB | LTE refarming band |
| n26 |  | 23 dBm ± 2 dB | LTE refarming band |
| n28 |  | 23 dBm +2/-2.5 dB | LTE refarming band |
| n38 |  | 23 dBm ± 2 dB | LTE refarming band |
| n41 | 26 dBm +2/-3 dB | 23 dBm ± 2 dB | LTE refarming band |
| n50 |  | 23 dBm ± 2 dB | LTE refarming band |
| n51 |  | 23 dBm ± 2 dB | LTE refarming band |
| n66 |  | 23 dBm ± 2 dB | LTE refarming band |
| n70 |  | 23 dBm ± 2 dB | LTE refarming band |
| n71 |  | 23 dBm +2/-2.5 dB | LTE refarming band |
| n74 |  | 23 dBm ± 2 dB | LTE refarming band |
| n77 | 26 dBm +2/-3 dB | 23 dBm +2/-3 dB | 3.3 - 4.2 GHz |
| n78 | 26 dBm +2/-3 dB | 23 dBm +2/-3 dB | 3.3 - 3.8 GHz |
| n79 | 26 dBm +2/-3 dB | 23 dBm +2/-3 dB | 4.4 - 5 GHz |
| n80 |  | TBD | SUL 1710 - 1785 MHz |
| n81 |  | TBD | SUL 880 - 915 MHz |
| n82 |  | TBD | SUL 832 - 862 MHz |
| n83 |  | TBD | SUL 703 - 748 MHz |
| NOTE: If the uplink/downlink configuration is 0 or 6, the requirements for power class 2 are not applicable, and the corresponding requirements for a power class 3 UE shall apply. |  |  |  |

- <End of Text Proposal>