3GPP TSG-WG RAN4 Meeting #86	R4-1801462

# Athens, Greece 26th Feb – 2nd March, 2018

Source: 	Nokia, Nokia Shanghai Bell

Title: 					TP to TR 36.715-04-01: CA_4DL_1A-3A-20A-32A_1UL_BCS0

Agenda Item: 			6.4.2

Document for:	Approval

# Introduction

This TP introduces 4DL/1UL CA configuration CA_1A-3A-20A-32A in to the TR 36.715-04-01.

**************************** Start of changes ********************************************

## 5.X	CA_1A-3A-20A-32A_BCS0

### 5.X.1	Channel bandwidths per operating band for CA

Table 5.X.1-1: Supported E-UTRA bandwidths per CA configuration for 4DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| CA_1A-3A-20A-32A | 1 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  | 20 |  |  |  | Yes | Yes | Yes |  |  |
|  | 32 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.X.2	Co-existence studies

Table 5.X.2-1 summarizes frequency ranges where harmonics occur due to Band 1, 8, 20 and 28 with 1 UL.

Table 5.X.2-1: Impact of 1 UL Harmonic Interference

|  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 1 | 1920 | 1980 | 3840 | 3960 | 5760 | 5940 |
| 3 | 1710 | 1785 | 3420 | 3570 | 5130 | 5355 |
| 20 | 832 | 862 | 1664 | 1724 | 2496 | 2586 |

It can be seen from Table 5.X.2-1 that there is no harmonic relation for this band combination.

### 5.X.3	∆TIB,c and ∆RIB,c values

For 4DL/1UL CA_1A-3A-20A-32A, the TIB,c and RIB,c values are shown in Table 5.X.3-1 and Table 5.X.3-2, respectively.

Table 5.X.3-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1A-3A-20A-32A | 1 | TBD |
|  | 3 | TBD |
|  | 20 | TBD |
|  | 32 | TBD |

Table 5.X.3-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1A-3A-20A-32A | 1 | TBD |
|  | 3 | TBD |
|  | 20 | TBD |
|  | 32 | TBD |

**************************** End of changes ********************************************