3GPP TSG-WG RAN4 Meeting #86	R4-1801463

# Athens, Greece 26th Feb – 2nd March, 2018

Source: 	Nokia, Nokia Shanghai Bell

Title: 					TP to TR 36.715-04-01: CA_4DL_1A-8A-20A-28A_1UL_BCS0

Agenda Item: 			6.4.2

Document for:	Approval

# Introduction

This TP introduces 4DL/1UL CA configuration CA_1A-8A-20A-28A in to the TR 36.715-04-01.

**************************** Start of changes ********************************************

## 5.X	CA_1A-8A-20A-28A_BCS0

### 5.X.1	Channel bandwidths per operating band for CA

Table 5.X.1-1: Supported E-UTRA bandwidths per CA configuration for 4DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| CA_1A-8A-20A-28A | 1 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  | 8 |  |  | Yes | Yes |  |  |  |  |
|  | 20 |  |  |  | Yes | Yes | Yes |  |  |
|  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.X.2	Co-existence studies

Table 5.X.2-1 summarizes frequency ranges where harmonics occur due to Band 1, 8, 20 and 28 with 1 UL.

Table 5.X.2-1: Impact of 1 UL Harmonic Interference

|  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 1 | 1920 | 1980 | 3840 | 3960 | 5760 | 5940 |
| 8 | 880 | 915 | 1760 | 1830 | 2640 | 2745 |
| 20 | 832 | 862 | 1664 | 1724 | 2496 | 2586 |
| 28 | 703 | 748 | 1406 | 1496 | 2109 | 2244 |

It can be seen from Table 5.X.2-1 that the third harmonic of band 28 UL falls into band 1 DL.

### 5.X.3	∆TIB,c and ∆RIB,c values

In the reference architecture of CA_8A-20A-28A, two antennas are assumed to support three bands, with one duplexer for one band and one quadplexer or triplexer for other two bands. A similar architecture can be considered for CA_1A-8A-20A-28A with band 1 diplexed to one of the antenna. If we avoid band 1 and 28 in the same antenna as illustrated in Figure 5.X.3-1, the harmonic issue is possibly avoided.

Figure 5.X.3-1: A possible two anenna architecture for CA_1A-8A-20A-28A

However, to allow different UE architectures such as including the harmonic trap filter in low bands in case band 1 and 28 are supported in the same antenna, we propose the maximum relaxation value among CA_1A-8A-20A, CA_1A-8A-28A, CA_1A-8A-20A, and CA_8A-20A-28A for this band combination. The proposed ΔTIB,c and ΔRIB,c are included in Table 5.X.3-1 and Table 5.X.3-2, respectively.

Table 5.X.3-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1A-8A-20A-28A | 1 | 0.3 |
|  | 8 | 0.6 |
|  | 20 | 0.6 |
|  | 28 | 0.6 |

Table 5.X.4-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1A-8A-20A-28A | 1 | 0 |
|  | 8 | 0.2 |
|  | 20 | 0.2 |
|  | 28 | 0.2 |

### 5.X.4	REFSENS requirement

The REFSENS exceptions due to the third harmonic issue are specified in Table 5.X.4-1 for the uplink configuration in Table 5.X.4-2.

Table 5.X.4-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issues in the combinations of intra-band and inter-band CA)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-8A-20A-28A5,6 | 133 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
|  | 8 |  |  | -96.8 | -93.8 |  |  |  |
|  | 20 |  |  |  | -93.8 | -91 | -89.8 |  |
|  | 28 |  |  | -98.3 | -95.3 | -93.5 | -90.8 |  |
| NOTE 5: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 6: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 33: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |

Table 5.X.4-2: Uplink configuration for the low band (exceptions due to harmonic issue)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-8A-20A-28A | 28 |  |  | 8 | 16 | 25 | 25 | FDD |

**************************** End of changes ********************************************