3GPP TSG-WG RAN4 Meeting #86	R4-1801466

# Athens, Greece 26th Feb – 2nd March, 2018

Source: 	Nokia, Nokia Shanghai Bell, AT&T

Title: 					TP to TR 36.715-05-01: CA_5DL_2A-5B-66A-66A_1UL_BCS0

Agenda Item: 			6.5.2

Document for:	Approval

# Introduction

This TP introduces 5DL/1UL CA configuration CA_2A-5B-66A-66A in to the TR 36.715-05-01.

**************************** Start of changes ********************************************

## 5.X	CA_2A-5B-66A-66A_BCS0

### 5.X.1	Channel bandwidths per operating band for CA

Table 5.X.1-1: Supported E-UTRA bandwidths per CA configuration for 5DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| CA_2A-5B-66A-66A | 2 |  |  | Yes | Yes | Yes | YEs | 80 | 0 |
|  | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 of TS36.101 |  |  |  |  |  |  |  |
|  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 of TS36.101 |  |  |  |  |  |  |  |

### 5.X.2	Co-existence studies

Table 5.X.2-1 summarizes frequency ranges where harmonics occur due to Band 2, 5 and 66 CA with 1 UL.

Table 5.X.2-1: Impact of 1 UL Harmonic Interference

|  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 2 | 1850 | 1910 | 3700 | 3820 | 5550 | 5730 |
| 5 | 824 | 849 | 1648 | 1798 | 2472 | 2547 |
| 66 | 1710 | 1780 | 3420 | 3560 | 5130 | 5340 |

It can be seen from Table 5.X.2-1 that there is no harmonic issue for CA_2A-5B-66A-66A.

### 5.X.3	∆TIB,c and ∆RIB,c values

For 5DL/1UL CA_2A-5B-66A-66A, the TIB,c and RIB,c values are shown in Table 5.X.3-1 and Table 5.X.3-2, respectively, as derived from CA_2A-5A-66A.

Table 5.X.3-1: ΔTIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_2A-5B-66A-66A | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 66 | 0.5 |

Table 5.X.3-2: ΔRIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_2A-5B-66A-66A | 2 | 0.3 |
|  | 5 | 0 |
|  | 66 | 0.3 |

**************************** End of changes ********************************************