3GPP TSG-RAN WG4 Meeting #86                       	 R4-1801499

Athens, Greece, Feb. 26 – March 2, 2018

Agenda Item:	7.9.7.1

Source:	MediaTek Inc.

Title:	Discussion on handover requirement for SA NR

Document for:		Discussion

- Introduction

In the RAN4 AH1801 meeting the way forward on the handover interruption time was agreed [1]. The proposed requirement structure are shown as follow.

- Handover delay and interruption time should be defined - Handover delay Dhandover is defined from the time when the UE receives a RRC message implying handover to the time when UE shall be ready to start the transmission of the new uplink PRACH channel: Dhandover = TRRC_procedure_delay + Tinterruption Where: - TRRC_procedure_delay: it is the RRC procedure delay defined TS 38.331 - Tinterruption: it is the time between end of the last TTI containing the RRC command on the old PDSCH and the time the UE starts transmission of the new PRACH, excluding TRRC_procedure_delay. - Tinterrupt shall be defined in TS38.133 Tinterrupt = Tsearch + TIU + Tmargin + TMIB ms Where: - Tsearch: is the time for PSS/SSS detection, which is 0 for known cell and [TBD] ms for unknown cell provided that the signal quality is sufficient for successful cell detection on the first attempt - TIU: is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to [TBD] ms. - Tmargin: it comprises at least Tprocessing_NR. Whether Tmargin also includes Tloops is FFS where: - Tprocessing_NR: is the UE processing time, which can be up to [TBD]ms - Tloops: is time for time refinement, which is up to [TBD] ms - TMIB: is the time to required to decode MIB, if MIB decode is necessary, otherwise TMIB = 0.

In this paper, we continue to discuss the constitution and requirement for handover interruption time for SA NR.

- NR-NR handover interruption delay

From UE perspective, the underlying operation that needs to be conducted during a handover are shown in Figure 1.

Figure 1. Overall HO procedure from UE perspective

In RAN4, it should discuss what procedures the interruption time is composed of and the requirement for each procedure. The interruption time in NR handover will depend on the SSB periodicity in target cell. The UE needs SSB to conduct search, frequency/time correction loops, SFN acquisition, etc. So each procedure requirement is basically based on SMTC periodicity.

Tsearch

Tsearch is the time required to do the PSS/SSS detection.

- If the target cell is known, Tsearch = 0.

- When the target cell is not already known and signal quality is sufficient for successful cell detection on the first attempt,

  - The PSS/SSS detection time should be 1 SMTC period (denoted by TSSB).

  - In FR1, when target cell is an intra-frequency cell, UE doesn’t need to re-tune AGC. But the additional AGC tuning time should be TSSB when target cell is an inter-frequency cell, because of the SSB-specific beamforming, the target cell’s received signal power could be very dynamic and independent for different SSBs. Therefore, UE cannot use the (n-1)th SSB to adjust AGC gain for the nth SSB in the same SSB burst. UE had to take one more SMTC periodicity for AGC tuning before cell search.

In FR2, considering Rx beam sweeping, AGC tuning will encounter more challenges and need more time.

Observation 1: When the target cell is not already known, the AGC tuning time in FR2 need to be further discussed.

Proposal 1: If the FR1 target cell is not already known and signal quality is sufficient for successful cell detection on the first attempt, Tsearch = 2 x TSSB, where TSSB is the SMTC periodicity of the target cell.

TMIB

In NR-NR handover, when the target cell MIB is not already known, UE should read the target cell’s MIB information for SSB time index (SBI), SFN and half-frame indicator which is different with LTE. UE may not know the target cell MIB in the following scenarios:

- Network did not configured SBI in measurement report

In this scenario, UE is not required to know target cell’s MIB information in its measurement report.

- Network configured SBI in measurement report

  - In FR1, only DMRS detection is needed for SBI acquisition. UE is not required to decode MIB. Therefore, additional MIB decoding is needed when UE executes handover procedure.

  - In FR2, both PBCH-DMRS detection and PBCH NR-MIB decoding are needed for SBI acquisition. So it does not need to decode PBCH NR-MIB again during handover.

- Network enabled useServingCellTimingForSync command

  - Serving cell and target cell SSB time index is alignment, but it haven’t defined serving cell and target cell SFN alignment. For example, if target cell’s SMTC periodicity is 5ms, UE can’t know target cell’s frame boundary only based on this command. So in this scenario, UE still need to decode NR-MIB.

Observation 2: RAN4 should consider to further discuss whether serving cell and target cell SFN is aligned once network enabled useServingCellTimingForSync.

Proposal 2: When the target cell NR-MIB is not already known and signal quality is sufficient for successful NR-MIB reading on the first with one single attempt, TMIB = TSSB.

Tloops

Tloops is the time for frequency/time refinement. In NR, it doesn’t have CRS for fast frequency/time refinement as LTE.

- When the target cell is known and target cell MIB is known,

UE does not need to run target cell’s PSS/SSS detection and PBCH decoding. This will result in target cell frequency/time inaccuracy so that UE should need 1 more SSB to refine the frequency/time before sending PRACH.

- When the target cell MIB is not already known,

UE need to execute target cell’s PBCH decoding. Considering after PSS/SSS detection, UE can only obtain the coarse frequency/time information. It should use 1 more SSB for frequency/time refinement before PBCH decoding to guarantee UE can successfully decode PBCH NR-MIB information. After that, it is believed that the frequency/time accuracy is enough to send PRACH, so UE does not need 1 more SSB to refine the frequency/time after successfully PBCH NR-MIB decoding.

Overall, UE need one more SMTC period to refine the frequency/time in NR.

Proposal 3: Tloops is the time for UE to refine frequency/time in the target cell. Tloops should be at least 1 SMTC period.

TIU

TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. According to table 6.3.3.2-2 and table 6.3.3.2-3 for paired spectrum/supplementary uplink and unpaired spectrum respectively defined in TS 38.211[2], existing PRACH configurations allow PRACH to be transmitted once every 160 ms. Hence the worst case uncertainly due to PRACH occasion can be up to 160 ms for both paired spectrum/supplementary uplink and unpaired spectrum. Thus, TIU can be up to 160ms.

Proposal 4: TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to 160 ms.

NOTE: The actual value of TIU shall depend upon the PRACH configuration used in the target cell.

- Summary

In this paper, we propose the handover interruption requirement for SA NR.

Observation 1: When the target cell is not already known, the AGC tuning time in FR2 need to be further discussed.

Observation 2: RAN4 should consider to further discuss serving cell and target cell SFN is alignment once RRC configured command useServingCellTimingForSync.

Proposal 1: If the FR1 target cell is not already known and signal quality is sufficient for successful cell detection on the first attempt, Tsearch = 2 x TSSB, where TSSB is the SMTC periodicity of the target cell.

Proposal 2: When the target cell NR-MIB is not already known and signal quality is sufficient for successful NR-MIB reading on the first with one single attempt, TMIB = TSSB.

Proposal 3: Tloops is the time for UE to refine frequency/time in the target cell. Tloops should be at least 1 SMTC period.

Proposal 4: TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to 160 ms.

NOTE: The actual value of TIU shall depend upon the PRACH configuration used in the target cell.

- References

- R4-1801057, “WF on the requirement for SS PBCH index acquisition time_v5”, Huawei, HiSilicon, Ericsson, Intel, Qualcomm, Samsung, MediaTek

- R1-1801283, “draftCR to 38.211 capturing the Jan18 ad-hoc meeting agreements”, Ericsson