3GPP TSG-RAN WG4 Meeting #86                                              R4-1801585

Athens , GR, 26th Feb  – 2nd March, 2018

Title:	Reply LS on NR Idle Mode Measurements

Response to:	R2-1801658

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN WG4

To:	RAN WG2

Cc:	RAN WG1

Contact Person:

Name:	Xiaoran ZHANG

E-mail Address:	zhangxiaoran@chinamobile.com

1. Overall Description:

RAN4 would like to thank RAN2 for the LS on NR Idle Mode Measurements (R2-1801658). During RAN4#86 meeting, RAN4 has discussed the questions in RAN2 LS and provide the answers as below.

[Question 1] RAN2 would like to ask RAN1 and RAN4 if there have been any requirements introduced for the derivation of cell quality measurements in NR Idle/Inactive mode (e.g. the minimum number of samples to be considered).

[RAN4 answer 1]: The requirements of cell quality measurements for NR Idle/Inactive mode will be specified in TS 38.133, including minimum number of samples, sample window, cell detection time, measurement interval and cell evaluation time. The exact requirements are still under RAN4 discussion.

[Question 2] RAN2 would like to ask RAN1 and RAN4 what an appropriate minimum value for NR would be to substitute for the  “-110dBm” used in E-UTRAN for PLMN selection criteria.

[RAN4 answer 2]:  “-110dBm” used in E-UTRAN for PLMN selection criteria can be reused for NR.

[Question 3] RAN2 would like to ask RAN4 whether a Pcompensation type parameter(s) is also needed in S-criteria for NR as in LTE and if there are any additional use cases and how it is applicable, e.g. for SUL.

[RAN4 answer 2]:

- Pcompensation type parameter for different UE power classes is needed in S-criteria for NR as in LTE, and the applicable way is same as LTE.

| Pcompensation | If the UE supports the additionalPmax in the NS-PmaxList, if present, in SIB1, SIB3 and SIB5: max(PEMAX1 –PPowerClass, 0) – (min(PEMAX2, PPowerClass) – min(PEMAX1, PPowerClass)) (dB); else: max(PEMAX1 –PPowerClass, 0) (dB) |
| --- | --- |

- Pcompensation type parameter for SUL is needed in S-criteria for NR.

RAN4 discussed two alternatives that can be considered to apply the Pcompensation type parameter for SUL. Other alternatives are not precluded. And how to apply the SUL compensation in S-criteria is up to RAN2 decision.

- Alternative 1:  Add another compensation parameter for SUL in cell selection S-criteria.

The cell selection criterion S in normal coverage is fulfilled when: Srxlev > 0 AND Squal > 0. where: Srxlev_SUL = Qrxlevmeas – (Qrxlevmin + Qrxlevminoffset) – Pcompensation - Qoffsettemp + Pcompensation_SUL Srxlev = Qrxlevmeas – (Qrxlevmin + Qrxlevminoffset) – Pcompensation - Qoffsettemp Squal = Qqualmeas – (Qqualmin + Qqualminoffset) - Qoffsettemp

- Alternative 2:  Use the same cell selection S-criteria as LTE, and network transmits two “Qrxlevmin” values for SUL and non-SUL UEs.

The cell selection criterion S in normal coverage is fulfilled when: Srxlev > 0 AND Squal > 0. where: Srxlev_SUL = Qrxlevmeas – (Qrxlevmin_SUL + Qrxlevminoffset) – Pcompensation - Qoffsettemp Srxlev = Qrxlevmeas – (Qrxlevmin + Qrxlevminoffset) – Pcompensation - Qoffsettemp Squal = Qqualmeas – (Qqualmin + Qqualminoffset) - Qoffsettemp

2. Actions:

To RAN1/ RAN2 WG:

RAN4 kindly asks RAN2 to take the above information into consideration.

3. Date of Next RAN4 Meetings:

TSG-RAN4 Meeting #86bis	April 16 – 20, 2018		Melbourne, Australia

TSG-RAN4 Meeting #87		May 21 – 25, 2018		Busan, Korea