3GPP TSG-RAN WG4 Meeting #86	R4-1802104

Athens, Greece, 26th Feb – 2nd Mar 2018

Source: 	Nokia, Nokia Shanghai Bell

Title: 					UTRA A-MPR

Agenda Item: 			7.4.7.3

Document for:	Approval

# 1				Discussion

In RAN4#85 we presented a study of necessary A-MPR for FR1 UE to be able to meet the UTRAACLR requirement. Need for A-MPR was simulated against MPR presented in Table 1.

Table 1: tentative MPR used in UTRA A-MPR study [1]

| Modulation | MPR (dB) |  |
| --- | --- | --- |
|  | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM PI/2 BPSK | ≤ 0 | ≤ 0 |
| DFT-s-OFDM QPSK | ≤ 1 | ≤ 0 |
| DFT-s-OFDM 16 QAM | ≤ 1.5 | ≤ 0.5 |
| DFT-s-OFDM 64 QAM | ≤ 2 |  |
| DFT-s-OFDM 256 QAM | ≤ 4 |  |
| CP-OFDM QPSK | ≤ 3 | ≤ 0.5 |
| CP-OFDM 16 QAM | ≤ 3 | ≤ 1.5 |
| CP-OFDM 64 QAM | ≤ 3 |  |
| CP-OFDM 256 QAM | ≤ 6 |  |

Studies presented in [1] showed that no A-MPR is necessary to be able to meet the UTRAACLR requirement.

In previous RAN4 meeting sub-6 MPR values were agreed [2] which were in some cases more than assumed in [1] therefor we propose

Proposal: No A-MPR is specified for UE to be able to meet the UTRAACLR requirement.

# 2	References

[1] R4-1712445, NR A-MPR for UTRA, Nokia, Nokia Shanghai Bell, Skyworks Solutions, Inc., RAN4#85

[2] R4-1801203, WF for FR1 MPR, Qualcomm, Intel, LGE, Skyworks, RAN4#1801