3GPP TSG-RAN WG4 Meeting #86	R4-1802113

Athens, Greece, 26th Feb – 2nd Mar 2018

Source: 	Nokia

Title: 					MPR for DC_(n)71B using single PA architecture

Agenda Item: 			7.2.2.1

Document for:	Discussion

# 1	Introduction

This contribution presents A-MPR simulation results for DC_(n)71B contiguous intraband EN-DC configuration.

# 2	Discussion

## 2.1	Simulation assumptions

- PA calibration point: QPSK 100 RB DFT-s-OFDM signal with 0.5 dB MPR

- LO leakage and IQ-Image = 28 dBc

- ALCR from 38.101-3, Table 6.5.3.2.4-1 (below)

- SEM from 38.101-3, Table 6.5.3.2.3.1-1 (according to our proposed correction [1]; below)

- NR general spurious limit from 38.101, Table 6.5.3.1-2

- Equal PSD for LTE and NR

- Same backoff reported for LTE and NR

Table 1: ACLR for intra-band EN-DC (contiguous sub-blocks)

| Parameter | Unit | Value |
| --- | --- | --- |
| EN-DCACLR | dBc | 30 |
| Measurement bandwidth |  | [0.95] ENBW |
| Frequency offset of adjacent channel |  | ENBW / -ENBW |
| NOTE 1: ENBW is the aggregated bandwidth in MHz of an E-UTRA sub-block and an adjacent NR sub-block; there is no frequency separation between the said sub-blocks. The sub-block bandwidths include any internal guard bands. NOTE 2: The frequency offset is that in between the centre frequencies of the measurement filters |  |  |

Table 2: Additional requirements

| ΔfOOB (MHz) | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement [dBm] | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.1 MHz | 0.015 MHz f_offset < 0.085 MHz | -13 | 30 kHz |
| 0.1 MHz f < ENBW | 0.15 MHz f_offset < ENBW – 0.05 MHz | -13 | 100 kHz |
| ENBW f < ENBW + 5 MHz | ENBW+0.5 MHz f_offset < ENBW + 4.5 MHz | -25 | 1 MHz |
| NOTE 1: ENBW is the aggregated bandwidth of an E-UTRA sub-block and an adjacent NR sub-block; there is no frequency separation between the said sub-blocks. The sub-block bandwidths include any internal guard bands. |  |  |  |

## 2.2	Results

Two different scenarios were simulated firstly a “contiguous case” where LTE and NR form a contiguous block or only LTE or NR is allocated and secondly a “non-contiguous” case where LTE and NR allocations start from outer edges of aggregated bandwidth. These two cases represent best and worst case scenarios from A-MPR point of view.

Figure 1: Backoff for LTE10+NR10 QPSK in contiguous case

From Figure 1 we can see that max 2.3 dB of backoff is necessary for contiguous case which is quite moderate considering the agreed MPR presented in Table 3.

Table 3: Maximum power reduction (MPR) for power class 3

| Modulation | MPR (dB) |  |
| --- | --- | --- |
|  | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM PI/2 BPSK | ≤ 0.5 | 0 |
| DFT-s-OFDM QPSK | ≤ 1 | 0 |
| DFT-s-OFDM 16 QAM | ≤ 2 | ≤ 1 |
| DFT-s-OFDM 64 QAM | ≤ 2.5 |  |
| DFT-s-OFDM 256 QAM | 4.5 |  |
| CP-OFDM QPSK | ≤ 3 | ≤ 1.5 |
| CP-OFDM 16 QAM | ≤ 3 | ≤ 2 |
| CP-OFDM 64 QAM | ≤ 3.5 |  |
| CP-OFDM 256 QAM | ≤ 6.5 |  |

From Figure 2 we can see that max 4.8 dB of backoff is necessary for extreme non-contiguous case. However in many cases the backoff is rather moderate when the allocation sizes are larger.

Figure 2: Backoff for LTE10+NR10 QPSK in non-contiguous case

In the appendix one can find contiguous and non-contiguous results for LTE5+NR15, LTE10+NR10 and LTE 15+NR5.

Results for these additional case are quite aligned with the ones presented in Figure 1 and 2.

# 3	Conclusion

In this contribution we have presented backoff simulation results for DC_(n)71B.

# 4	Reference

[1] R4-1802527, Correction to Additional spectrum emissions mask DC_(n)71B,	Nokia, RAN4#86

# Appendix