3GPP TSG-RAN WG4 #86	R4-1802160

Athens, Greece, February 26th-March 2nd, 2018

Agenda item:	 7.9.9.1.1

Source: 	Qualcomm Incorporated

Title: 	Updated Link Level Results for SS-based RLM measurements in NR

Document for:	Discussion

# 1. 	Introduction

In this contribution, we present updated simulation results from [1] related to SS-based RLM measurements for both sub-6 and mmWave bands, according to the agreed simulation assumptions in [2].

# 2. 	Assumptions

SS RLM measurements simulations have been done according to the assumption in [2] and with these particular cases:

- Reference signal: NR-SSS, with Single transmitted SS block per SS burst set (K=1)

- 1 TX antenna with cross-polarization and 2 RX antennas with ideal receiver beamforming (Rx beam is pointing towards the Rx signal and no beam sweep is simulated)

The selected parameters for subcarrier spacing and propagation conditions are listed directly in Table 1 and Table 2 with the results. The SNR value is measured at the baseband (detector) and no implementation margin nor RF impairments have been simulated. We focus on the results related to the SNR values (after bias removal) that correspond to the 2% and 10% BLER of PDCCH channel, according to the analysis and results in [3] and agreed DCI formats in [4].

# 3. Simulations Results

## 3.1	Results for 4 GHz

The reported results refer to NR-SSS SNR measurements. It can be observed that for both subcarrier spacing and variable propagation conditions, a span of max 4 dB can be achieved with NR-SSS measurements and 5 samples.

Figure 1. NR-SSS SNR measurement distribution for 4 GHz band, SCS = 30 kHz and ETU70.

Table 1. NR-SSS SNR measurement accuracy for 4 GHz propagation conditions and various average

| Channel | Averaged number of samples | Distribution of SNR Span [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SCS = 15 kHz |  | SCS = 30 kHz |  |
|  |  | INS | OOS | INS | OOS |
| AWGN | 5 | 1 | 1.9 | 1 | 2 |
|  | 10 | 0.7 | 1.4 | 0.8 | 1.4 |
|  | 20 | 0.5 | 1 | 0.5 | 1 |
| ETU30 | 5 | 3.3 | 4.2 | 2.9 | 3.7 |
|  | 10 | 2.4 | 3.1 | 2.1 | 2.7 |
|  | 20 | 1.8 | 2.4 | 1.5 | 2.1 |
| ETU70 | 5 | 3.2 | 4.1 | 2.8 | 3.6 |
|  | 10 | 2.3 | 2.9 | 2 | 2.6 |
|  | 20 | 1.6 | 2.1 | 1.4 | 1.9 |

Further averaging up to 10 samples reduces the maximum accuracy span to about 3 dB.

## 3.2	Results for 30 GHz

The reported results refer to NR-SSS SNR measurements. Also for this band, for both subcarrier spacing a span of about 5.5 dB can be achieved with NR-SSS measurements and 5 sample in fading conditions. Further averaging up to 10 samples reduces the maximum accuracy to less than 4dB.

Table 2. NR-SSS SNR measurement accuracy for 30 GHz propagation conditions and various average

| Channel | Averaged number of samples | Distribution of delta SNR [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SCS = 120 kHz |  | SCS = 240 kHz |  |
|  |  | INS | OOS | INS | OOS |
| AWGN | 5 | 1 | 2 | 1 | 2 |
|  | 10 | 0.7 | 1.4 | 0.7 | 1.4 |
|  | 20 | 0.5 | 1 | 0.5 | 1 |
| CDL-A | 5 | 4.4 | 5.1 | 4 | 4.8 |
|  | 10 | 3.1 | 3.7 | 2.9 | 3.5 |
|  | 20 | 2.3 | 2.6 | 2.1 | 2.5 |
| CDL-B | 5 | 3.5 | 4.3 | 3 | 3.7 |
|  | 10 | 2.5 | 3 | 2.1 | 2.7 |
|  | 20 | 1.7 | 2.1 | 1.5 | 1.8 |
| CDL-C | 5 | 3.9 | 4.8 | 4 | 4.8 |
|  | 10 | 2.9 | 3.4 | 2.8 | 3.4 |
|  | 20 | 2 | 2.8 | 2 | 2.4 |
| TDL-A | 5 | 4.8 | 5.5 | 4.8 | 5.5 |
|  | 10 | 3.5 | 4.1 | 3.4 | 4.1 |
|  | 20 | 2.4 | 2.9 | 2.4 | 2.8 |
| TDL-B | 5 | 4.9 | 5.5 | 4.9 | 5.4 |
|  | 10 | 3.5 | 3.9 | 3.5 | 4 |
|  | 20 | 2.6 | 2.9 | 2.6 | 2.9 |
| TDL-C | 5 | 4.8 | 5.5 | 4.6 | 5.5 |
|  | 10 | 3.4 | 3.9 | 3.4 | 3.8 |
|  | 20 | 2.4 | 2.9 | 2.3 | 2.9 |

# 4. 	Conclusions

In this paper, we provided link level simulation results for SS-based RLM measurements in NR bands, for a number of parameter combinations according to the simulation assumptions agreed in [2], with focus on SSS-SNR.

Observation: For both bands, a span of 4-5.5 dB can be achieved for SS-based RLM measurements with NR-SSS measurements and 5 samples, in the SNR ranges reflecting the target PDCCH BLER of interest. Further averaging up to 10 samples, reduces the maximum accuracy span to 3-4 dB.

The results should be taken into account when RAN4 discusses the requirements.

# 6. 	References

- R4-1800954, “Link Level Results for SS-based RLM measurements in NR”, Qualcomm

- R4-1709896, “Simulation assumptions on radio link monitoring in NR”, Huawei, HiSilicon

- R4-1802348, “PDCCH Simulation Results for NR RLM”, Qualcomm

- R4-1801068, “Simulation assumption for NR RLM”, Nokia, Nokia Shanghai Bell