3GPP TSG-RAN WG4 Meeting #86	R4-1802211

# Athens, Greece, 26 Feb - 02 Mar, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-01 | CR | xxxx | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for 38.101-1: Uplink configuration for FR1 NR REFSENS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Skyworks Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-02-16 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Adoption of QPSK DFT-S-OFDM waveforms for uplink transmissions in Reference sensitivity power level test cases (section 7.3) places constraints intrinsic to DFT-S-OFDM onto the number of resource blocks of “Table 7.3-3: Uplink configuration for reference sensitivity”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | RB number for UL configuration in Table 7.3-3 and preceding text. Resource block configuration values have been rounded down to the closest allowed value by DFT-S-OFDM waveforms for all frequency bands/SCS/channel bandwidth/Duplex-mode combinations. Previous values that matched a number of resource block compatible with DFT-S-OFDM have been left unchanged. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Reference waveform for uplink transmissions in REFSENS test cases are not defined. Both CP-OFDM and DFT-s-OFDM will need to be tested and later would have undefined RB allocation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | x |  | Test specifications |  |  | 38.521 |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## <Start of Changes>

## 7.3	Reference sensitivity power level

The reference sensitivity power level REFSENS is the minimum mean power applied to each one of the UE antenna ports for all UE categories, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

In later subclauses of Section 7 where the value of REFSENS is used as a reference to set the corresponding requirement:

-	in all bands, the UE shall be verified against those requirements by applying the REFSENS value in Table 7.3-1 with 2 Rx antenna ports tested;

-	for bands where the UE is required to be equipped with 4 Rx antenna ports, the UE shall additionally be verified against those requirements by applying the resulting REFSENS value derived from the requirement in Table 7.3-2 with 4 Rx antenna ports tested.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3-1 and Table 7.3-2

Table 7.3-1: Two antenna port reference sensitivity QPSK PREFSENS

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band |  |  |  |  | SCS kHz |  |  | 5 MHz (dBm) |  |  | 10 MHz (dBm) |  | 15 MHz (dBm) |  |  | 20 MHz (dBm) |  |  | 25 MHz (dBm) |  |  | 30 MHz (dBm) |  | 40 MHz (dBm) |  |  | 50 MHz (dBm) |  |  | 60 MHz (dBm) |  |  | 80 MHz (dBm) |  |  | 100 MHz (dBm) |  |  | Duplex Mode |  |  |
| n1 |  |  |  | 15 |  |  | -100.0 |  |  | -96.8 |  | -95.0 |  |  | -93.8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |  |
|  |  |  |  | 30 |  |  |  |  |  | -97.1 |  | -95.1 |  |  | -94.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  | -97.5 |  | -95.4 |  |  | -94.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n2 | 15 | [-98.0] | [-94.8] | [-93.0] | [-91.8] |  |  |  |  |  |  |  | FDD |
|  | 30 |  | [-95.1] | [-93.1] | [-92.0] |  |  |  |  |  |  |  |  |
|  | 60 |  | [-95.5] | [-93.4] | [-92.2] |  |  |  |  |  |  |  |  |
| n3 |  |  |  | 15 |  |  | [-97.0] |  |  | [-93.8] |  | [-92.0] |  |  | [-90.8] |  |  | [-89.7] |  |  | [-88.9] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |  |
|  |  |  |  | 30 |  |  |  |  |  | [-94.1] |  | [-92.1] |  |  | [-91.0] |  |  | [-89.8] |  |  | [-89.0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  | [-94.5] |  | [-92.4] |  |  | [-91.2] |  |  | [-90.0] |  |  | [-89.1] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n5 | 15 | [-98.0] | [-94.8] | [-93.0] | [-91.8] |  |  |  |  |  |  |  | FDD |
|  | 30 |  | [-95.1] | [-93.1] | [-92.0] |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n7 | 15 | -98.0 | -94.8 | -93.0 | -91.8 |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -95.1 | -93.1 | -92.0 |  |  |  |  |  |  |  |  |
|  | 60 |  | -95.5 | -93.4 | -92.2 |  |  |  |  |  |  |  |  |
| n7 |  |  |  | 15 |  |  | -98.0 |  |  | -94.8 |  | -93.0 |  |  | -91.8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |  |
|  |  |  |  | 30 |  |  |  |  |  | -95.1 |  | -93.1 |  |  | -92.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  | -95.5 |  | -93.4 |  |  | -92.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n8 |  |  |  | 15 |  |  | [-97.0] |  |  | [-93.8] |  | [-92.0] |  |  | [-90.8] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |  |
|  |  |  |  | 30 |  |  |  |  |  | [-94.1] |  | [-92.1] |  |  | [-91.0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n20 |  |  |  | 15 |  |  | [-97.0] |  |  | [-93.8] |  | [-91.0] |  |  | [-89.8] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |  |
|  |  |  |  | 30 |  |  |  |  |  | [-94.1] |  | [-91.1] |  |  | [-90.0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n28 |  |  |  | 15 |  |  | [-98.5] |  |  | [-95.5] |  | [-93.5] |  |  | [-90.8] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |  |
|  |  |  |  | 30 |  |  |  |  |  | [-95.6] |  | [-93.6] |  |  | [-91.0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n38 |  |  |  | 15 |  |  | -100.0 |  |  | -96.8 |  | -95.0 |  |  | -93.8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | TDD |  |
|  |  |  |  | 30 |  |  |  |  |  | -97.1 |  | -95.1 |  |  | -94.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  | -97.5 |  | -95.4 |  |  | -94.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n41 |  |  |  | 15 |  |  |  |  |  | -94.8 |  | -93.0 |  |  | -91.8 |  |  |  |  |  |  |  |  |  | -88.6 |  |  | -87.6 |  |  |  |  |  |  |  |  |  |  |  | TDD |  |
|  |  |  |  | 30 |  |  |  |  |  | -95.1 |  | -93.1 |  |  | -92.0 |  |  |  |  |  |  |  |  |  | -88.7 |  |  | -87.7 |  |  | -86.9 |  |  | -85.6 |  |  | -84.7 |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  | -95.5 |  | -93.4 |  |  | -92.2 |  |  |  |  |  |  |  |  |  | -88.9 |  |  | -87.8 |  |  | -87.1 |  |  | -85.6 |  |  | -84.7 |  |  |  |  |
| n50 |  |  |  | 15 |  |  | -100.0 |  |  | -96.8 |  | -95.0 |  |  | -93.8 |  |  |  |  |  |  |  |  |  | -90.6 |  |  | -89.6 |  |  |  |  |  |  |  |  |  |  |  | TDD |  |
|  |  |  |  | 30 |  |  |  |  |  | -97.1 |  | -95.1 |  |  | -94.0 |  |  |  |  |  |  |  |  |  | -90.7 |  |  | -89.7 |  |  | -88.9 |  |  | -87.6 |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  | -97.5 |  | -95.4 |  |  | -94.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n51 |  |  |  | 15 |  |  | -100.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | TDD |  |
|  |  |  |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n66 |  |  |  | 15 |  |  | -99.5 |  |  | -96.3 |  | -94.5 |  |  | -93.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |  |
|  |  |  |  | 30 |  |  |  |  |  | -96.6 |  | -94.6 |  |  | -93.5 |  |  |  |  |  |  |  |  |  | -90.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  | -97.0 |  | -94.9 |  |  | -93.7 |  |  |  |  |  |  |  |  |  | -90.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n70 |  |  |  | 15 |  |  | -100.0 |  |  | -96.8 |  | -95.0 |  |  | -93.8 |  |  | -92.7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |  |
|  |  |  |  | 30 |  |  |  |  |  | -97.1 |  | -95.1 |  |  | -94.0 |  |  | -92.8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  | -97.5 |  | -95.4 |  |  | -94.2 |  |  | -93.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n71 |  |  |  |  | 15 |  |  | -97.2 |  |  | -94.0 |  | -91.6 |  |  | -86.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |  |  |
|  |  |  |  |  | 30 |  |  |  |  |  | -94.3 |  | -91.9 |  |  | -87.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  |  | - |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n74 | 15 | -99.5 | -96.3 | -94.5 | -93.3 |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -96.6 | -94.6 | -93.5 |  |  |  |  |  |  |  |  |
|  | 60 |  | -97.0 | -94.9 | -93.7 |  |  |  |  |  |  |  |  |
| n771 |  |  |  |  | 15 |  |  |  |  |  | -95.8 |  | -94.0 |  |  | -92.7 |  |  |  |  |  |  |  | -89.6 |  |  | -88.6 |  |  |  |  |  |  |  |  |  |  |  | TDD |  |  |
|  |  |  |  |  | 30 |  |  |  |  |  | -96.1 |  | -94.1 |  |  | -92.9 |  |  |  |  |  |  |  | -89.7 |  |  | -88.7 |  |  | -87.9 |  |  | -86.6 |  |  | -85.6 |  |  |  |  |  |
|  |  |  |  |  | 60 |  |  | - |  |  | -96.5 |  | -94.4 |  |  | -93.1 |  |  |  |  |  |  |  | -89.9 |  |  | -88.8 |  |  | -88.0 |  |  | -86.7 |  |  | -85.7 |  |  |  |  |  |
| n77 (3.8 to 4.2 GHz)1 |  |  |  |  | 15 |  |  |  |  |  | -95.3 |  | -93.5 |  |  | -92.2 |  |  |  |  |  |  |  | -89.1 |  |  | -88.1 |  |  |  |  |  |  |  |  |  |  |  | TDD |  |  |
|  |  |  |  |  | 30 |  |  |  |  |  | -95.6 |  | -93.6 |  |  | -92.4 |  |  |  |  |  |  |  | -89.2 |  |  | -88.2 |  |  | -87.4 |  |  | -86.1 |  |  | -85.1 |  |  |  |  |  |
|  |  |  |  |  | 60 |  |  | - |  |  | -96.0 |  | -93.9 |  |  | -92.6 |  |  |  |  |  |  |  | -89.4 |  |  | -88.3 |  |  | -87.5 |  |  | -86.2 |  |  | -85.2 |  |  |  |  |  |
| n78 |  |  |  |  | 15 |  |  |  |  |  | -95.8 |  | -94.0 |  |  | -92.7 |  |  |  |  |  |  |  | -89.6 |  |  | -88.6 |  |  |  |  |  |  |  |  |  |  |  | TDD |  |  |
|  |  |  |  |  | 30 |  |  |  |  |  | -96.1 |  | -94.1 |  |  | -92.9 |  |  |  |  |  |  |  | -89.7 |  |  | -88.7 |  |  | -87.9 |  |  | -86.6 |  |  | -85.6 |  |  |  |  |  |
|  |  |  |  |  | 60 |  |  |  |  |  | -96.5 |  | -94.4 |  |  | -93.1 |  |  |  |  |  |  |  | -89.9 |  |  | -88.8 |  |  | -88.0 |  |  | -86.7 |  |  | -85.7 |  |  |  |  |  |
| n79 |  |  |  |  | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -89.6 |  |  | -88.6 |  |  |  |  |  |  |  |  |  |  |  | TDD |  |  |
|  |  |  |  |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -89.7 |  |  | -88.7 |  |  | -87.9 |  |  | -86.6 |  |  | -85.6 |  |  |  |  |  |
|  |  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -89.9 |  |  | -88.8 |  |  | -88.0 |  |  | -86.7 |  |  | -85.7 |  |  |  |  |  |
|  |  |  | NOTE 1: Four Rx antenna ports shall be the baseline for this operating band NOTE 2: The transmitter shall be set to PUMAX as defined in subclause 6.2.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

For UE(s) equipped with 4 Rx antenna ports, reference sensitivity for 2Rx antenna ports in Table 7.3-1 shall be modified by the amount given in ΔRIB,4R in Table 7.3-2 for the applicable operating bands.

Table 7.3-2: Four antenna port reference sensitivity allowance ΔRIB,4R

| Operating band | ΔRIB,4R (dB) |
| --- | --- |
| n7, n38, n41 | -2.7 |
| n77, n78, n79 | -2.2 |

The reference receive sensitivity (REFSENS) requirement specified in Table 7.3-1 and Table 7.3-2 shall be met for an uplink transmission using QPSK DFT-s-OFDM waveforms and for uplink transmission bandwidth less than or equal to that specified in Table 7.3-3.

Table 7.3-3: Uplink configuration for reference sensitivity

| Operating band / SCS / Channel bandwidth / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Duplex Mode |
| n1 | 15 | 25 | 501 | 751 | 1001 |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 24 | 361 | 501 |  |  |  |  |  |  |  |  |
|  | 60 |  | 101 | 18 | 24 |  |  |  |  |  |  |  |  |
| n2 | 15 | 25 | 501 | 501 | 501 |  |  |  |  |  |  |  | FDD |
|  | 30 | 101 | 24 | 241 | 241 |  |  |  |  |  |  |  |  |
|  | 60 |  | 101 | 101 | 101 |  |  |  |  |  |  |  |  |
| n3 | 15 | 25 | 501 | 501 | 501 | [501] | [501] |  |  |  |  |  | FDD |
|  | 30 |  | 24 | 241 | 241 | [241] | [241] |  |  |  |  |  |  |
|  | 60 |  | 101 | 101 | 101 | [101] | [101] |  |  |  |  |  |  |
| n5 | 15 | 25 | 251 | [251] | [251] |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 101 | [101] | [101] |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n7 | 15 | 25 | 501 | 751 | 751 |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 24 | 361 | 361 |  |  |  |  |  |  |  |  |
|  | 60 |  | 101 | 18 | 181 |  |  |  |  |  |  |  |  |
| n8 | 15 | 25 | 251 | [251] | [251] |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 101 | [101] | [101] |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n20 | 15 | 25 | 201 | 202 | 202 |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 101 | 102 | 102 |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n28 | 15 | 25 | 251 | 251 | 251 |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 101 | 101 | 101 |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n38 | 15 | 25 | 501 | 751 | 1001 |  |  |  |  |  |  |  | TDD |
|  | 30 |  | 24 | 361 | 501 |  |  |  |  |  |  |  |  |
|  | 60 |  | 101 | 18 | 24 |  |  |  |  |  |  |  |  |
| n41 | 15 |  | 501 | 751 | 1001 |  |  | 216 | 270 |  |  |  | TDD |
|  | 30 |  | 24 | 361 | 501 |  |  | 1001 | 1281 | 162 | 2161 | 2701 |  |
|  | 60 |  | 101 | 18 | 24 |  |  | 501 | 641 | 751 | 1001 | 135 |  |
| n50 | 15 | 25 | 501 | 751 | 1001 |  |  | 216 | 270 |  |  |  | TDD |
|  | 30 |  | 24 | 361 | 501 |  |  | 1001 | 1281 | 162 | 2161 |  |  |
|  | 60 |  | 101 | 18 | 24 |  |  |  |  |  |  |  |  |
| n51 | 15 | 25 |  |  |  |  |  |  |  |  |  |  | TDD |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n66 | 15 | 25 | 501 | 751 | 1001 |  |  | 216 |  |  |  |  | FDD |
|  | 30 |  | 24 | 361 | 501 |  |  | 1001 |  |  |  |  |  |
|  | 60 |  | 101 | 18 | 24 |  |  |  |  |  |  |  |  |
| n70 | 15 | 25 | 501 | 751 | 1001 | 1281 |  |  |  |  |  |  | FDD |
|  | 30 |  | 24 | 361 | 501 | 641 |  |  |  |  |  |  |  |
|  | 60 |  | 101 | 18 | 24 | 301 |  |  |  |  |  |  |  |
| n71 | 15 | 25 | 251 | 201 | 201 |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 121 | 101 | 101 |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n74 | 15 | 25 | 251 | 251 | 251 |  |  |  |  | - |  |  | FDD |
|  | 30 |  | 101 | 101 | 101 |  |  |  |  |  |  |  |  |
|  | 60 | - | 51 | 51 | 51 |  |  |  |  |  |  |  |  |
| n77 | 15 |  | 501 | 751 | 1001 |  |  | 216 | 270 |  |  |  | TDD |
|  | 30 |  | 24 | 361 | 501 |  |  | 1001 | 1281 | 162 | 2161 | 2701 |  |
|  | 60 | - | 101 | 18 | 24 |  |  | 501 | 641 | 751 | 1001 | 135 |  |
| n77 (3.8 to 4.2 GHz) | 15 |  | 501 | 751 | 1001 |  |  | 216 | 270 |  |  |  | TDD |
|  | 30 |  | 24 | 38 | 51 |  |  | 1001 | 1281 | 162 | 2161 | 2701 |  |
|  | 60 |  | 101 | 18 | 24 |  |  | 501 | 641 | 751 | 1001 | 135 |  |
| n78 | 15 |  | 501 | 751 | 1001 |  |  | 216 | 270 |  |  |  | TDD |
|  | 30 |  | 24 | 361 | 501 |  |  | 1001 | 1281 | 162 | 2161 | 2701 |  |
|  | 60 |  | 101 | 18 | 24 |  |  | 501 | 641 | 751 | 1001 | 135 |  |
| n79 | 15 |  |  |  |  |  |  | 216 | 270 |  |  |  | TDD |
|  | 30 |  |  |  |  |  |  | 1001 | 1281 | 162 | 2161 | 2701 |  |
|  | 60 |  |  |  |  |  |  | 501 | 641 | 751 | 1001 | 135 |  |
| NOTE 1: 1 Refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). NOTE 2: 2 refers to Band 20; for 15kHz SCS, in the case of 15MHz channel bandwidth, the UL resource blocks shall be located at RBstart 11 and in the case of 20MHz channel bandwidth, the UL resource blocks shall be located at RBstart 16; for 30kHz SCS, in the case of 15MHz channel bandwidth, the UL resource blocks shall be located at RBstart 6 and in the case of 20MHz channel bandwidth, the UL resource blocks shall be located at RBstart 8; for 60kHz SCS, in the case of 15MHz channel bandwidth, the UL resource blocks shall be located at RBstart 3 and in the case of 20MHz channel bandwidth, the UL resource blocks shall be located at RBstart 4; |  |  |  |  |  |  |  |  |  |  |  |  |  |

## <End of Changes>