3GPP TSG-RAN WG4 Meeting #86			R4-1802367

Athens, Greece, 26 February – 2 March 2018

Title:	[DRAFT] LS response on CRS muting in eFeMTC

Response to:	R1-1706734

Release:	Rel-15

Work Item:	LTE_eMTC4-Core

Source:	RAN WG4

To:	RAN WG1, RAN WG2

Cc:

Contact Person:

#### Name:	Kazuyoshi Uesaka

Tel. Number:

E-mail Address:	kazuyoshi.uesaka (at) ericsson.com

Attachments:	None

1. Overall Description:

RAN WG4 would like to thank RAN WG1 for the LS on CRS muting for Rel-15 LTE-MTC. RAN WG4 have discussed the minimum amount of CRS to be maintained in time and frequency.

For UE demodulation perspective, RAN WG4 has concluded the necessary amount of CRS(s) for X PRB(s) in frequency domain and Y subframes in time domain as follows:

- Frequency domain

  - X=1 PRB for Cat-M1 (1.4MHz channel bandwidth)

  - X=0 PRBs for Cat-M2 (5MHz channel bandwidth)

- Time domain

  - Cool-down period (after the transmission)

    - Y=1 subframe for CRS-based PDSCH transmission at the end of the last narrowband transmission (both CE Mode A and CE Mode B)

    - Y=0 for other cases

  - Warm-up period (before the transmission)

    - Y=1 subframe before the target MPDCCH and PDSCH transmission (both CE Mode A and CE Mode B)

    - Y=0 for other cases

For RRM perspective, RAN WG4 has concluded the necessary amount of CRS(s) as follows:

- Warm-up/Cool-down

  - Warmup period before active time of DRX (e.g. DRX ON, MIB, paging, SIBx etc):

    - 1 DL subframe

  - Cool down period before active time of DRX:

    - 0 DL subframe

- CRS transmission in the center of cell BW

  - Cat-M1

    - 6 central PRBs containing CRS are always transmitted

  - Cat-M2

    - FFS

- Band scanning/initial cell search

  - CRS Light up configuration

    - CRS transmission over Y central PRBs within the cell BW in N subframe(s) once every M subframes

      - Y = full BW

      - N = 1

      - M = FFS

  - Pre-provisioning of EARFCN is CRS transmissions are switched ON over full BW in 1 DL subframe

    - RAN4 found it beneficial for eMTC UE to have pre-provisioning of EARFCN for eMTC cell to facilitate initial cell search

      - beneficial irrespective of CRS muting but benefit is bigger when CRS is muted in the cell

- Network signalling for CRS muting

  - FFS whether eNB should indicate CRS muting and its associated location and timing information in cell-specific broadcast signaling

    - Rel-15 UE can optimize receiver/transmit algorithm depending on whether CRS muting is enabled in the eMTC cell or not

- Measurements under high velocity

  - Intra-frequency RSRP/RSRQ measurement (measurement period and measurement accuracy) and cell identification requirements under non-DRX for gap pattern ID#0 are reused under high velocity (up to 220 Hz Doppler spread).

  - RAN4 shall study the corresponding requirements under DRX for next meeting (RAN4#86).

  - UEs under high-velocity shall identify and measure cells on both serving and non-serving carriers

    - FFS optimization of gap sharing allocation for high velocity UE

- Information about high velocity operation

  - FFS whether it beneficial to inform UEs about high velocity operation in a cell.

2. Actions:

To RAN1 group.

ACTION: 	RAN WG4 kindly asks RAN WG1 to take our conclusion into account.

To RAN2 group.

ACTION: 	RAN WG4 kindly asks RAN WG2 to take our conclusion into account and introduce the necessary signalling.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN WG4 Meeting #86bis 	16th – 20th April 2018	Melbourne, Australia.

TSG-RAN WG4 Meeting #87	21st – 25th May 2018	Busan, South Korea.