3GPP TSG-RAN WG4 Meeting #86	R4-1802369

Athens, Greece, 26 February – 2 March 2018

Agenda Item:	6.21.7

Source:	Ericsson

Title:	sTTI SPDCCH simulation assumption

Document for:	Discussion

# 1	Simulation assumption for SPDCCH demodulation requirements

## 1.1	Test parameters

| Test number | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- |
| Description | CRS-based, Distributed | CRS-based, Localized | DMRS-based, Distributed | DMRS-based, Localized |
| Bandwidth | 10MHz | 10MHz | 10MHz | 10MHz |
| Number of PDCCH symbols | 2 | 2 | 2 | 2 |
| Cyclic prefix | Normal | Normal | Normal | Normal |
| Subframe configuration | Non-MBSFN | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Propagation condition | EVA70 | EVA5 | EVA5 | EVA5 |
| Antenna configuration | 2x2 Low | 2x2 Low | 2x2 Low | 2x2 Low |
| Antenna ports | 0 and 1 | 0 and 1 | 107 | 107 |
| Precoder update granularity | N/A | N/A | 2 SREGs every 1ms | 2 SREGs every 1ms |
| Beamforming precoder | N/A | N/A | Random precoder (TS36.101 B.4.4) | Random precoder (TS36.101 B.4.4) |
| Short TTI length (Note 1) | Slot-based | Slot-based | Slot-based | Slot-based |
| Configured SPDCCH-PRB-sets | {Set 1, Set 3} | {Set 2, Set 4} | {Set 1, Set 3} | {Set 2, Set 4} |
| SPDCCH-PRB-set used for DCI transmission | Set 1 | Set 2 | Set 3 | Set 4 |
| Aggregation level | 4 SCCE | 1 SCCE | 8 SCCE | 2 SCCE |
| DCI format | 7-1A | 7-1A | 7-1F | 7-1F |
| PDSCH TM | TM2 | TM2 | TM9 | TM9 |
| Duplex modes (Note 2) | FDD / TDD | FDD / TDD | FDD / TDD | FDD / TDD |
| Evaluation criteria | 1% Pm-dsg | 1% Pm-dsg | 1% Pm-dsg | 1% Pm-dsg |
| Note 1: SPDCCH demodulation requirements should be specified for both slot-based and subslot-based. Applicability is based on the UE capability. Note 2: TDD is applicable for slot-based SPDCCH demodulation only. |  |  |  |  |

## 1.2	SPDCCH-PRB-set configuration

|  | Set 1 | Set 2 | Set 3 | Set 4 |
| --- | --- | --- | --- | --- |
| Reference symbol | CRS | CRS | DMRS | DMRS |
| Transmission type | Distributed | Localized | Distributed | Localized |
| Number of OFDM symbols | 1 | 2 | 2 | 2 |
| Number of PRBs | 16 | 8 | 16 | 8 |
| Allocated PRBs | {0, 3, 6, 9, 12, 15, 18, 21, 28, 31, 34, 37, 40, 43, 46, 49} | {0, 1, … 7} | {1, 2, 7, 8, 13, 14, 19, 20, 29, 30, 35, 36, 41, 42, 47, 48} | {8, 9, …, 15} |
| Rate matching mode | Mode 1 | Mode 1 | Mode 1 | Mode 1 |
| SPDCCH L1 Reuse Indication | Not configured | Not configured | Not configured | Not configured |
| Total number of SCCEs | 4 (16 SREGs) | 4 (16 SREGs) | 8 (32 SREGs) | 4 (16 SREGs) |

## 1.3	Payload size (without CRC)

| Parameter | Format 7-1A | Format 7-1F | Notes |
| --- | --- | --- | --- |
| UL/DL differentiation flag | 1 | 1 |  |
| Resource block assignment | 8 | 8 | PDSCH allocation type 0 with 10MHz (RBG=6) |
| Modulation and coding scheme | 5 | 5 |  |
| HARQ process number | 4 | 4 |  |
| New data indicator | 1 | 1 |  |
| Redundancy version | 2 | 2 |  |
| TPC command for PUCCH | 2 | 2 |  |
| DMRS position indicator | 0 | 0 | No signalling due to slot-based transmission |
| Downlink assignment index | 0 (FDD) 2 (TDD) | 0 (FDD) 2 (TDD) |  |
| SRS request | 0 | 0 | SRS triggering not configured |
| Used/Unused SPDCCH resource indication | 0 | 0 | L1-based reuse indication not configured |
| SPUCCH resource indication | 2 | 2 |  |
| Antenna port(s), scrambling identity and number of layers | N/A | 3 |  |
| Total (bits) | 25 (FDD) 27 (TDD) | 28 (FDD) 30 (TDD) |  |

# 2	References

R4-1802368, “PDSCH demodulation requirements for sTTI”, Ericsson.