3GPP TSG-RAN WG4 Meeting #86	R4-1802371

Athens, Greece, 26 February – 2 March 2018

Agenda Item:	6.21.7

Source:	Ericsson

Title:	sTTI SPDSCH simulation assumption

Document for:	Information

# 1	Simulation assumption

## 1.1	PDSCH parameters

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
| Bandwidth | 10MHz | 10MHz | 10MHz |
| Number of PDCCH symbols | 2 | 2 | 2 |
| Cyclic prefix | Normal | Normal | Normal |
| Subframe configuration | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Propagation channel | EVA70 | EVA5 | EVA5 |
| Antenna configuration | 2x2 Low | 2x2 Low | 2x2 Low |
| Transmission modes | TM3 dual layer | TM4 single layer | TM9 single layer |
| Antenna ports | 0 and 1 | 0 and 1 | 7 |
| Precoder update granularity | N/A | N/A | 1 PRG every 1ms |
| Beamforming precoder | N/A | N/A | Random precoder (TS36.101 B.4.1) |
| TTI length (Note 1) | Slot-based / Subslot-based | Slot-based / Subslot-based | Slot-based / Subslot-based |
| MCS | 16QAM 1/2 | 64QAM 1/2 | QPSK 1/3 |
| Duplex mode (Note 2) | FDD / TDD | FDD / TDD | FDD / TDD |
| Evaluation criteria | 70% of Max Tput | 70% of Max Tput | 70% of Max Tput |
| Associated SPDCCH transmission | CRS-based, Localized | CRS-based, Localized | CRS-based, Localized |
| Aggregation level | [4 SCCE] | [2 SCCE] | [4 SCCE] |
| Used/Unused SPDCCH resource indication | N/A | ‘10’ | N/A |
| DCI format | 7-1B | 7-1C | 7-1F |
| Configured SPDCCH-PRB-sets | {Set 1, Set 2} | {Set 3, Set 4} | {Set 5, Set 6} |
| SPDCCH-PRB-set used for DCI transmission | Set 1 | Set 3 | Set 5 |
| Note 1: sTTI PDSCH demodulation requirements should be specified for both slot-based and subslot-based. Applicability is based on the UE capability. Note 2: TDD is applicable for slot-based PDSCH demodulation only. |  |  |  |

## 1.2	SPDCCH-PRB-set configuration

|  | Set 1 | Set 2 | Set 3 | Set 4 | Set 5 | Set 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Reference symbol | CRS | CRS | CRS | CRS | CRS | CRS |
| Transmission type | Localized | Localized | Localized | Localized | Localized | Localized |
| Number of OFDM symbols | 1 | 1 | 1 | 1 | 2 | 2 |
| Number of PRBs | 16 | 16 | 16 | 16 | 8 | 8 |
| Allocated PRBs | {0, 1, …, 15} | {34, 35, …, 49} | {0, 1, …, 15} | {34, 35, …, 49} | {0, 1, …, 7} | {8, 9, …, 15} |
| Rate matching mode | Mode 2 | Mode 2 | Mode 3 | Mode 3 | Mode 4 | Mode 4 |
| SPDCCH L1 Reuse Indication | Not configured | Not configured | {2, 0} | {2, 0} | Not configured | Not configured |
| Total number of SCCEs | 4 (16 SREGs) | 4 (16 SREGs) | 4 (16 SREGs) | 4 (16 SREGs) | 4 (16 SREGs) | 4 (16 SREGs) |

## 1.3	FRC tables

### 1.3.1	CRS-based PDSCH

#### 1.3.1.1	Slot-based transmission

|  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- |
| Allocated resource blocks |  | 50 | 50 |
| Modulation |  | 16QAM | 64QAM |
| Target coding rate |  | 1/2 | 1/2 |
| Number of HARQ processes |  | 16 | 16 |
| Maximum number of HARQ transmissions |  | 4 | 4 |
| Information bit payload per slot |  |  |  |
| Slot index 0 | Bits | TBD | TBD |
| Slot index 1 | Bits | TBD | TBD |
| Binary channel bits per slot |  |  |  |
| Slot index 0 | Bits | TBD | TBD |
| Slot index 1 | Bits | TBD | TBD |

#### 1.3.1.2	Subslot-based transmission

|  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- |
| Allocated resource blocks |  | 50 | 50 |
| Modulation |  | 16QAM | 64QAM |
| Target coding rate |  | 1/2 | 1/2 |
| Number of HARQ processes |  | 16 | 16 |
| Maximum number of HARQ transmissions |  | 4 | 4 |
| Information bit payload per subslots |  |  |  |
| Subslot index 0 | Bits | N/A | N/A |
| Subslot index 1 | bits | TBD | TBD |
| Subslot index 2 | Bits | TBD | TBD |
| Subslot index 3 | Bits | TBD | TBD |
| Subslot index 4 | Bits | TBD | TBD |
| Binary channel bits per subslot |  |  |  |
| Subslot index 0 | Bits | N/A | N/A |
| Subslot index 1 | Bits | TBD | TBD |
| Subslot index 2 | Bits | TBD | TBD |
| Subslot index 3 | Bits | TBD | TBD |
| Subslot index 4 | Bits | TBD | TBD |

### 1.3.2	DMRS-based PDSCH

#### 1.3.2.1	Slot-based transmission

|  | Unit | Test 3 |
| --- | --- | --- |
| Allocated resource blocks |  | 50 |
| Modulation |  | 16QAM |
| Target coding rate |  | 1/2 |
| Number of HARQ processes |  | 16 |
| Maximum number of HARQ transmissions |  | 4 |
| Information bit payload per slot |  |  |
| Slot index 0 | Bits | TBD |
| Slot index 1 | Bits | TBD |
| Binary channel bits per slot |  |  |
| Slot index 0 | Bits | TBD |
| Slot index 1 | Bits | TBD |

Number of bits changes whether CSI-RS is transmitted within the same slot.

#### 1.3.2.2	Subslot-based transmission

|  | Unit | Test 1 |
| --- | --- | --- |
| Allocated resource blocks |  | 50 |
| Modulation |  | 16QAM |
| Target coding rate |  | 1/2 |
| Number of HARQ processes |  | 16 |
| Maximum number of HARQ transmissions |  | 4 |
| Information bit payload per subslots |  |  |
| Subslot index 0 | Bits | N/A |
| Subslot index 1 | bits | TBD |
| Subslot index 2 | Bits | TBD |
| Subslot index 3 | Bits | TBD |
| Subslot index 4 | Bits | TBD |
| Binary channel bits per subslot |  |  |
| Subslot index 0 | Bits | N/A |
| Subslot index 1 | Bits | TBD |
| Subslot index 2 | Bits | TBD |
| Subslot index 3 | Bits | TBD |
| Subslot index 4 | Bits | TBD |

Number of bits changes whether CSI-RS is transmitted within the same slot.

# 2	References

R4-1802370, “SPDCCH demodulation requirements for sTTI”, Ericsson.