3GPP TSG-RAN4 Meeting #86	R4-1802462

# Athens, Greece, 26 Feb – 2 Mar 2018

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev |  | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for 38.133: Adding GSM/UMTS inter-RAT to measurement gap requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2018-02-19 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | EN-DC inter-RAT requirements are not complete, which is not aligned with recent agreements and RAN LS (RP-172823). |  |  |  |  |  |  |  |  |
| Summary of change: |  | GSM and UMTS are added to the measurment gap requirements as supported technologies monitored during the gaps. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements are not aligned with RAN guidance. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  | TS 36.521-3 |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### 9.1.2	Measurement gap

If the UE requires measurement gaps to identify and measure intra-frequency cells and/or inter-frequency cells, and the UE does not support independent measurement gap patterns for different frequency ranges as specified in Table 5.1-1 in [41], in order for the requirements in the following subsections to apply the [network] must provide a single per-UE measurement gap pattern for concurrent monitoring of all frequency layers.

If the UE requires measurement gaps to identify and measure intra-frequency cells and/or inter-frequency cells and the UE supports independent measurement gap patterns for different frequency ranges as specified in Table 5.1-1 in [41], in order for the requirements in the following subsections to apply the [network] must provide either  per-FR measurement gap patterns for frequency range where UE requires per-FR measurement gap for concurrent monitoring of all frequency layers of each frequency range independently, or a single per-UE measurement gap pattern for concurrent monitoring of all frequency layers of all frequency ranges.

During the per-UE measurement gaps the UE:

-	shall not transmit any data

-	is not required to tune its receiver on any of the carrier frequencies of the corresponding E-UTRAN PCell, E-UTRAN SCell(s) and serving cells

-	 [is not required to receive any signal other than the signals used for the measurements in any of the serving cell(s) if there is no receiver tuning.]

During the per-FR measurement gaps the UE:

-	shall not transmit any data on serving cells in the corresponding frequency range

-	is not required to tune its receiver on any of the carrier frequencies of the corresponding E-UTRAN PCell, E-UTRAN SCell(s) and serving cells in the corresponding frequency range

-	[is not required to receive any signal other the signals used for the measurements in any of the serving cell(s) in the corresponding frequency range if there is no receiver tuning.]

UEs shall support the measurement gap patterns listed in Table 9.1.2-1 based on the applicability specified in table 9.1.2.-2.

# Table 9.1.2-1: Gap Pattern Configurations supported by the E-UTRA-NR dual connectivity UE

| Gap Pattern Id | Measurement Gap Length (MGL, ms) | Measurement Gap Repetition Period (MGRP, ms) |
| --- | --- | --- |
| 0 | 6 | 40 |
| 1 | 6 | 80 |
| 2 | 3 | 40 |
| 3 | 3 | 80 |
| FFS: [4] | [6] | [20] |
| [5] | 6 | 160 |
| FFS: [6] | [4] | [20] |
| [7] | 4 | 40 |
| [8] | 4 | 80 |
| [9] | [4] | [160] |
| [10] | 3 | 20 |
| [11] | 3 | 160 |
| FFS: [12] | [5.5] | [20] |
| [13] | [5.5] | 40 |
| [14] | [5.5] | 80 |
| [15] | [5.5] | 160 |
| [16] | [3.5] | 20 |
| [17] | [3.5] | 40 |
| [18] | [3.5] | 80 |
| [19] | [3.5] | 160 |
| [20] | [1.5] | 20 |
| [21] | [1.5] | 40 |
| [22] | [1.5] | 80 |
| [23] | [1.5] | [160] |

Table 9.1.2-2: Applicability for Gap Pattern Configurations supported by the E-UTRA-NR dual connectivity UE

| Measurement gap pattern configuration | Serving cell | Measurement Purpose | Applicable Gap Pattern Id |
| --- | --- | --- | --- |
| Per-UE measurement gap | E-UTRA + FR1, or E-UTRA + FR2, or E-UTRA + FR1 + FR2 | non-NR RAT Note1 | [0,1,2,3] |
|  |  | FR1 and/or FR2 | [0-11] |
|  |  | non-NR RAT and FR1 and/or FR2 | [0,1,2,3] |
| Per FR measurement gap | E-UTRA and, FR1 if configured | non-NR RAT | [0,1,2,3] |
|  | FR2 if configured |  | No gap |
|  | E-UTRA and, FR1 if configured | FR1 only | [0-11] |
|  | FR2 if configured |  | No gap |
|  | E-UTRA and, FR1 if configured | FR2 only | No gap |
|  | FR2 if configured |  | [12-23] |
|  | E-UTRA and, FR1 if configured | non-NR RAT and FR1 | [0,1,2,3] |
|  | FR2 if configured |  | No gap |
|  | E-UTRA and, FR1 if configured | FR1 and FR2 | [0-11] |
|  | FR2 if configured |  | [12-23] |
|  | E-UTRA and, FR1 if configured | non-NR RAT and FR2 | [0,1,2,3] |
|  | FR2 if configured |  | [12-23] |
|  | E-UTRA and, FR1 if configured | non-NR RAT and FR1 and FR2 | [0,1,2,3] |
|  | FR2 if configured |  | [12-23] |
| Editor notes: this applicability table might need revise according to any change made on Table 9.1.2-1: Gap Pattern Configurations supported by the UE NOTE 1: Non-NR RAT includes E-UTRA, UTRA and/or GSM. |  |  |  |

For E-UTRA-NR dual connectivity, if UE is not capable of per-FR-gap, total interruption time on SCG during MGL is defined only when MGL(N) = 6ms, 4ms and 3ms.

[Editor’s note]The total interruption time on SCG is FFS until the definition for synchronization and asynchronization is specified.

When UE requires measurement gaps to identify and measure intra-frequency cells or when SMTC configured for intra-frequency measurement are fully overlapping with measurement gaps, and when UE is configured to identify and measure cells on inter-frequency carriers,

- the performance of intra-frequency measurement as specified in section 9.2 is scaled by Kintra = 1 / X * 100,

- the performance of inter-frequency measurement as specified in section 9.3 is scaled by Kinter = 1 / (100 – X) * 100,

where X is a signalled RRC parameter TBD and is defined as in Table 9.1.2-5.

# Table 9.1.2-5: Value of parameter X

| Network signaling ParameterName (to be determined by RAN2) | Value of X (%) |
| --- | --- |
| ‘00’ | [TBD] |
| ‘01’ | [TBD] |
| ‘10’ | [TBD] |
| ‘11’ | [TBD] |