3GPP TSG-RAN WG4 Meeting #86	R4-1802539

Athens, Greece, 26th Feb – 2nd Mar 2018

Source: 	Nokia

Title: 					n41 A-MPR simulations results

Agenda Item: 			7.4.7.3

Document for:	Discussion

# 1		Introduction

In previous RAN4 meeting we have presented initial A-MPR results for band n41 [1][2]. This contribution provides complete set of necessary backoff results for all agreed channel bandwidths and the necessary offset to a position where additional spurious emission requirement does not affect the results. Results are presented as necessary back off on top of agreed MPR [3].

# 2	Discussion

## 2.1	Emission requirements

In addition to general spurious emission requirements following requirement apply.

### 2.1.1	Spectrum emission mask

Band 41 has special emission mask for LTE due to FCC requirements, see below Table 1.

Table 1: Band 41 LTE SEM

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -15 | -18 | -20 | -21 | 30 kHz |
| 1-2.5 | -10 | -10 | -10 | -10 | 1 MHz |
| 2.5-2.8 | -10 | -10 | -10 | -10 | 1 MHz |
| 2.8-5 | -10 | -10 | -10 | -10 | 1 MHz |
| 5-6 | -13 | -13 | -13 | -13 | 1 MHz |
| 6-9 | -25 | -13 | -13 | -13 | 1 MHz |
| 9-10 | -25 | -25 | -13 | -13 | 1 MHz |
| 10-13.5 |  | -25 | -13 | -13 | 1 MHz |
| 13.5-15 |  | -25 | -25 | -13 | 1 MHz |
| 15-18 |  |  | -25 | -13 | 1 MHz |
| 18-20 |  |  | -25 | -25 | 1 MHz |
| 20-25 |  |  |  | -25 | 1 MHz |

What is noticeable in band 41 SEM is that the breakpoint from -13 dBm/1 MHz to -25 dBm / 1 MHz is

- 6 MHz for 5 MHz Channel

- 9 MHz for 10 MHz Channel

- 13.5 MHz for 15 MHz Channel

- 18 MHz for 20 MHz Channel

Apart from 5 MHz channel this break point corresponds to LTE transmission bandwidth and comes from the way FCC has defined the mask requirement.

Band 41 emission mask for NR needs to be defined in similar manner by taking into account the actual transmission bandwidths of the signals when setting the breakpoint form -13 dBm/1 MHz to -25 dBm / 1 MHz. Actual NR transmission bandwidths are presented in Table 2. Also RAN4 needs to consider what is the requirement for first 1 MHz outside than channel bandwidth and one possibility is to use same requirement as for NR general emission mask. Hence the proposed n41 SEM is presented in Table 3.

Table 2: NR transmission bandwidths

| SCS [kHz] | Channel bandwidths [MHz] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 | 25 | 40 | 50 | 60 | 80 | 100 |
| 15 | 4.5 | 9.36 | 14.22 | 19.08 | 23.94 | 38.88 | 48.6 | N.A | N.A | N.A |
| 30 | 3.96 | 8.64 | 13.68 | 18.36 | 23.40 | 38.16 | 47.88 | 58.32 | 78.12 | 98.28 |
| 60 | N.A | 7.92 | 12.96 | 17.28 | 22.32 | 36.72 | 46.8 | 56.88 | 77.04 | 97.20 |

Table 3: n41 SEM

| ΔfOOB MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ± 0 - 1 | -18 | -20 | -21 | -24 | -24 | -24 | -24 | -24 | 30 kHz |
| ± 1 - 5 | -10 |  |  |  |  |  |  |  | 1 MHz |
| ± 5 - X | -13 |  |  |  |  |  |  |  |  |
| ± X - (BWChannel + 5 MHz) | -25 |  |  |  |  |  |  |  |  |
| Note 1: X is defined in Table 2 |  |  |  |  |  |  |  |  |  |

### 2.1.2	Additional spurious emissions

In addition to band 41 specific emission mask UE needs to full fill additional spurious emission requirement as presented in Table 4.

Table 4: Additional spurious emission requirement

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | All channel bandwidths |  |
| 2490.5 ≤ f < 2495 | -13 | 1 MHz |
| 0 < f < 2490.5 | -25 | 1 MHz |

### 2.1.2	ACLR

Standard NR ACLR requirement applies as in Table 5 and 6.

Table 5: NR ACLR measurement bandwidth

| NR channel bandwidth / NR ACLR measurement bandwidth |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| NR ACLR measurement bandwidth | 4.515 | 9.375 | 14.235 | 19.095 | 23.955 | 38.895 | 48.615 | 58.35 | 78.15 | 98.31 |

Table 6: NR ACLR requirement

|  | Power class 1 | Power class 2 | Power class 3 |
| --- | --- | --- | --- |
| NRACLR |  | 31 dB | 30 dB |

UTRAACLR does not apply for band 41.

## 2.2. Simulation assumptions

The channel was placed at the lower edge of band n41. This maximizes the required A-MPR due to the additional spurious limit.

Simulation assumptions were as follows.

- PA calibration point: QPSK 100 RB DFT-s-OFDM signal with 0.5 dB MPR

- LO leakage and IQ-Image = 28 dBc

- NR ALCR = 30 dBc

- n41 SEM according to Table 3

- n41 additional spurious emission limit according to Table 4

- MPR as in Table 6 [3]

Table 6: MPR assumption

|  |  | Tentative MPR [dB] |  |
| --- | --- | --- | --- |
| WF type | Modulation | Outer allocation | Inner allocation |
| DFT-S-OFDM | pi/2-BPSK | 0.5 | 0 |
|  | QPSK | 1 | 0 |
|  | 16-QAM | 2 | 1 |
|  | 64-QAM | 2.5 |  |
|  | 256-QAM | 4.5 |  |
| CP-OFDM | QPSK | 3 | 1.5 |
|  | 16-QAM | 3 | 2 |
|  | 64-QAM | 3.5 |  |
|  | 256-QAM | 6.5 |  |

## 2.3	Simulation results

### 2.3.1	Carrier in a lowest position of the band

In Figure 1 we present two plots for necessary total back-off for 100 MHz channel and Figure 2 similarly for 20 MHz channel. Points not exceeding the agreed MPR are shown as zero. Carrier placement is the lowest position in a band which represents the worst case situation due to additional spurious emission requirement.

Figure 1: A-MPR 100 MHz channel, allocations exceeding the agreed MPR

From Figure 1 we can notice three distinguish areas that require A-MPR however if we study 16-QAM only two A-MPR areas exist and for 64-QAM no A-MPR seems to be necessary, see the attached zip-file for more results. The low left area which always starts from RB_start 0 is caused by IMD spikes hitting the additional spurious emission region. Other two regions are due to spectral re-growth hitting the additional spurious emission region.

From 2) we can notice that the amount of A-MPR regions needed for 20 MHz signal is two and from additional results in zip file amount of A-MPR regions is varying from two to zero depending on modulation and SCS. Similarly, as for 100 MHz signal the lower left region is caused by IMD and the other one by re-growth.

Figure 2: A-MPR 100 MHz channel, allocations exceeding the agreed MPR

### 2.3.2	Necessary frequency offset to no A-MPR region.

As was already discussed in [2] our simulations seem to indicate that no A-MPR is necessary once the signal is pulled away from low part of the band.

In Figure 3 we present the necessary offset between low edge of the bands and low edge of the channel when A-MPR is not anymore necessary. Figure 4 presents offsets for 20 MHz channel. Rest of the results are in Zip-file.

Figure 3: Offset for 100 MHz channel

Figure 4: Offset for 20 MHz channel

From Figure 3 we can observe that maximum offset for 100 MHz QPSK is roughly 90 MHz and this is caused by the IMD region where as the re-growth region needs only 20 MHz offset.

From Figure 4 we can see that the 20 MHz QPSK signal IMD region needs 13 MHz offset and re-growth region needs 5 MHz offset.

## 2.4	How to formulate A-MPR

In LTE the A-MPR is presented in table format. Due to large amount of NR channel bandwidths, modulations and SCS we do not think that table format is viable for NR therefore we propose to formulate the NR A-MPR as follows

WORK IN PROGRESS

# 3	Conclusion

In this contribution we have provided full set of A-MPR results for n41. We have also presented the necessary offset from low edge of the band when no A-MPR is necessary due to additional spurious emission requirement.

# 4	References

[1] R4-1712448, Band n41 A-MPR, Nokia, Nokia Shanghai Bell, Skyworks Solutions, Inc., RAN4#85

[2] R4-1800062, n41 A-MPR simulations results, Nokia, Nokia Shanghai Bell, RAN4# AH1801

[3] R4-1801203, WF for FR1 MPR, Qualcomm, Intel, LGE, Skyworks, RAN4#AH 1801