3GPP TSG-RAN4 #86 	R4-1802817

# Athens, GR, February 26th , March 2nd 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 4958 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of PLTE term in the Pcmax,c equation in support of Rel-15 EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2018-02-25 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Introduction of PLTE term in the Pcmax equation to have EN-DC power control on the LTE RAT working properly |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change 1: Added PLTE symbol definition in sub-clause 3.2 and then EN-DC and NR abbreviations definitions in sub-clause 3.3 Change 2: Introduced the PLTE term in the Pcmax,c equation in sub-clause 6.2.5, related to the EN-DC Rel-15 functionality. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-15 EN-DC is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 3.3 6.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR … CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### ---------------------------------------Start of Change 1-------------------------------

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

BWChannel	Channel bandwidth

BWChannel,block 	Sub-block bandwidth, expressed in MHz. BWChannel,block= Fedge,block,high- Fedge,block,low.

BWChannel_CA 	Aggregated channel bandwidth, expressed in MHz.

BWGB	Virtual guard band to facilitate transmitter (receiver) filtering above / below edge CCs.

Transmitted energy per RE for reference symbols during the useful part of the symbol, i.e. excluding the cyclic prefix, (average power normalized to the subcarrier spacing) at the eNode B transmit antenna connector

The averaged received energy per RE of the wanted signal during the useful part of the symbol, i.e. excluding the cyclic prefix, at the UE antenna connector; average power is computed within a set of REs used for the transmission of physical channels (including user specific RSs when present), divided by the number of REs within the set, and normalized to the subcarrier spacing

F					Frequency

Fagg_alloc_low	Aggregated Transmission Bandwidth Configuration. The lowest frequency of the simultaneously transmitted resource blocks.

Fagg_alloc_high	Aggregated Transmission Bandwidth Configuration. The highest frequency of the simultaneously transmitted resource blocks.

FInterferer (offset)	Frequency offset of the interferer (between the center frequency of the interferer and the carrier frequency of the carrier measured)

FInterferer			Frequency of the interferer

FIoffset				Frequency offset of the interferer (between the center frequency of the interferer and the closest edge of the carrier measured)

FC	Frequency of the carrier centre frequency

FC_agg	Aggregated Transmission Bandwidth Configuration.  Center frequency of the aggregated carriers.

FC,block, high		Center frequency of the highest transmitted/received carrier in a sub-block.

FC,block, low		Center frequency of the lowest transmitted/received carrier in a sub-block.

FC_low 	The centre frequency of the lowest carrier, expressed in MHz.

FC_high 	The centre frequency of the highest carrier, expressed in MHz.

FDL_low	The lowest frequency of the downlink operating band

FDL_high	The highest frequency of the downlink operating band

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

Fedge,block,low 	The lower sub-block edge, where Fedge,block,low = FC,block,low - Foffset.

Fedge,block,high 	The upper sub-block edge, where Fedge,block,high = FC,block,high + Foffset.

Fedge_low 	The lower edge of aggregated channel bandwidth, expressed in MHz.

Fedge_high 	The higher edge of aggregated channel bandwidth, expressed in MHz.

Foffset 	Frequency offset from FC_high to the higher edge or FC_low to the lower edge.

Foffset,block,low	Separation between lower edge of a sub-block and the center of the lowest component carrier within the sub-block

Foffset,block,high	Separation between higher edge of a sub-block and the center of the highest component carrier within the sub-block

Foffset_NS_23	Frequency offset in MHz needed if NS_23 is used

FOOB	The boundary between the E-UTRA out of band emission and spurious emission domains.

The power spectral density of the total input signal (power averaged over the useful part of the symbols within the transmission bandwidth configuration, divided by the total number of RE for this configuration and normalised to the subcarrier spacing) at the UE antenna connector, including the own-cell downlink signal

The total transmitted power spectral density of the own-cell downlink signal (power averaged over the useful part of the symbols within the transmission bandwidth configuration, divided by the total number of RE for this configuration and normalised to the subcarrier spacing) at the eNode B transmit antenna connector

The total received power spectral density of the own-cell downlink signal (power averaged over the useful part of the symbols within the transmission bandwidth configuration, divided by the total number of RE for this configuration and normalised to the subcarrier spacing) at the UE antenna connector

The received power spectral density of the total noise and interference for a certain RE (average power obtained within the RE and normalized to the subcarrier spacing) as measured at the UE antenna connector

LCRB	Transmission bandwidth which represents the length of a contiguous resource block allocation expressed in units of resources blocks

LCtone	Transmission bandwidth which represents the length of a contiguous sub-carrier allocation expressed in units of tones

Ncp	Cyclic prefix length

NDL 	Downlink EARFCN

The power spectral density of a white noise source (average power per RE normalised to the subcarrier spacing), simulating interference from cells that are not defined in a test procedure, as measured at the UE antenna connector

The power spectral density of a white noise source (average power per RE normalized to the subcarrier spacing), simulating interference in non-CRS symbols in ABS subframe from cells that are not defined in a test procedure, as measured at the UE antenna connector.

The power spectral density of a white noise source (average power per RE normalized to the subcarrier spacing), simulating interference in CRS symbols in ABS subframe from all cells that are not defined in a test procedure, as measured at the UE antenna connector.

The power spectral density of a white noise source (average power per RE normalised to the subcarrier spacing), simulating interference in non-ABS subframe from cells that are not defined in a test procedure, as measured at the UE antenna connector

The power spectral density (average power per RE normalised to the subcarrier spacing) of the summation of the received power spectral densities of the strongest interfering cells explicitly defined in a test procedure plus , as measured at the UE antenna connector. The respective power spectral density of each interfering cell relative to  is defined by its associated DIP value, or the respective power spectral density of each interfering cell relative to  is defined by its associated Es/Noc value.

NOffs-DL 	Offset used for calculating downlink EARFCN

NOffs-UL 	Offset used for calculating uplink EARFCN

The power spectral density of a white noise source (average power per RE normalised to the subcarrier spacing) simulating eNode B transmitter impairments as measured at the eNode B transmit antenna connector

NRB	Transmission bandwidth configuration, expressed in units of resource blocks

NRB_agg	The number of the aggregated RBs within the fully allocated Aggregated Channel bandwidth.

NRB_alloc	Total number of simultaneously transmitted resource blocks in Channel bandwidth or Aggregated Channel Bandwidth.

NRB,c	The transmission bandwidth configuration of component carrier c, expressed in units of resource blocks

NRB,largest BW	The largest transmission bandwidth configuration of the component carriers in the bandwidth combination, expressed in units of resource blocks

NRX	Number of receiver antennas

Ntone	Transmission bandwidth configuration for category NB1 and NB2, expressed in units of tones.

Ntone 3.75kHz	Transmission bandwidth configuration for category NB1 and NB2 with 3.75 kHz sub-carrier spacing, expressed in units of tones.

Ntone 15kHz 	Transmission bandwidth configuration for category NB1 and NB2 with 15 kHz sub-carrier spacing, expressed in units of tones.

NUL	Uplink EARFCN.

Rav		Minimum average throughput per RB.

PCMAX	The configured maximum UE output power.

PCMAX, c	The configured maximum UE output power for serving cell c.

PEMAX 	Maximum allowed UE output power signalled by higher layers. Same as IE P-Max, defined in [7].

PEMAX, c	Maximum allowed UE output power signalled by higher layers for serving cell c. Same as IE P-Max, defined in [7].

PInterferer	Modulated mean power of the interferer

PLTE 	Maximum allowed UE output power for E-UTRA signalled by higher layers when the UE is configured with EN-DC, defined in [7].

PPowerClass	PPowerClass is the nominal UE power (i.e., no tolerance).

PPowerClass_Default	PPowerClass_Default is the default nominal UE power (i.e., no tolerance) for the band.

PUMAX	The measured configured maximum UE output power.

Puw	Power of an unwanted DL signal

Pw	Power of a wanted DL signal

RBstart 	Indicates the lowest RB index of transmitted resource blocks.

RBend	Indicates the highest RB index of transmitted resource blocks.

Tno_hopping	Transmission period within a TTI duration when consecutive symbols are transmitted without applying any frequency hopping

ΔfOOB	Δ Frequency of Out Of Band emission.

ΔPPowerClass 	Adjustment to maximum output power for a given power class.

ΔRIB,c	Allowed reference sensitivity relaxation due to support for inter-band CA operation, for serving cell c.

ΔRIB,4R	Reference sensitivity adjustment due to support for 4 antenna ports.

ΔTIB,c	Allowed maximum configured output power relaxation due to support for inter-band CA operation, for serving cell c.

TC	Allowed operating band edge transmission power relaxation.

TC,c 	Allowed operating band edge transmission power relaxation for serving cell c.

TProSe	Allowed operating band transmission power relaxation due to support of E-UTRA ProSe on an operating band.

According to Clause 5.2 in TS 36.213 [6]

According to Clause 5.2 in TS 36.213 [6]

Test specific auxiliary variable used for the purpose of downlink power allocation, defined in Annex C.3.2.

Wgap	Sub-block gap size

Wgap_L	Sub-block gap size between lowest two CCs in frequency domain on CA_X-X-X

Wgap_H	Sub-block gap size between highest two CCs in frequency domain on CA_X-X-X

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

ABS	Almost Blank Subframe

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

A-MPR	Additional Maximum Power Reduction

AWGN	Additive White Gaussian Noise

BS	Base Station

CA	Carrier Aggregation

CA_X	Intra-band contiguous CA of component carriers in one sub-block within Band X where X is the applicable E-UTRA operating band

CA_X-X	Intra-band non-contiguous CA of component carriers in two sub-blocks within Band X where X is the applicable E-UTRA operating band

CA_X-X-X	Intra-band non-contiguous CA of component carriers in three sub-blocks within Band X where X is the applicable E-UTRA operating band

CA_X-Y	Inter-band CA of component carrier(s) in one sub-block within Band X and component carrier(s) in one sub-block within Band Y where X and Y are the applicable E-UTRA operating band

CA_X-X-Y	CA of component carriers in two sub-blocks within Band X and component carrier(s) in one sub-block within Band Y where X and Y are the applicable E-UTRA operating bands

CC	Component Carriers

CG	Carrier Group

CPE	Customer Premise Equipment

CPE_X	Customer Premise Equipment for E-UTRA operating band X

CW	Continuous Wave

DC	Dual Connectivity

DC_X-Y	Inter-band DC of component carrier(s) in one sub-block within Band X and component carrier(s) in one sub-block within Band Y where X and Y are the applicable E-UTRA operating band

DL	Downlink

DIP	Dominant Interferer Proportion

EARFCN 	E-UTRA Absolute Radio Frequency Channel Number

EIRP	Effective Isotropic Radiated Power

EPRE	Energy Per Resource Element

E-UTRA 	Evolved UMTS Terrestrial Radio Access

EUTRAN	Evolved UMTS Terrestrial Radio Access Network

EN-DC	E-UTRA-NR Dual Connectivity

EVM	Error Vector Magnitude

FDD	Frequency Division Duplex

FRC	Fixed Reference Channel

GNSS	Global Navigation Satellite Systems

HD	Half-Duplex for Sidelink Operation

HD-FDD	Half- Duplex FDD

ITS	Intelligent Transportation Systems

MCS	Modulation and Coding Scheme

MCG	Main Carrier Group

MOP	Maximum Output Power

MPR	Maximum Power Reduction

MSD	Maximum Sensitivity Degradation

NR	New Radio

OCNG	OFDMA Channel Noise Generator

OFDMA	Orthogonal Frequency Division Multiple Access

OOB	Out-of-band

PA	Power Amplifier

PCC	Primary Component Carrier

P-MPR	Power Management Maximum Power Reduction

ProSe	Proximity-based Services

PSBCH	Physical Sidelink Broadcast CHannel

PSCCH	Physical Sidelink Control CHannel

PSDCH	Physical Sidelink Discovery CHannel

PSS	Primary Synchronization Signal

PSS_RA	PSS-to-RS EPRE ratio for the channel PSS

SSSS	Secondary Sidelink Synchronization Signal

PSSCH	Physical Sidelink Shared CHannel

PSSS	Primary Sidelink Synchronization Signal

RE	Resource Element

REFSENS	Reference Sensitivity power level

r.m.s	Root Mean Square

SCC	Secondary Component Carrier

SCG	Secondary Carrier Group

SINR	Signal-to-Interference-and-Noise Ratio

SNR	Signal-to-Noise Ratio

SSS	Secondary Synchronization Signal

SSS_RA	SSS-to-RS EPRE ratio for the channel SSSSSSS	Secondary Sidelink Synchronization Signal

TDD	Time Division Duplex

UE	User Equipment

UL	Uplink

UL-MIMO	Up Link Multiple Antenna transmission

UMTS	Universal Mobile Telecommunications System

UTRA	UMTS Terrestrial Radio Access

UTRAN	UMTS Terrestrial Radio Access Network

V2X	Vehicle to Everything

xCH_RA	xCH-to-RS EPRE ratio for the channel xCH in all transmitted OFDM symbols not containing cell-specific RS

xCH_RB	xCH-to-RS EPRE ratio for the channel xCH in all transmitted OFDM symbols containing cell-specific RS

### ----------------------------------------End of Change 1-------------------------------

### ---------------------------------------Start of Change 2-------------------------------

### 6.2.5	Configured transmitted power

The UE is allowed to set its configured maximum output power PCMAX,c for serving cell c. The configured maximum output power PCMAX,c is set within the following bounds:

PCMAX_L,c ≤  PCMAX,c  ≤  PCMAX_H,c with

PCMAX_L,c = MIN {MIN(PEMAX,c , PLTE) – TC,c,  (PPowerClass – ΔPPowerClass) – MAX(MPRc + A-MPRc + ΔTIB,c + TC,c + TProSe, P-MPRc)}

PCMAX_H,c = MIN {PEMAX,c,  PLTE, PPowerClass – ΔPPowerClass}

where

-	PEMAX,c is the value given by IE P-Max for serving cell c, defined in [7];

-	PPowerClass is the maximum UE power specified in Table 6.2.2-1 without taking into account the tolerance specified in the Table 6.2.2-1;

-	ΔPPowerClass = 3 dB for a power class 2 capable UE operating in Band 41, when P-max of 23 dBm or lower is indicated or if the uplink/downlink configuration is 0 or 6 in the cell; otherwise, ΔPPowerClass = 0 dB

- 	PLTE is the maximum allowed power for E-UTRA when UE is configured with EN-DC, defined in [7]. If not signalled, then PLTE = PPowerClas in the above equations;

-	MPRc and A-MPRc for serving cell c are specified in subclause 6.2.3 and subclause 6.2.4, respectively;

-	TIB,c is the additional tolerance for serving cell c as specified in Table 6.2.5-2; TIB,c = 0 dB otherwise;

-	TC,c = 1.5 dB when NOTE 2 in Table 6.2.2-1 applies;

-	TC,c = 0 dB when NOTE 2 in Table 6.2.2-1 does not apply;

-	TProSe = 0.1 dB when the UE supports ProSe Direct Discovery and/or ProSe Direct Communication on the corresponding E-UTRA ProSe band; TProSe = 0 dB otherwise.

-	For a power class higher than default UE power class capable UE, ΔPPowerClass = PPowerClass – PPowerClass_Default dB, when the band is a TDD band whose frame configuration is 0 or 6; or P-max is not indicated in the cell; or P-Max is provided and set to the maximum output power of the default power class or lower, otherwise, ΔPPowerClass = 0 dB.

P-MPRc is the allowed maximum output power reduction for

a)	ensuring compliance with applicable electromagnetic energy absorption requirements and addressing unwanted emissions / self desense requirements in case of simultaneous transmissions on multiple RAT(s) for scenarios not in scope of 3GPP RAN specifications;

b)	ensuring compliance with applicable electromagnetic energy absorption requirements in case of proximity detection is used to address such requirements that require a lower maximum output power.

The UE shall apply P-MPR c for serving cell c only for the above cases. For UE conducted conformance testing P-MPR shall be 0 dB

NOTE 1:	P-MPRc was introduced in the PCMAX,c equation such that the UE can report to the eNB the available maximum output transmit power. This information can be used by the eNB for scheduling decisions.

NOTE 2: P-MPRc may impact the maximum uplink performance for the selected UL transmission path.

TREF and Teval are specified in Table 6.2.5-0 for different TTI patterns. For each TREF, the PCMAX_L,c for serving cell c is evaluated perTeval and given by the minimum value taken over the transmission(s) within the Teval; the minimum PCMAX_ L,c over the one or more Teval is then applied for the entire TREF. PPowerClass shall not be exceeded by the UE during any period of time.

Table 6.2.5-0: PCMAX evaluation window for different TTI patterns

| TTI pattern | TREF | Teval | Teval with frequency hopping |
| --- | --- | --- | --- |
| Subframe | 1 subframe | 1 slot | 1 slot |
| Subslot | 2 OS | 2 OS | Min(Tno_hopping, 2OS) |
| Slot | 7 OS | 7 OS | Min(Tno_hopping, 7OS) |

The measured configured maximum output power PUMAX,c shall be within the following bounds:

PCMAX_L,c  –  MAX{TL,c, T(PCMAX_L,c)}  ≤  PUMAX,c  ≤  PCMAX_H,c  +  T(PCMAX_H,c).

where the tolerance T(PCMAX,c) for applicable values of PCMAX,c is specified in Table 6.2.5-1, and Table 6.2.5-1A. The tolerance TL,c is the absolute value of the lower tolerance for the applicable operating band as specified in Table 6.2.2-1.

Table 6.2.5-1: PCMAX tolerance

| PCMAX,c (dBm) | Tolerance T(PCMAX,c) (dB) |
| --- | --- |
| 23 < PCMAX,c ≤ 33 | 2.0 |
| 21 ≤ PCMAX,c ≤ 23 | 2.0 |
| 20 ≤ PCMAX,c < 21 | 2.5 |
| 19 ≤ PCMAX,c < 20 | 3.5 |
| 18 ≤ PCMAX,c < 19 | 4.0 |
| 13 ≤ PCMAX,c < 18 | 5.0 |
| 8 ≤ PCMAX,c < 13 | 6.0 |
| -40 ≤ PCMAX,c < 8 | 7.0 |

Table 6.2.5-1A: PCMAX tolerance for power class 5

| PCMAX,c (dBm) | Tolerance T(PCMAX,c) (dB) |
| --- | --- |
| PCMAX,c = 20 | 2.0 |
| 19 ≤ PCMAX,c < 20 | 3.5 |
| 18 ≤ PCMAX,c < 19 | 4.0 |
| 13 ≤ PCMAX,c < 18 | 5.0 |
| 8 ≤ PCMAX,c < 13 | 6.0 |
| -40 ≤ PCMAX,c < 8 | 7.0 |

For the UE which supports inter-band carrier aggregation configurations with the uplink assigned to one or two E-UTRA bands the ΔTIB,c is defined for applicable bands in Table 6.2.5-2, Table 6.2.5-3 and Table 6.2.5-4.

Table 6.2.5-2: ΔTIB,c (two bands)

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1A-3A | 1 | 0.3 |
|  | 3 | 0.3 |
| CA_1A-1A-3A | 1 | 0.3 |
|  | 3 | 0.3 |
| CA_1A-1A-7A | 1 | 0.5 |
|  | 7 | 0.6 |
| CA_1A-3A-3A | 1 | 0.3 |
|  | 3 | 0.3 |
| CA_1A-3C | 1 | 0.3 |
|  | 3 | 0.3 |
| CA_1A-1A-3C | 1 | 0.3 |
|  | 3 | 0.3 |
| CA_1C-3A | 1 | 0.3 |
|  | 3 | 0.3 |
| CA_1A-5A | 1 | 0.3 |
|  | 5 | 0.3 |
| CA_1A-1A-5A | 1 | 0.3 |
|  | 5 | 0.3 |
| CA_1A-7A | 1 | 0.5 |
|  | 7 | 0.6 |
| CA_1A-7A-7A | 1 | 0.5 |
|  | 7 | 0.6 |
| CA_1A-7C | 1 | 0.5 |
|  | 7 | 0.6 |
| CA_1A-8A | 1 | 0.3 |
|  | 8 | 0.3 |
| CA_1A-11A | 1 | 0.3 |
|  | 11 | 0.3 |
| CA_1A-18A | 1 | 0.3 |
|  | 18 | 0.3 |
| CA_1A-19A | 1 | 0.3 |
|  | 19 | 0.3 |
| CA_1A-20A | 1 | 0.3 |
|  | 20 | 0.3 |
| CA_1A-21A | 1 | 0.3 |
|  | 21 | 0.3 |
| CA_1A-26A | 1 | 0.3 |
|  | 26 | 0.3 |
| CA_1A-28A | 1 | 0.3 |
|  | 28 | 0.6 |
| CA_1A-1A-28A | 1 | 0.3 |
|  | 28 | 0.6 |
| CA_1A-32A | 1 | 0.5 |
|  | 32 | N/A |
| CA_1A-38A | 1 | 0.5 |
|  | 38 | 0.5 |
| CA_1A-40A | 1 | 0.5 |
|  | 40 | 0.5 |
| CA_1A-40C | 1 | 0.5 |
|  | 40 | 0.5 |
| CA_1A-41A8 | 1 | 0.5 |
|  | 41 | 0.5 |
| CA_1A-41C8 | 1 | 0.5 |
|  | 41 | 0.5 |
| CA_1A-42A | 1 | 0.3 |
|  | 42 | 0.8 |
| CA_1A-42A-42A | 1 | 0.3 |
|  | 42 | 0.8 |
| CA_1A-42A-42C | 1 | 0.3 |
|  | 42 | 0.8 |
| CA_1A-42C-42C | 1 | 0.3 |
|  | 42 | 0.8 |
| CA_1A-42D | 1 | 0.3 |
|  | 42 | 0.8 |
| CA_1A-42E | 1 | 0.3 |
|  | 42 | 0.8 |
| CA_1A-46A | 1 | 0 |
| CA_1A-46C | 1 | 0 |
| CA_1A-46D | 1 | 0 |
| CA_1A-46E | 1 | 0 |
| CA_2A-4A | 2 | 0.5 |
|  | 4 | 0.5 |
| CA_2A-2A-4A | 2 | 0.5 |
|  | 4 | 0.5 |
| CA_2A-4A-4A | 2 | 0.5 |
|  | 4 | 0.5 |
| CA_2A-2A-4A-4A | 2 | 0.5 |
|  | 4 | 0.5 |
| CA_2A-5A | 2 | 0.3 |
|  | 5 | 0.3 |
| CA_2A-2A-5A | 2 | 0.3 |
|  | 5 | 0.3 |
| CA_2C-5A | 2 | 0.3 |
|  | 5 | 0.3 |
| CA_2A-5B | 2 | 0.3 |
|  | 5 | 0.3 |
| CA_2C-5B | 2 | 0.3 |
|  | 5 | 0.3 |
| CA_2A-7A | 2 | 0.5 |
|  | 7 | 0.5 |
| CA_2A-7C | 2 | 0.5 |
|  | 7 | 0.5 |
| CA_2A-7A-7A | 2 | 0.5 |
|  | 7 | 0.5 |
| CA_2A-12A | 2 | 0.3 |
|  | 12 | 0.3 |
| CA_2A-2A-12A | 2 | 0.3 |
|  | 12 | 0.3 |
| CA_2A-2A-12A-12A | 2 | 0.3 |
|  | 12 | 0.3 |
| CA_2A-12A-12A | 2 | 0.3 |
|  | 12 | 0.3 |
| CA_2A-2A-12B | 2 | 0.3 |
|  | 12 | 0.3 |
| CA_2A-12B | 2 | 0.3 |
|  | 12 | 0.3 |
| CA_2C-12A | 2 | 0.3 |
|  | 12 | 0.3 |
| CA_2A-13A | 2 | 0.3 |
|  | 13 | 0.3 |
| CA_2A-2A-13A | 2 | 0.3 |
|  | 13 | 0.3 |
| CA_2A-14A | 2 | 0.3 |
|  | 14 | 0.3 |
| CA_2A-2A-14A | 2 | 0.3 |
|  | 14 | 0.3 |
| CA_2A-17A | 2 | 0.3 |
|  | 17 | 0.8 |
| CA_2A-28A | 2 | 0.3 |
|  | 28 | 0.3 |
| CA_2A-29A | 2 | 0.3 |
| CA_2A-2A-29A | 2 | 0.3 |
| CA_2C-29A | 2 | 0.3 |
| CA_2A-30A | 2 | 0.5 |
|  | 30 | 0.3 |
| CA_2A-2A-30A | 2 | 0.5 |
|  | 30 | 0.3 |
| CA_2C-30A | 2 | 0.5 |
|  | 30 | 0.3 |
| CA_2A-46A | 2 | 0 |
| CA_2A-2A-46A | 2 | 0 |
| CA_2A-46A-46C | 2 | 0 |
| CA_2A-46C | 2 | 0 |
| CA_2A-2A-46C | 2 | 0 |
| CA_2A-46D | 2 | 0 |
| CA_2A-46A-46A | 2 | 0 |
| CA_2A-46A-46D | 2 | 0 |
| CA_2A-48A | 2 | 0.6 |
|  | 48 | 0.8 |
| CA_2A-48A-48A | 2 | 0.6 |
|  | 48 | 0.8 |
| CA_2A-48C | 2 | 0.6 |
|  | 48 | 0.8 |
| CA_2A-48A-48C | 2 | 0.6 |
|  | 48 | 0.8 |
| CA_2A-48A-48D | 2 | 0.6 |
|  | 48 | 0.8 |
| CA_2A-48C-48C | 2 | 0.6 |
|  | 48 | 0.8 |
| CA_2A-48D | 2 | 0.6 |
|  | 48 | 0.8 |
| CA_2A-48E | 2 | 0.6 |
|  | 48 | 0.8 |
| CA_2A-49A | 2 | 0.6 |
| CA_2A-66A | 2 | 0.5 |
|  | 66 | 0.5 |
| CA_2A-66B | 2 | 0.5 |
|  | 66 | 0.5 |
| CA_2A-66C | 2 | 0.5 |
|  | 66 | 0.5 |
| CA_2A-66D | 2 | 0.5 |
|  | 66 | 0.5 |
| CA_2A-2A-66A | 2 | 0.5 |
|  | 66 | 0.5 |
| CA_2A-2A-66A-66A | 2 | 0.5 |
|  | 66 | 0.5 |
| CA_2A-2A-66A-66B | 2 | 0.5 |
|  | 66 | 0.5 |
| CA_2A-2A-66A-66C | 2 | 0.5 |
|  | 66 | 0.5 |
| CA_2A-66A-66A | 2 | 0.5 |
|  | 66 | 0.5 |
| CA_2A-66A-66A-66A | 2 | 0.5 |
|  | 66 | 0.5 |
| CA_2A-66A-66B | 2 | 0.5 |
|  | 66 | 0.5 |
| CA_2A-66A-66C | 2 | 0.5 |
|  | 66 | 0.5 |
| CA_2A-2A-66B | 2 | 0.5 |
|  | 66 | 0.5 |
| CA_2A-2A-66C | 2 | 0.5 |
|  | 66 | 0.5 |
| CA_2A-2A-66D | 2 | 0.5 |
|  | 66 | 0.5 |
| CA_2C-66A | 2 | 0.5 |
|  | 66 | 0.5 |
| CA_2A-71A | 2 | 0.3 |
|  | 71 | 0.3 |
| CA_2A-2A-71A | 2 | 0.3 |
|  | 71 | 0.3 |
| CA_3A-3A-19A | 3 | 0.3 |
|  | 19 | 0.3 |
| CA_3A-3A-21A | 3 | 0.8 |
|  | 21 | 0.9 |
| CA_3A-3A-42A | 3 | 0.6 |
|  | 42 | 0.8 |
| CA_3A-3A-46A | 3 | 0 |
| CA_3A-5A | 3 | 0.3 |
|  | 5 | 0.3 |
| CA_3C-5A | 3 | 0.3 |
|  | 5 | 0.3 |
| CA_3A-7A | 3 | 0.5 |
|  | 7 | 0.5 |
| CA_3A-3A-7A | 3 | 0.5 |
|  | 7 | 0.5 |
| CA_3A-3A-7A-7A | 3 | 0.5 |
|  | 7 | 0.5 |
| CA_3A-7A-7A | 3 | 0.5 |
|  | 7 | 0.5 |
| CA_3A-7B | 3 | 0.5 |
|  | 7 | 0.5 |
| CA_3A-7C | 3 | 0.5 |
|  | 7 | 0.5 |
| CA_3C-7A | 3 | 0.5 |
|  | 7 | 0.5 |
| CA_3C-7C | 3 | 0.5 |
|  | 7 | 0.5 |
| CA_3A-8A | 3 | 0.3 |
|  | 8 | 0.3 |
| CA_3A-3A-8A | 3 | 0.3 |
|  | 8 | 0.3 |
| CA_3C-8A | 3 | 0.3 |
|  | 8 | 0.3 |
| CA_3A-11A | 3 | 0.8 |
|  | 11 | 0.9 |
| CA_3A-19A | 3 | 0.3 |
|  | 19 | 0.3 |
| CA_3A-20A | 3 | 0.3 |
|  | 20 | 0.3 |
| CA_3C-20A | 3 | 0.3 |
|  | 20 | 0.3 |
| CA_3A-3A-20A | 3 | 0.3 |
|  | 20 | 0.3 |
| CA_3A-21A | 3 | 0.8 |
|  | 21 | 0.9 |
| CA_3A-26A | 3 | 0.3 |
|  | 26 | 0.3 |
| CA_3A-27A | 3 | 0.3 |
|  | 27 | 0.3 |
| CA_3A-28A | 3 | 0.3 |
|  | 28 | 0.3 |
| CA_3C-28A | 3 | 0.3 |
|  | 28 | 0.3 |
| CA_3A-31A | 3 | 0.3 |
|  | 31 | 0.6 |
| CA_3A-32A | 3 | 0.5 |
| CA_3C-32A | 3 | 0.5 |
| CA_3A-38A | 3 | 0,5 |
|  | 38 | 0,5 |
| CA_3C-38A | 3 | 0.5 |
|  | 38 | 0.5 |
| CA_3A-40A | 3 | 0.5 |
|  | 40 | 0.5 |
| CA_3A-40A-40A | 3 | 0.5 |
|  | 40 | 0.5 |
| CA_3A-40C | 3 | 0.5 |
|  | 40 | 0.5 |
| CA_3A-40D | 3 | 0.5 |
|  | 40 | 0.5 |
| CA_3A-40E | 3 | 0.5 |
|  | 40 | 0.5 |
| CA_3C-40A | 3 | 0.5 |
|  | 40 | 0.5 |
| CA_3C-40C | 3 | 0.5 |
|  | 40 | 0.5 |
| CA_3A-41A | 3 | 0.5 |
|  | 41 | 0.310 |
|  |  | 0.811 |
| CA_3A-41C | 3 | 0.5 |
|  | 41 | 0.310 |
|  |  | 0.811 |
| CA_3A-41D | 3 | 0.5 |
|  | 41 | 0.310 |
|  |  | 0.811 |
| CA_3A-3A-41A | 3 | 0.5 |
|  | 41 | 0.310 |
|  |  | 0.811 |
| CA_3C-41A | 3 | 0.5 |
|  | 41 | 0.310 |
|  |  | 0.811 |
| CA_3C-41C | 3 | 0.5 |
|  | 41 | 0.310 |
|  |  | 0.811 |
| CA_3C-41D | 3 | 0.5 |
|  | 41 | 0.310 |
|  |  | 0.811 |
| CA_3A-42A | 3 | 0.6 |
|  | 42 | 0.8 |
| CA_3A-42A-42A | 3 | 0.6 |
|  | 42 | 0.8 |
| CA_3A-42C | 3 | 0.6 |
|  | 42 | 0.8 |
| CA_3A-42D | 3 | 0.6 |
|  | 42 | 0.8 |
| CA_3A-3A-42C | 3 | 0.6 |
|  | 42 | 0.8 |
| CA_3A-3A-42D | 3 | 0.6 |
|  | 42 | 0.8 |
| CA_3A-42A-42C | 3 | 0.6 |
|  | 42 | 0.8 |
| CA_3A-42C-42C | 3 | 0.6 |
|  | 42 | 0.8 |
| CA_3A-42E | 3 | 0.6 |
|  | 42 | 0.8 |
| CA_3A-46A | 3 | 0 |
| CA_3A-46C | 3 | 0 |
| CA_3A-46D | 3 | 0 |
| CA_3A-46E | 3 | 0 |
| CA_3A-69A | 3 | 0.5 |
| CA_4A-5A | 4 | 0.3 |
|  | 5 | 0.3 |
| CA_4A-4A-5A | 4 | 0.3 |
|  | 5 | 0.3 |
| CA_4A-5B | 4 | 0.3 |
|  | 5 | 0.3 |
| CA_4A-4A-5B | 4 | 0.3 |
|  | 5 | 0.3 |
| CA_4A-7A | 4 | 0.5 |
|  | 7 | 0.5 |
| CA_4A-4A-7A | 4 | 0.5 |
|  | 7 | 0.5 |
| CA_4A-7A-7A | 4 | 0.5 |
|  | 7 | 0.5 |
| CA_4A-7C | 4 | 0.5 |
|  | 7 | 0.5 |
| CA_4A-12A | 4 | 0.3 |
|  | 12 | 0.8 |
| CA_4A-12A-12A | 4 | 0.3 |
|  | 12 | 0.8 |
| CA_4A-4A-12A | 4 | 0.3 |
|  | 12 | 0.8 |
| CA_4A-4A-12A-12A | 4 | 0.3 |
|  | 12 | 0.8 |
| CA_4A-4A-12B | 4 | 0.3 |
|  | 12 | 0.8 |
| CA_4A-12B | 4 | 0.3 |
|  | 12 | 0.8 |
| CA_4A-13A | 4 | 0.3 |
|  | 13 | 0.3 |
| CA_4A-4A-13A | 4 | 0.3 |
|  | 13 | 0.3 |
| CA_4A-17A | 4 | 0.3 |
|  | 17 | 0.8 |
| CA_4A-27A | 4 | 0.3 |
|  | 27 | 0.3 |
| CA_4A-28A | 4 | 0.3 |
|  | 28 | 0.6 |
| CA_4A-29A | 4 | 0.3 |
| CA_4A-4A-29A | 4 | 0.3 |
| CA_4A-30A | 4 | 0.5 |
|  | 30 | 0.3 |
| CA_4A-4A-30A | 4 | 0.5 |
|  | 30 | 0.3 |
| CA_4A-46A | 4 | 0 |
| CA_4A-46A-46C | 4 | 0 |
| CA_4A-46C | 4 | 0 |
| CA_4A-46D | 4 | 0 |
| CA_4A-46A-46A | 4 | 0 |
| CA_4A-46A-46D | 4 | 0 |
| CA_4A-4A-71A | 4 | 0.3 |
|  | 71 | 0.3 |
| CA_4A-71A | 4 | 0.3 |
|  | 71 | 0.3 |
| CA_5A-7A | 5 | 0.3 |
|  | 7 | 0.3 |
| CA_5A-7A-7A | 5 | 0.3 |
|  | 7 | 0.3 |
| CA_5A-7C | 5 | 0.3 |
|  | 7 | 0.3 |
| CA_5A-12A | 5 | 0.8 |
|  | 12 | 0.4 |
| CA_5A-12A-12A | 5 | 0.8 |
|  | 12 | 0.4 |
| CA_5A-12B | 5 | 0.8 |
|  | 12 | 0.4 |
| CA_5A-13A | 5 | 0.5 |
|  | 13 | 0.5 |
| CA_5A-17A | 5 | 0.8 |
|  | 17 | 0.4 |
| CA_5A-25A | 5 | 0.3 |
|  | 25 | 0.3 |
| CA_5A-29A | 5 | 0.5 |
| CA_5A-30A | 5 | 0.3 |
|  | 30 | 0.3 |
| CA_5B-30A | 5 | 0.3 |
|  | 30 | 0.3 |
| CA_5A-38A | 5 | 0.3 |
|  | 38 | 0.3 |
| CA_5A-40A | 5 | 0.3 |
|  | 40 | 0.3 |
| CA_5A-5A-40A | 5 | 0.3 |
|  | 40 | 0.3 |
| CA_5A-40A-40A | 5 | 0.3 |
|  | 40 | 0.3 |
| CA_5A-40C | 5 | 0.3 |
|  | 40 | 0.3 |
| CA_5A-41A | 5 | 0.3 |
|  | 41 | 0.3 |
| CA_5A-46A | 5 | 0 |
| CA_5A-46C | 5 | 0 |
| CA_5A-46D | 5 | 0 |
| CA_5A-46E | 5 | 0 |
| CA_5A-46C-66A | 5 | 0.3 |
|  | 66 | 0.3 |
| CA_5A-48A | 5 | 0.3 |
|  | 48 | 0.3 |
| CA_5A-66A | 5 | 0.3 |
|  | 66 | 0.3 |
| CA_5A-5A-66A | 5 | 0.3 |
|  | 66 | 0.3 |
| CA_5A-5A-66A-66A | 5 | 0.3 |
|  | 66 | 0.3 |
| CA_5A-5A-66A-66B | 5 | 0.3 |
|  | 66 | 0.3 |
| CA_5A-5A-66A-66C | 5 | 0.3 |
|  | 66 | 0.3 |
| CA_5A-5A-66B | 5 | 0.3 |
|  | 66 | 0.3 |
| CA_5A-5A-66C | 5 | 0.3 |
|  | 66 | 0.3 |
| CA_5A-5A-66D | 5 | 0.3 |
|  | 66 | 0.3 |
| CA_5A-66A-66A | 5 | 0.3 |
|  | 66 | 0.3 |
| CA_5A-66A-66B | 5 | 0.3 |
|  | 66 | 0.3 |
| CA_5A-66A-66C | 5 | 0.3 |
|  | 66 | 0.3 |
| CA_5A-66B | 5 | 0.3 |
|  | 66 | 0.3 |
| CA_5A-66C | 5 | 0.3 |
|  | 66 | 0.3 |
| CA_5A-66D | 5 | 0.3 |
|  | 66 | 0.3 |
| CA_5B-66A | 5 | 0.3 |
|  | 66 | 0.3 |
| CA_5B-66A-66A | 5 | 0.3 |
|  | 66 | 0.3 |
| CA_5B-66A-66B | 5 | 0.3 |
|  | 66 | 0.3 |
| CA_5B-66A-66C | 5 | 0.3 |
|  | 66 | 0.3 |
| CA_5B-66B | 5 | 0.3 |
|  | 66 | 0.3 |
| CA_5B-66C | 5 | 0.3 |
|  | 66 | 0.3 |
| CA_7A-8A | 7 | 0.3 |
|  | 8 | 0.6 |
| CA_7A-7A-8A | 7 | 0.3 |
|  | 8 | 0.6 |
| CA_7A-12A | 7 | 0.3 |
|  | 12 | 0.3 |
| CA_7A-20A | 7 | 0.3 |
|  | 20 | 0.3 |
| CA_7C-20A | 7 | 0.5 |
|  | 20 | 0.3 |
| CA_7A-22A | 7 | 0.5 |
|  | 22 | 0.8 |
| CA_7A-26A | 7 | 0.3 |
|  | 26 | 0.3 |
| CA_7A-7A-26A | 7 | 0.3 |
|  | 26 | 0.3 |
| CA_7A-28A | 7 | 0.3 |
|  | 28 | 0.3 |
| CA_7B-28A | 7 | 0.3 |
|  | 28 | 0.3 |
| CA_7C-28A | 7 | 0.3 |
|  | 28 | 0.3 |
| CA_7A-32A | 7 | 0.7 |
| CA_7A-40A | 7 | 0.5 |
|  | 40 | [0.6] |
| CA_7A-40C | 7 | 0.5 |
|  | 40 | [0.6] |
| CA_7A-40D | 7 | 0.5 |
|  | 40 | [0.6] |
| CA_7A-42A | 7 | 0.5 |
|  | 42 | 0.8 |
| CA_7A-42A-42A | 7 | 0.5 |
|  | 42 | 0.8 |
| CA_7A-46A | 7 | 0 |
| CA_7A-7A-46A | 7 | 0 |
| CA_7A-46C | 7 | 0 |
| CA_7A-46D | 7 | 0 |
| CA_7A-46E | 7 | 0 |
| CA_7C-46A | 7 | 0 |
| CA_7C-46C | 7 | 0 |
| CA_7C-46D | 7 | 0 |
| CA_7A-66A | 7 | 0.5 |
|  | 66 | 0.5 |
| CA_7A-66A-66A | 7 | 0.5 |
|  | 66 | 0.5 |
| CA_7C-66A | 7 | 0.3 |
|  | 66 | 0.3 |
| CA_7C-66A-66A | 7 | 0.5 |
|  | 66 | 0.5 |
| CA_8A-11A | 8 | 0.3 |
|  | 11 | 0.4 |
| CA_8A-20A | 8 | 0.4 |
|  | 20 | 0.4 |
| CA_8A-28A14 | 8 | 0.6 |
|  | 28 | 0.5 |
| CA_8A-32A | 8 | 0.3 |
| CA_8A-38A | 8 | 0.3 |
|  | 38 | 0.3 |
| CA_8A-39A | 8 | 0,3 |
|  | 39 | 0,3 |
| CA_8A-39C | 8 | 0.3 |
|  | 39 | 0.3 |
| CA_8B-39A | 8 | 0.3 |
|  | 39 | 0.3 |
| CA_8B-39C | 8 | 0.3 |
|  | 39 | 0.3 |
| CA_8A-40A | 8 | 0.3 |
|  | 40 | 0.3 |
| CA_8A-40C | 8 | 0.3 |
|  | 40 | 0.3 |
| CA_8A-41A | 8 | 0.3 |
|  | 41 | 0.3 |
| CA_8A-41C | 8 | 0.3 |
|  | 41 | 0.3 |
| CA_8A-41D | 8 | 0.3 |
|  | 41 | 0.3 |
| CA_8B-41A | 8 | 0.3 |
|  | 41 | 0.3 |
| CA_8B-41C | 8 | 0.3 |
|  | 41 | 0.3 |
| CA_8B-41D | 8 | 0.3 |
|  | 41 | 0.3 |
| CA_8A-42A | 8 | 0.6 |
|  | 42 | 0.8 |
| CA_8A-42C | 8 | 0.6 |
|  | 42 | 0.8 |
| CA_8A-46A | 8 | 0 |
| CA_8A-46C | 8 | 0 |
| CA_8A-46D | 8 | 0 |
| CA_8A-46E | 8 | 0 |
| CA_8B-46A | 8 | 0 |
| CA_8B-46C | 8 | 0 |
| CA_8B-46D | 8 | 0 |
| CA_11A-18A | 11 | 0.3 |
|  | 18 | 0.3 |
| CA_11A-26A | 11 | 0.3 |
|  | 26 | 0.3 |
| CA_11A-28A | 11 | 0.4 |
|  | 28 | 0.6 |
| CA_11A-41A | 11 | 0.3 |
|  | 41 | 0.3 |
| CA_11A-41C | 11 | 0.3 |
|  | 41 | 0.3 |
| CA_11A-42A | 11 | 0.4 |
|  | 42 | 0.8 |
| CA_11A-42C | 11 | 0.4 |
|  | 42 | 0.8 |
| CA_11A-46A | 11 | 0 |
| CA_11A-46C | 11 | 0 |
| CA_11A-46D | 11 | 0 |
| CA_11A-46E | 11 | 0 |
| CA_12A-25A | 12 | 0.3 |
|  | 25 | 0.3 |
| CA_12A-30A | 12 | 0.3 |
|  | 30 | 0.3 |
| CA_12A-46A | 12 | 0 |
|  | 46 | 0 |
| CA_12A-46C | 12 | 0 |
|  | 46 | 0 |
| CA_12A-46D | 12 | 0 |
|  | 46 | 0 |
| CA_12A-46E | 12 | 0 |
|  | 46 | 0 |
| CA_12A-66A | 12 | 0.8 |
|  | 66 | 0.3 |
| CA_12A-66A-66A | 12 | 0.8 |
|  | 66 | 0.3 |
| CA_12A-66C | 12 | 0.8 |
|  | 66 | 0.3 |
| CA_12B-66A | 12 | 0.8 |
|  | 66 | 0.3 |
| CA_12B-66A-66A | 12 | 0.8 |
|  | 66 | 0.3 |
| CA_13A-46A | 13 | 0 |
| CA_13A-46C | 13 | 0 |
| CA_13A-46D | 13 | 0 |
| CA_13A-46E | 13 | 0 |
| CA_13A-48A | 13 | 0.3 |
|  | 48 | 0.3 |
| CA_13A-48A-48A | 13 | 0.3 |
|  | 48 | 0.3 |
| CA_13A-48C | 13 | 0.3 |
|  | 48 | 0.3 |
| CA_13A-48A-48C | 13 | 0.3 |
|  | 48 | 0.3 |
| CA_13A-48A-48D | 13 | 0.3 |
|  | 48 | 0.3 |
| CA_13A-48C-48C | 13 | 0.3 |
|  | 48 | 0.3 |
| CA_13A-48D | 13 | 0.3 |
|  | 48 | 0.3 |
| CA_13A-48E | 13 | 0.3 |
|  | 48 | 0.3 |
| CA_13A-66A | 13 | 0.3 |
|  | 66 | 0.3 |
| CA_13A-66A-66A | 13 | 0.3 |
|  | 66 | 0.3 |
| CA_13A-66A-66C | 13 | 0.3 |
|  | 66 | 0.3 |
| CA_13A-66A-66D | 13 | 0.3 |
|  | 66 | 0.3 |
| CA_13A-66B | 13 | 0.3 |
|  | 66 | 0.3 |
| CA_13A-66C | 13 | 0.3 |
|  | 66 | 0.3 |
| CA_13A-66D | 13 | 0.3 |
|  | 66 | 0.3 |
| CA_14A-66A | 14 | 0.3 |
|  | 66 | 0.3 |
| CA_14A-66A-66A | 14 | 0.3 |
|  | 66 | 0.3 |
| CA_14A-30A | 14 | 0.3 |
|  | 30 | 0.3 |
| CA_14A-66A-66A-66A | 14 | 0.3 |
|  | 66 | 0.3 |
| CA_18A-28A9 | 18 | 0.5 |
|  | 28 | 0.5 |
| CA_19A-21A | 19 | 0.3 |
|  | 21 | 0.4 |
| CA_19A-28A9 | 19 | 0.5 |
|  | 28 | 0.5 |
| CA_19A-42A | 19 | 0.3 |
|  | 42 | 0.8 |
| CA_19A-42C | 19 | 0.3 |
|  | 42 | 0.8 |
| CA_19A-46A | 19 | 0 |
| CA_19A-46C | 19 | 0 |
| CA_19A-46D | 19 | 0 |
| CA_19A-46E | 19 | 0 |
| CA_20A-28A | 20 | 0.5 |
|  | 28 | 0.5 |
| CA_20A-31A | 20 | 0.5 |
|  | 31 | 0.5 |
| CA_20A-32A | 20 | 0.3 |
| CA_20A-38A | 20 | 0.3 |
|  | 38 | 0.3 |
| CA_20A-38C | 20 | 0.3 |
|  | 38 | 0.3 |
| CA_20A-40A | 20 | 0.3 |
|  | 40 | 0.3 |
| CA_20A-40A-40A | 20 | 0.3 |
|  | 40 | 0.3 |
| CA_20A-40C | 20 | 0.3 |
|  | 40 | 0.3 |
| CA_20A-40D | 20 | 0.3 |
|  | 40 | 0.3 |
| CA_20A-42A | 20 | 0.6 |
|  | 42 | 0.8 |
| CA_20A-42A-42A | 20 | 0.6 |
|  | 42 | 0.8 |
| CA_20A-67A | 20 | 0.5 |
| CA_20A-75A | 20 | 0.3 |
| CA_20A-76A | 20 | 0.3 |
| CA_21A-28A | 21 | 0.4 |
|  | 28 | 0.3 |
| CA_21A-42A | 21 | 0.4 |
|  | 42 | 0.8 |
| CA_21A-42C | 21 | 0.4 |
|  | 42 | 0.8 |
| CA_21A-46A | 21 | 0 |
| CA_21A-46C | 21 | 0 |
| CA_21A-46D | 21 | 0 |
| CA_21A-46E | 21 | 0 |
| CA_23A-29A | 23 | 0.3 |
| CA_25A-26A | 25 | 0.3 |
|  | 26 | 0.3 |
| CA_25A-25A-26A | 25 | 0.3 |
|  | 26 | 0.3 |
| CA_25A-41A8 | 25 | 0.5 |
|  | 41 | 0.5 |
| CA_25A-41C8 | 25 | 0.5 |
|  | 41 | 0.5 |
| CA_25A-41D8 | 25 | 0.5 |
|  | 41 | 0.5 |
| CA_26A-41A | 26 | 0.3 |
|  | 41 | 0.3 |
| CA_26A-41C | 26 | 0.3 |
|  | 41 | 0.3 |
| CA_26A-46A | 26 | 0 |
| CA_26A-48A | 26 | 0.3 |
|  | 48 | 0.3 |
| CA_28A-38A | 28 | 0.3 |
|  | 38 | 0.3 |
| CA_28A-40A | 28 | 0.3 |
|  | 40 | 0.3 |
| CA_28A-40C | 28 | 0.3 |
|  | 40 | 0.3 |
| CA_28A-40D | 28 | 0.3 |
|  | 40 | 0.3 |
| CA_28A-41A | 28 | 0.3 |
|  | 41 | 0.3 |
| CA_28A-41C | 28 | 0.3 |
|  | 41 | 0.3 |
| CA_28A-42A | 28 | 0.5 |
|  | 42 | 0.8 |
| CA_28A-42C | 28 | 0.5 |
|  | 42 | 0.8 |
| CA_28A-46A | 28 | 0 |
| CA_28A-46C | 28 | 0 |
| CA_28A-46D | 28 | 0 |
| CA_28A-46E | 28 | 0 |
| CA_29A-30A | 30 | 0.3 |
| CA_29A-66A | 66 | 0.3 |
| CA_29A-66A-66A | 66 | 0.3 |
| CA_29A-66C | 66 | 0.3 |
| CA_29A-70A | 70 | 0.3 |
| CA_29A-70C |  |  |
|  | 70 | 0.3 |
| CA_30A-66A | 30 | 0.3 |
|  | 66 | 0.5 |
| CA_30A-66A-66A | 30 | 0.3 |
|  | 66 | 0.5 |
| CA_34A-39A | 34 | 01 |
|  | 39 | 01 |
| CA_34A-41A | 34 | 01 |
|  | 41 | 01 |
| CA_38A-40A | 38 | 04 |
|  | 40 | 04 |
| CA_38A-40A-40A | 38 | 04 |
|  | 40 | 04 |
| CA_38A-40C | 38 | 04 |
|  | 40 | 04 |
| CA_38A-40D | 38 | 04 |
|  | 40 | 04 |
| CA_39A-40A | 39 | 04 |
|  | 40 | 04 |
| CA_39A-40C | 39 | 04 |
|  | 40 | 04 |
| CA_39A-40D | 39 | 0 |
|  | 40 | 0 |
| CA_39A-40E | 39 | 04 |
|  | 40 | 04 |
| CA_39C-40A | 39 | 04 |
|  | 40 | 04 |
| CA_39C-40C | 39 | 0 |
|  | 40 | 0 |
| CA_39C-40D | 39 | 04 |
|  | 40 | 04 |
| CA_39A-41A | 39 | 04 |
|  | 41 | 04 |
| CA_39A-41A | 39 | 0.57 |
|  | 41 | 0.57 |
| CA_39A-41C | 39 | 04 |
|  | 41 | 04 |
| CA_39A-41C | 39 | 0.57 |
|  | 41 | 0.57 |
| CA_39A-41D | 39 | 04 |
|  | 41 | 04 |
| CA_39A-42A | 39 | 04 |
|  | 42 | 0.54 |
| CA_39A-46D | 39 | 0 |
| CA_39A-46E | 39 | 0 |
| CA_39C-41A | 39 | 04 |
|  | 41 | 04 |
| CA_39C-41A | 39 | 0.57 |
|  | 41 | 0.57 |
| CA_39C-41C | 39 | 04 |
|  | 41 | 04 |
| CA_39C-41D | 39 | 04 |
|  | 41 | 04 |
| CA_39A-42C | 39 | 04 |
|  | 42 | 0.54 |
| CA_39A-42D | 39 | 0 |
|  | 42 | 0.5 |
| CA_39A-42E | 39 | 04 |
|  | 42 | 0.54 |
| CA_39C-42A | 39 | 04 |
|  | 42 | 0.54 |
| CA_39C-42C | 39 | 0 |
|  | 42 | 0.5 |
| CA_39C-42D | 39 | 04 |
|  | 42 | 0.54 |
| CA_39A-46A | 39 | 0 |
| CA_39A-46C | 39 | 0 |
| CA_39C-46A | 39 | 0 |
| CA_39C-46C | 39 | 0 |
| CA_39C-46D | 39 | 0 |
| CA_40A-41A | 40 | 0.54 |
|  | 41 | 0.54 |
| CA_40A-42A | 40 | 04 |
|  | 42 | 0.54 |
| CA_40A-42C | 40 | 04 |
|  | 42 | 0.54 |
| CA_40C-42A | 40 | 04 |
|  | 42 | 0.54 |
| CA_40C-42C | 40 | 04 |
|  | 42 | 0.54 |
| CA_40A-43A | 40 | 04 |
|  | 43 | 0.54 |
| CA_40A-46A | 40 | 0 |
| CA_40A-46C | 40 | 0 |
| CA_40A-46D | 40 | 0 |
| CA_40A-46E | 40 | 0 |
| CA_40C-46A | 40 | 0 |
| CA_40C-46C | 40 | 0 |
| CA_40C-46D | 40 | 0 |
| CA_40D-46A | 40 | 0 |
| CA_40D-46C | 40 | 0 |
| CA_41A-42A | 41 | 04 |
|  | 42 | 0.54 |
| CA_41A-42A | 41 | 0.37 |
|  | 42 | 0.87 |
| CA_41A-42C | 41 | 04 |
|  | 42 | 0.54 |
| CA_41A-42C | 41 | 0.37 |
|  | 42 | 0.87 |
| CA_41A-42D | 41 | 04 |
|  | 42 | 0.54 |
| CA_41C-42A | 41 | 04 |
|  | 42 | 0.54 |
| CA_41C-42A | 41 | 0.37 |
|  | 42 | 0.87 |
| CA_41C-42C | 41 | 04 |
|  | 42 | 0.54 |
| CA_41C-42C | 41 | 0.37 |
|  | 42 | 0.87 |
| CA_41D-42A | 41 | 04 |
|  | 42 | 0.54 |
| CA_41C-42D | 41 | 04 |
|  | 42 | 0.54 |
| CA_41D-42C | 41 | 04 |
|  | 42 | 0.54 |
| CA_41A-46A | 41 | 0 |
| CA_41A-46C | 41 | 0 |
| CA_41A-46D | 41 | 0 |
| CA_41A-46E | 41 | 0 |
| CA_41C-46A | 41 | 0 |
| CA_41C-46C | 41 | 0 |
| CA_41C-46D | 41 | 0 |
| CA_41D-46A | 41 | 0 |
| CA_41D-46C | 41 | 0 |
| CA_42A-46A | 42 | [0.5] |
| CA_46A-46C-66A | 66 | 0 |
| CA_46A-46D-66A | 66 | 0 |
| CA_46A-66A | 66 | 0 |
| CA_46A-66A-66A | 66 | 0 |
| CA_46C-66A-66A | 66 | 0 |
| CA_46A-66C | 66 | 0 |
| CA_46C-66A | 66 | 0 |
| CA_46D-66A | 66 | 0 |
| CA_46E-66A | 66 | 0 |
| CA_46A-46A-66A | 66 | 0 |
| CA_46A-70A | 70 | 0 |
| CA_48A-66A | 48 | 0.8 |
|  | 66 | 0.6 |
| CA_48A-66B | 48 | 0.8 |
|  | 66 | 0.6 |
| CA_48A-66C | 48 | 0.8 |
|  | 66 | 0.6 |
| CA_48A-48A-66A | 48 | 0.8 |
|  | 66 | 0.6 |
| CA_48A-48C-66A | 48 | 0.8 |
|  | 66 | 0.6 |
| CA_48A-48C-66B | 48 | 0.8 |
|  | 66 | 0.6 |
| CA_48A-48C-66C | 48 | 0.8 |
|  | 66 | 0.6 |
| CA_48A-48D-66A | 48 | 0.8 |
|  | 66 | 0.6 |
| CA_48C-48C-66A | 48 | 0.8 |
|  | 66 | 0.6 |
| CA_48A-66A-66A | 48 | 0.8 |
|  | 66 | 0.6 |
| CA_48A-48A-66A-66A | 48 | 0.8 |
|  | 66 | 0.6 |
| CA_48A-48A-66B | 48 | 0.8 |
|  | 66 | 0.6 |
| CA_48A-48A-66C | 48 | 0.8 |
|  | 66 | 0.6 |
| CA_48C-66B | 48 | 0.8 |
|  | 66 | 0.6 |
| CA_48C-66C | 48 | 0.8 |
|  | 66 | 0.6 |
| CA_48C-66A | 48 | 0.8 |
|  | 66 | 0.6 |
| CA_48D-66A | 48 | 0.8 |
|  | 66 | 0.6 |
| CA_48E-66A | 48 | 0.8 |
|  | 66 | 0.6 |
| CA_66A-70A | 66 | 0.5 |
|  | 70 | 0.5 |
| CA_66A-66A-70A | 66 | 0.5 |
|  | 70 | 0.5 |
| CA_66A-70C | 66 | 0.5 |
|  | 70 | 0.5 |
| CA_66A-66A-70C | 66 | 0.5 |
|  | 70 | 0.5 |
| CA_66C-70A | 66 | 0.5 |
| CA_66C-70C | 66 | 0.5 |
|  | 70 | 0.5 |
| CA_66A-71A | 66 | 0.3 |
|  | 71 | 0.3 |
| CA_66A-66A-71A | 66 | 0.3 |
|  | 71 | 0.3 |
| CA_66C-71A | 66 | 0.3 |
|  | 71 | 0.3 |
| CA_70A-71A | 70 | 0.3 |
|  | 71 | 0.6 |
| CA_70C-71A | 70 | 0.3 |
|  | 71 | 0.6 |
| NOTE 1: The above additional tolerances are only applicable for the E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations NOTE 2: The above additional tolerances also apply in non-aggregated operation for the supported E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations NOTE 3: In case the UE supports more than one of the above 2DL inter-band carrier aggregation configurations and a E-UTRA operating band belongs to more than one 2DL inter-band carrier aggregation configurations then: - When the E-UTRA operating band frequency range is ≤ 1GHz, the applicable additional tolerance shall be the average of the 2DL tolerances above, truncated to one decimal place for that operating band among the supported 2DL CA configurations. In case there is a harmonic relation between low band UL and high band DL, then the maximum tolerance among the different supported 2DL carrier aggregation configurations involving such band shall be applied - When the E-UTRA operating band frequency range is >1GHz, the applicable additional 2DL tolerance shall be the maximum tolerance above that applies for that operating band among the supported 2DL CA configurations NOTE 4: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. NOTE 5: Unless otherwise specified, in case the UE supports more than one of the above 3DL inter-band carrier aggregation configurations and a E-UTRA operating band belongs to more than one 3DL inter-band carrier aggregation configurations then: - When the E-UTRA operating band frequency range is ≤ 1GHz and the tolerances are the same, the value applies to the band. If the tolerances are different, the applicable additional 3DL tolerance is FFS. In case there is a harmonic relation between low band UL and high band DL, then the maximum tolerance among the different supported 3DL carrier aggregation configurations involving such band shall be applied - When the E-UTRA operating band frequency range is >1GHz, the applicable additional 3DL tolerance shall be the maximum tolerance above that applies for that operating band among the supported 3DL CA configurations. NOTE 6: The above additional tolerances applicable for the E-UTRA operating bands that belong to the supported highest order inter-band carrier aggregation configuration, also applies to the same E-UTRA operating bands that belong to a supported lower order CA configuration. NOTE 7: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. NOTE 8: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in the FDD band. NOTE 9: For Band 28, the requirements only apply for the restricted frequency range specified for this CA configuration (Table 5.5A-2). NOTE 10: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 11: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. NOTE 12: For UE supporting E-UTRA band 65 and CA configurations including Band 1, the Band 65 ΔTIB,c is the max(Band 65 ΔTIB,c , Band 1 ΔTIB,c) NOTE 13: For UE supporting E-UTRA band 42, 43 or 48 and CA configurations including Band 42, 43 or 48, the applicable ΔTIB,c in Band 42, 43, or 48 is the max(Band 42 ΔTIB,c , Band 43 ΔTIB,c, Band 48 ΔTIB,c). NOTE 14: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 8. |  |  |

NOTE:	The above additional tolerances do not apply to supported UTRA operating bands with frequency range below 1 GHz that correspond to the E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations when such bands are belonging only to band combination(s) where one band is <1GHz and another band is >1.7GHz and there is no harmonic relationship between the low band UL and high band DL. Otherwise the above additional tolerances also apply to supported UTRA operating bands that correspond to the E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations.

NOTE:	To meet the TIB,c requirements for CA_3A-7A with state-of-the-art technology, an increase in power consumption of the UE may be required. It is also expected that as the state-of-the-art technology evolves in the future, this possible power consumption increase can be reduced or eliminated.

Table 6.2.5-3: ΔTIB,c (three bands)

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1A-3A-5A | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 5 | 0.3 |
| CA_1A-1A-3A-5A | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 5 | 0.3 |
| CA_1A-3C-5A | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 5 | 0.3 |
| CA_1A-3A-7A | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
| CA_1A-1A-3A-7A | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
| CA_1A-1A-3C-7A | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
| CA_1A-3A-7A-7A | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
| CA_1A-3A-7C | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
| CA_1A-3C-7A | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
| CA_1A-3C-7C | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
| CA_1A-3A-8A | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 8 | 0.3 |
| CA_1A-3A-3A-8A | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 8 | 0.3 |
| CA_1A-3C-8A | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 8 | 0.3 |
| CA_1A-3A-11A | 1 | 0.3 |
|  | 3 | 0.8 |
|  | 11 | 0.9 |
| CA_1A-3A-19A | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 19 | 0.3 |
| CA_1A-3A-3A-19A | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 19 | 0.3 |
| CA_1A-3A-20A | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 20 | 0.3 |
| CA_1A-3C-20A | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 20 | 0.3 |
| CA_1A-3A-21A | 1 | 0.3 |
|  | 3 | 0.8 |
|  | 21 | 0.9 |
| CA_1A-3A-3A-21A | 1 | 0.3 |
|  | 3 | 0.8 |
|  | 21 | 0.9 |
| CA_1A-3A-26A | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 26 | 0.3 |
| CA_1A-3A-28A | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 28 | 0.6 |
| CA_1A-1A-3A-28A | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 28 | 0.6 |
| CA_1A-3C-28A | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 28 | 0.6 |
| CA_1A-3A-38A | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 38 | 0.5 |
| CA_1A-3A-40A | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 40 | 0.5 |
| CA_1A-3A-40C | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 40 | 0.5 |
| CA_1A-3C-40A | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 40 | 0.5 |
| CA_1A-3C-40C | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 40 | 0.5 |
| CA_1A-3A-41A | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 41 | 0.35/0.86 |
| CA_1A-3A-42A | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 42 | 0.8 |
| CA_1A-3A-3A-42A | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 42 | 0.8 |
| CA_1A-3A-42C | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 42 | 0.8 |
| CA_1A-3A-3A-42C | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 42 | 0.8 |
| CA_1A-5A-7A | 1 | 0.5 |
|  | 5 | 0.3 |
|  | 7 | 0.6 |
| CA_1A-5A-7A-7A | 1 | 0.5 |
|  | 5 | 0.3 |
|  | 7 | 0.6 |
| CA_1A-5A-40A | 1 | 0.5 |
|  | 5 | 0.3 |
|  | 40 | 0.5 |
| CA_1A-5A-41A | 1 | 0.5 |
|  | 5 | 0.3 |
|  | 41 | 0.5 |
| CA_1A-5A-46A | 1 | 0.3 |
|  | 5 | 0.3 |
| CA_1A-5A-46C | 1 | 0.3 |
|  | 5 | 0.3 |
| CA_1A-5A-46D | 1 | 0.3 |
|  | 5 | 0.3 |
| CA_1A-7A-8A | 1 | 0.5 |
|  | 7 | 0.6 |
|  | 8 | 0.6 |
| CA_1A-7A-20A | 1 | 0.5 |
|  | 7 | 0.6 |
|  | 20 | 0.3 |
| CA_1A-7A-26A | 1 | 0.5 |
|  | 7 | 0.6 |
|  | 26 | 0.3 |
| CA_1A-7A-7A-26A | 1 | 0.5 |
|  | 7 | 0.6 |
|  | 26 | 0.3 |
| CA_1A-7A-28A | 1 | 0.5 |
|  | 7 | 0.6 |
|  | 28 | 0.6 |
| CA_1A-7C-28A | 1 | 0.5 |
|  | 7 | 0.6 |
|  | 28 | 0.6 |
| CA_1A-7A-32A | 1 | 0.7 |
|  | 7 | 0.7 |
| CA_1A-7A-40A | 1 | 0.6 |
|  | 7 | 0.8 |
|  | 40 | 0.9 |
| CA_1A-7A-40C | 1 | 0.6 |
|  | 7 | 0.8 |
|  | 40 | 0.9 |
| CA_1A-7A-42A | 1 | 0.6 |
|  | 7 | 0.6 |
|  | 42 | 0.8 |
| CA_1A-7A-46A | 1 | 0.5 |
|  | 7 | 0.8 |
| CA_1A-7A-46C | 1 | 0.5 |
|  | 7 | 0.6 |
| CA_1A-7A-46D | 1 | 0.5 |
|  | 7 | 0.6 |
| CA_1A-8A-11A | 1 | 0.3 |
|  | 8 | 0.3 |
|  | 11 | 0.4 |
| CA_1A-8A-20A | 1 | 0.3 |
|  | 8 | 0.4 |
|  | 20 | 0.4 |
| CA_1A-8A-28A11 | 1 | 0.3 |
|  | 8 | 0.6 |
|  | 28 | 0.6 |
| CA_1A-8A-38A | 1 | 0.5 |
|  | 8 | 0.3 |
|  | 38 | 0.5 |
| CA_1A-8A-40A | 1 | 0.5 |
|  | 8 | 0.3 |
|  | 40 | 0.5 |
| CA_1A-8A-40C | 1 | 0.5 |
|  | 8 | 0.3 |
|  | 40 | 0.5 |
| CA_1A-11A-18A | 1 | 0.3 |
|  | 11 | 0.4 |
|  | 18 | 0.3 |
| CA_1A-11A-28A | 1 | 0.3 |
|  | 11 | 0.4 |
|  | 28 | 0.6 |
| CA_1A-18A-28A | 1 | 0.3 |
|  | 18 | 0.5 |
|  | 28 | 0.5 |
| CA_1A-19A-21A | 1 | 0.3 |
|  | 19 | 0.3 |
|  | 21 | 0.4 |
| CA_1A-19A-28A | 1 | 0.3 |
|  | 19 | 0.5 |
|  | 28 | 0.5 |
| CA_1A-19A-42A | 1 | 0.3 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
| CA_1A-19A-42C | 1 | 0.3 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
| CA_1A-20A-28A | 1 | 0.3 |
|  | 20 | 0.6 |
|  | 28 | 0.6 |
| CA_1A-20A-32A | 1 | 0.5 |
|  | 20 | 0.3 |
| CA_1A-20A-42A | 1 | 0.3 |
|  | 20 | 0.3 |
|  | 42 | 0.8 |
| CA_1A-21A-28A | 1 | 0.3 |
|  | 21 | 0.4 |
|  | 28 | 0.6 |
| CA_1A-21A-42A | 1 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
| CA_1A-21A-42C | 1 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
| CA_1A-28A-42A | 1 | 0.3 |
|  | 28 | 0.6 |
|  | 42 | 0.8 |
| CA_1A-28A-42C | 1 | 0.3 |
|  | 28 | 0.6 |
|  | 42 | 0.8 |
| CA_1A-41A-42A8, 13 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | 42 | 0.8 |
| CA_1A-41A-42C8, 13 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | 42 | 0.8 |
| CA_1A-41C-42C8, 13 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | 42 | 0.8 |
| CA_1A-41C-42A8, 13 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | 42 | 0.8 |
| CA_2A-2A-4A-12A | 2 | 0.5 |
|  | 4 | 0.5 |
|  | 12 | 0.8 |
| CA_2A-4A-5A | 2 | 0.5 |
|  | 4 | 0.5 |
|  | 5 | 0.3 |
| CA_2A-2A-4A-5A | 2 | 0.5 |
|  | 4 | 0.5 |
|  | 5 | 0.3 |
| CA_2A-2A-14A-66A-66A | 2 | 0.5 |
|  | 14 | 0.3 |
|  | 66 | 0.5 |
| CA_2A-4A-4A-5A | 2 | 0.5 |
|  | 4 | 0.5 |
|  | 5 | 0.3 |
| CA_2A-4A-5B | 2 | 0.5 |
|  | 4 | 0.5 |
|  | 5 | 0.3 |
| CA_2A-4A-7A | 2 | 0.5 |
|  | 4 | 0.5 |
|  | 7 | 0.5 |
| CA_2A-4A-7C | 2 | 0.5 |
|  | 4 | 0.5 |
|  | 7 | 0.5 |
| CA_2A-4A-7A-7A | 2 | 0.5 |
|  | 4 | 0.5 |
|  | 7 | 0.5 |
| CA_2A-4A-12A | 2 | 0.5 |
|  | 4 | 0.5 |
|  | 12 | 0.8 |
| CA_2A-4A-12A-12A | 2 | 0.5 |
|  | 4 | 0.5 |
|  | 12 | 0.8 |
| CA_2A-4A-12B | 2 | 0.5 |
|  | 4 | 0.5 |
|  | 12 | 0.8 |
| CA_2A-4A-4A-12A | 2 | 0.5 |
|  | 4 | 0.5 |
|  | 12 | 0.8 |
| CA_2A-4A-13A | 2 | 0.5 |
|  | 4 | 0.5 |
|  | 13 | 0.3 |
| CA_2A-4A-28A | 2 | 0.5 |
|  | 4 | 0.5 |
|  | 28 | 0.8 |
| CA_2A-4A-29A | 2 | [0.5] |
|  | 4 | 0.5 |
| CA_2A-4A-30A | 2 | 0.5 |
|  | 4 | 0.5 |
|  | 30 | 0.3 |
| CA_2A-4A-71A | 2 | 0,5 |
|  | 4 | 0.5 |
|  | 71 | 0.3 |
| CA_2A-5A-12A | 2 | 0.3 |
|  | 5 | 0.8 |
|  | 12 | 0.4 |
| CA_2A-2A-5A-12A | 2 | 0.3 |
|  | 5 | 0.8 |
|  | 12 | 0.4 |
| CA_2A-5A-12A-12A | 2 | 0.3 |
|  | 5 | 0.8 |
|  | 12 | 0.4 |
| CA_2A-5A-46C | 2 | 0.3 |
|  | 5 | 0.3 |
| CA_2A-2A-5A-66A | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 66 | 0.5 |
| CA_2A-2A-5A-66A-66A | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 66 | 0.5 |
| CA_2A-2A-5A-66B | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 66 | 0.5 |
| CA_2A-2A-5A-66C | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 66 | 0.5 |
| CA_2A-2A-12B-66A | 2 | 0.5 |
|  | 12 | 0.8 |
|  | 66 | 0.5 |
| CA_2A-2A-13A-66A | 2 | 0.5 |
|  | 13 | 0.3 |
|  | 66 | 0.5 |
| CA_2A-5A-12B | 2 | 0.3 |
|  | 5 | 0.8 |
|  | 12 | 0.4 |
| CA_2A-5A-13A | 2 | 0.3 |
|  | 5 | 0.5 |
|  | 13 | 0.5 |
| CA_2A-5A-29A | 2 | 0.3 |
|  | 5 | 0.5 |
| CA_2A-5A-30A | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 30 | 0.3 |
| CA_2A-2A-5A-30A | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 30 | 0.3 |
| CA_2C-5B-30A | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 30 | 0.3 |
| CA_2C-5A-30A | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 30 | 0.3 |
| CA_2A-5B-30A | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 30 | 0.3 |
| CA_2A-5A-46A | 2 | 0.3 |
|  | 5 | 0.3 |
| CA_2A-5A-46D | 2 | 0.3 |
|  | 5 | 0.3 |
| CA_2A-5A-66A | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 66 | 0.5 |
| CA_2A-5A-66A-66A | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 66 | 0.5 |
| CA_2A-5A-66B | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 66 | 0.5 |
| CA_2A-5A-66C | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 66 | 0.5 |
| CA_2A-5A-66D | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 66 | 0.5 |
| CA_2A-5B-66A | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 66 | 0.5 |
| CA_2A-5B-66B | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 66 | 0.5 |
| CA_2A-5B-66C | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 66 | 0.5 |
| CA_2A-7A-12A | 2 | 0.5 |
|  | 7 | 0.5 |
|  | 12 | 0.3 |
| CA_2A-7A-66A | 2 | 0.5 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
| CA_2A-12A-30A | 2 | 0.5 |
|  | 12 | 0.3 |
|  | 30 | 0.3 |
| CA_2A-2A-12A-30A | 2 | 0.5 |
|  | 12 | 0.3 |
|  | 30 | 0.3 |
| CA_2C-12A-30A | 2 | 0.5 |
|  | 12 | 0.3 |
|  | 30 | 0.3 |
| CA_2A-12A-66A | 2 | 0.5 |
|  | 12 | 0.8 |
|  | 66 | 0.5 |
| CA_2A-2A-12A-66A | 2 | 0.5 |
|  | 12 | 0.8 |
|  | 66 | 0.5 |
| CA_2A-12A-66A-66A | 2 | 0.5 |
|  | 12 | 0.8 |
|  | 66 | 0.5 |
| CA_2A-12A-66C | 2 | 0.5 |
|  | 12 | 0.8 |
|  | 66 | 0.5 |
| CA_2A-12B-66A | 2 | 0.5 |
|  | 12 | 0.8 |
|  | 66 | 0.5 |
| CA_2A-12B-66A-66A | 2 | 0.5 |
|  | 12 | 0.8 |
|  | 66 | 0.5 |
| CA_2A-13A-46A | 2 | 0.3 |
|  | 13 | 0.3 |
| CA_2A-13A-46C | 2 | 0.3 |
|  | 13 | 0.3 |
| CA_2A-13A-46D | 2 | 0.3 |
|  | 13 | 0.3 |
| CA_2A-13A-66A | 2 | 0.5 |
|  | 13 | 0.3 |
|  | 66 | 0.5 |
| CA_2A-13A-66D | 2 | 0.5 |
|  | 13 | 0.3 |
|  | 66 | 0.5 |
| CA_2A-13A-66A-66A | 2 | 0.5 |
|  | 13 | 0.3 |
|  | 66 | 0.5 |
| CA_2A-13A-66A-66B | 2 | 0.5 |
|  | 13 | 0.3 |
|  | 66 | 0.5 |
| CA_2A-13A-66A-66C | 2 | 0.5 |
|  | 13 | 0.3 |
|  | 66 | 0.5 |
| CA_2A-13A-66B | 2 | 0.5 |
|  | 13 | 0.3 |
|  | 66 | 0.5 |
| CA_2A-13A-66C | 2 | 0.5 |
|  | 13 | 0.3 |
|  | 66 | 0.5 |
| CA_2A-14A-30A | 2 | 0.5 |
|  | 14 | 0.3 |
|  | 30 | 0.5 |
| CA_2A-2A-14A-30A | 2 | 0.5 |
|  | 14 | 0.3 |
|  | 30 | 0.5 |
| CA_2A-14A-66A | 2 | 0.5 |
|  | 14 | 0.3 |
|  | 66 | 0.5 |
| CA_2A-2A-14A-66A | 2 | 0.5 |
|  | 14 | 0.3 |
|  | 66 | 0.5 |
| CA_2A-14A-66A-66A | 2 | 0.5 |
|  | 14 | 0.3 |
|  | 66 | 0.5 |
| CA_2A-14A-66A-66A-66A | 2 | 0.5 |
|  | 14 | 0.3 |
|  | 66 | 0.5 |
| CA_2A-29A-30A | 2 | 0.5 |
|  | 30 | 0.3 |
| CA_2A-2A-29A-30A | 2 | 0.5 |
|  | 30 | 0.3 |
| CA_2C-29A-30A | 2 | 0.5 |
|  | 30 | 0.3 |
| CA_2A-29A-66A | 2 | 0.5 |
|  | 66 | 0.5 |
| CA_2A-2A-30A-66A | 2 | 0.5 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
| CA_2A-30A-66A | 2 | 0.5 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
| CA_2A-30A-66A-66A | 2 | 0.5 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
| CA_2A-46A-66A | 2 | 0.5 |
|  | 66 | 0.5 |
| CA_2A-46A-46A-66A | 2 | 0.5 |
|  | 66 | 0.5 |
| CA_2A-46C-66A | 2 | 0.5 |
|  | 66 | 0.5 |
| CA_2A-46A-46C-66A | 2 | 0.5 |
|  | 66 | 0.5 |
| CA_2A-46D-66A | 2 | 0.5 |
|  | 66 | 0.5 |
| CA_2A-48A-66A | 2 | 0.6 |
|  | 48 | 0.8 |
|  | 66 | 0.6 |
| CA_2A-48A-48A-66A | 2 | 0.6 |
|  | 48 | 0.8 |
|  | 66 | 0.6 |
| CA_2A-66A-71A | 2 | 0.5 |
|  | 66 | 0.5 |
|  | 71 | 0.3 |
| CA_3A-5A-7A | 3 | 0.5 |
|  | 5 | 0.3 |
|  | 7 | 0.5 |
| CA_3A-5A-7A-7A | 3 | 0.5 |
|  | 5 | 0.3 |
|  | 7 | 0.5 |
| CA_3A-5A-40A | 3 | 0.5 |
|  | 5 | 0.3 |
|  | 40 | 0.5 |
| CA_3A-5A-40A-40A | 3 | 0.5 |
|  | 5 | 0.3 |
|  | 40 | 0.5 |
| CA_3A-5A-41A | 3 | 0.5 |
|  | 5 | 0.318 |
|  | 41 | 0.35 |
|  |  | 0.86 |
| CA_3A-3A-7A-7A-8A | 3 | 0.5 |
|  | 7 | 0.5 |
|  | 8 | 0.6 |
| CA_3A-3A-7A-8A | 3 | 0.5 |
|  | 7 | 0.5 |
|  | 8 | 0.6 |
| CA_3A-7A-7A-8A | 3 | 0.5 |
|  | 7 | 0.5 |
|  | 8 | 0.6 |
| CA_3A-7A-8A | 3 | 0.5 |
|  | 7 | 0.5 |
|  | 8 | 0.6 |
| CA_3C-7A-8A | 3 | 0.5 |
|  | 7 | 0.5 |
|  | 8 | 0.6 |
| CA_3A-7A-20A | 3 | 0.5 |
|  | 7 | 0.5 |
|  | 20 | 0.3 |
| CA_3C-7A-20A | 3 | 0.5 |
|  | 7 | 0.5 |
|  | 20 | 0.3 |
| CA_3C-7C-20A | 3 | 0.5 |
|  | 7 | 0.5 |
|  | 20 | 0.3 |
| CA_3A-7C-20A | 3 | 0.5 |
|  | 7 | 0.5 |
|  | 20 | 0.3 |
| CA_3A-7A-26A | 3 | 0.5 |
|  | 7 | 0.5 |
|  | 26 | 0.3 |
| CA_3A-7A-7A-26A | 3 | 0.5 |
|  | 7 | 0.5 |
|  | 26 | 0.3 |
| CA_3A-7A-28A | 3 | 0.5 |
|  | 7 | 0.5 |
|  | 28 | 0.3 |
| CA_3A-7C-28A | 3 | 0.5 |
|  | 7 | 0.5 |
|  | 28 | 0.3 |
| CA_3C-7A-28A | 3 | 0.5 |
|  | 7 | 0.5 |
|  | 28 | 0.3 |
| CA_3C-7C-28A | 3 | 0.5 |
|  | 7 | 0.5 |
|  | 28 | 0.3 |
| CA_3A-7A-32A | 3 | 0.7 |
|  | 7 | 0.7 |
| CA_3C-7A-32A | 3 | 0.7 |
|  | 7 | 0.7 |
| CA_3A-7A-38A | 3 | 0.5 |
|  | 7 | 0.5 |
|  | 38 | 0.5 |
| CA_3C-7A-38A | 3 | 0.5 |
|  | 7 | 0.5 |
|  | 38 | 0.5 |
| CA_3A-7A-40A | 3 | 0.6 |
|  | 7 | 0.8 |
|  | 40 | 0.9 |
| CA_3A-7A-40C | 3 | 0.6 |
|  | 7 | 0.8 |
|  | 40 | 0.9 |
| CA_3A-7A-42A | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 42 | 0.8 |
| CA_3A-8A-11A | 3 | 0.8 |
|  | 8 | 0.3 |
|  | 11 | 0.9 |
| CA_3A-8A-20A | 3 | 0.3 |
|  | 8 | 0.4 |
|  | 20 | 0.4 |
| CA_3A-8A-28A12 | 3 | 0.3 |
|  | 8 | 0.6 |
|  | 28 | 0.5 |
| CA_3A-8A-32A | 3 | 0.8 |
|  | 8 | 0.3 |
| CA_3A-8A-40A | 3 | 0.5 |
|  | 8 | 0.3 |
|  | 40 | 0.5 |
| CA_3A-11A-28A | 3 | 0.8 |
|  | 11 | 0.9 |
|  | 28 | 0.6 |
| CA_3A-19A-21A | 3 | 0.8 |
|  | 19 | 0.3 |
|  | 21 | 0.9 |
| CA_3A-3A-19A-21A | 3 | 0.8 |
|  | 19 | 0.3 |
|  | 21 | 0.9 |
| CA_3A-19A-42A | 3 | 0.6 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
| CA_3A-19A-42C | 3 | 0.6 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
| CA_3A-20A-28A | 3 | 0.3 |
|  | 20 | 0.5 |
|  | 28 | 0.5 |
| CA_3C-20A-28A | 3 | 0.3 |
|  | 20 | 0.5 |
|  | 28 | 0.5 |
| CA_3A-20A-32A | 3 | 0.5 |
|  | 20 | 0.3 |
| CA_3A-20A-42A | 3 | 0.6 |
|  | 20 | 0.3 |
|  | 42 | 0.8 |
| CA_3A-21A-28A | 3 | 0.8 |
|  | 21 | 0.9 |
|  | 28 | 0.3 |
| CA_3A-21A-42A | 3 | 0.8 |
|  | 21 | 0.9 |
|  | 42 | 0.8 |
| CA_3A-21A-42C | 3 | 0.8 |
|  | 21 | 0.9 |
|  | 42 | 0.8 |
| CA_3A-28A-38A | 3 | 0.5 |
|  | 28 | 0.5 |
|  | 38 | 0.5 |
| CA_3C-28A-38A | 3 | 0.5 |
|  | 28 | 0.5 |
|  | 38 | 0.5 |
| CA_3A-28A-40A | 3 | 0.5 |
|  | 28 | 0.3 |
|  | 40 | 0.5 |
| CA_3A-28A-40C | 3 | 0.5 |
|  | 28 | 0.3 |
|  | 40 | 0.5 |
| CA_3A-28A-40D | 3 | 0.5 |
|  | 28 | 0.3 |
|  | 40 | 0.5 |
| CA_3A-28A-41A | 3 | 0.5 |
|  | 28 | 0.3 |
|  | 41 | 0.35/0.86 |
| CA_3A-28A-41C | 3 | 0.5 |
|  | 28 | 0.3 |
|  | 41 | 0.35/0.86 |
| CA_3A-28A-42A | 3 | 0.6 |
|  | 28 | 0.5 |
|  | 42 | 0.8 |
| CA_3A-28A-42C | 3 | 0.6 |
|  | 28 | 0.5 |
|  | 42 | 0.8 |
| CA_3A-41A-42A14 | 3 | 1 |
|  | 41 | 0.35/0.86 |
|  | 42 | 0.8 |
| CA_3A-41A-42C14 | 3 | 1 |
|  | 41 | 0.35/0.86 |
|  | 42 | 0.8 |
| CA_3A-41C-42A14 | 3 | 1 |
|  | 41 | 0.35/0.86 |
|  | 42 | 0.8 |
| CA_3A-41C-42C14 | 3 | 1 |
|  | 41 | 0.35/0.86 |
|  | 42 | 0.8 |
| CA_4A-5A-12A | 4 | 0.3 |
|  | 5 | 0.8 |
|  | 12 | 0.8 |
| CA_4A-5A-12A-12A | 4 | 0.3 |
|  | 5 | 0.8 |
|  | 12 | 0.8 |
| CA_4A-5A-12B | 4 | 0.3 |
|  | 5 | 0.8 |
|  | 12 | 0.8 |
| CA_4A-4A-5A-12A | 4 | 0.3 |
|  | 5 | 0.8 |
|  | 12 | 0.8 |
| CA_4A-5A-13A | 4 | 0.3 |
|  | 5 | 0.5 |
|  | 13 | 0.5 |
| CA_4A-5A-29A | 4 | 0.3 |
|  | 5 | 0.5 |
| CA_4A-5A-30A | 4 | 0.5 |
|  | 5 | 0.3 |
|  | 30 | 0.3 |
| CA_4A-4A-5A-30A | 4 | 0.5 |
|  | 5 | 0.3 |
|  | 30 | 0.3 |
| CA_4A-5B-30A | 4 | 0.5 |
|  | 5 | 0.3 |
|  | 30 | 0.3 |
| CA_4A-4A-5B-30A | 4 | 0.5 |
|  | 5 | 0.3 |
|  | 30 | 0.3 |
| CA_4A-7A-12A | 4 | 0.5 |
|  | 7 | 0.5 |
|  | 12 | 0.8 |
| CA_4A-12A-30A | 4 | 0.5 |
|  | 12 | 0.8 |
|  | 30 | 0.3 |
| CA_4A-4A-12A-30A | 4 | 0.5 |
|  | 12 | 0.8 |
|  | 30 | 0.3 |
| CA_4A-29A-30A | 4 | 0.5 |
|  | 30 | 0.3 |
| CA_4A-4A-29A-30A | 4 | 0.5 |
|  | 30 | 0.3 |
| CA_5A-7A-46A | 5 | 0.3 |
|  | 7 | 0.3 |
| CA_5A-7A-46C | 5 | 0.3 |
|  | 7 | 0.3 |
| CA_5A-7A-46D | 5 | 0.3 |
|  | 7 | 0.3 |
| CA_5A-12A-46A | 5 | 0.8 |
|  | 12 | 0.4 |
| CA_5A-12A-46C | 5 | 0.8 |
|  | 12 | 0.4 |
|  | 46 | 0 |
| CA_5A-12A-66A | 5 | 0.3 |
|  | 12 | 0.8 |
|  | 66 | 0.8 |
| CA_5A-12A-46D | 5 | 0.8 |
|  | 12 | 0.4 |
|  | 46 | 0 |
| CA_5A-30A-66A | 5 | 0.3 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
| CA_5A-30A-66A-66A | 5 | 0.3 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
| CA_5B-30A-66A | 5 | 0.3 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
| CA_5A-40A-41A | 5 | 0.3 |
|  | 40 | 0.5 |
|  | 41 | 0.5 |
| CA_5A-46A-66A | 5 | 0.3 |
|  | 66 | 0.3 |
| CA_5A-46D-66A | 5 | 0.3 |
|  | 66 | 0.3 |
| CA_7A-8A-20A | 7 | 0.3 |
|  | 8 | 0.6 |
|  | 20 | [0.6] |
| CA_7A-8A-38A | 7 | 0.5 |
|  | 8 | 0.5 |
|  | 38 | 0.5 |
| CA_7A-8A-40A | 7 | 0.5 |
|  | 8 | 0.6 |
|  | 40 | 0.6 |
| CA_7A-8A-40C | 7 | 0.5 |
|  | 8 | 0.6 |
|  | 40 | 0.6 |
| CA_7A-20A-28A | 7 | 0.3 |
|  | 20 | 0.6 |
|  | 28 | 0.6 |
| CA_7A-20A-32A | 7 | 0.7 |
|  | 20 | 0.3 |
|  | 32 | N/A |
| CA_7A-20A-38A | 7 | 0.3 |
|  | 20 | 0.3 |
|  | 38 | 0.3 |
| CA_7A-20A-42A | 7 | 0.3 |
|  | 20 | 0.3 |
|  | 42 | 0.8 |
| CA_7A-28A-38A | 7 | 0.3 |
|  | 28 | 0.3 |
|  | 38 | 0.3 |
| CA_8A-11A-28A17 | 8 | 0.6 |
|  | 11 | 0.4 |
|  | 28 | 0.6 |
| CA_8A-20A-28A | 8 | 0.6 |
|  | 20 | 0.5 |
|  | 28 | 0.5 |
| CA_8A-28A-41A15 | 8 | 0.6 |
|  | 28 | 0.5 |
|  | 41 | 0.3 |
| CA_8A-39A-41A | 8 | 0.3 |
|  | 39 | 0.319 |
|  | 41 | 0.319 |
| CA_12A-30A-66A | 12 | 0.8 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
| CA_12A-30A-66A-66A | 12 | 0.8 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
| CA_13A-46A-66A | 13 | 0.3 |
|  | 66 | 0.3 |
| CA_13A-46C-66A | 13 | 0.3 |
|  | 66 | 0.3 |
| CA_13A-46D-66A | 13 | 0.3 |
|  | 66 | 0.3 |
| CA_13A-48A-66A | 13 | 0.3 |
|  | 48 | 0.8 |
|  | 66 | 0.6 |
| CA_13A-48A-48A-66A | 13 | 0.3 |
|  | 48 | 0.8 |
|  | 66 | 0.6 |
| CA_13A-48C-66A | 13 | 0.3 |
|  | 48 | 0.8 |
|  | 66 | 0.6 |
| CA_14A-30A-66A | 14 | 0.3 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
| CA_14A-30A-66A-66A | 14 | 0.3 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
| CA_19A-21A-42A | 19 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
| CA_19A-21A-42C | 19 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
| CA_20A-38A-40A | 20 | 0.3 |
|  | 38 | 0.3 |
|  | 40 | 0.3 |
| CA_20A-38A-40D9 | 20 | 0.3 |
|  | 38 | 0.3 |
|  | 40 | 0.3 |
| CA_21A-28A-42A | 21 | 0.4 |
|  | 28 | 0.5 |
|  | 42 | 0.8 |
| CA_21A-28A-42C | 21 | 0.4 |
|  | 28 | 0.5 |
|  | 42 | 0.8 |
| CA_20A-38A-40A-40A9 | 20 | 0.3 |
|  | 38 | 0.3 |
|  | 40 | 0.3 |
| CA_20A-38A-40C9 | 20 | 0.3 |
|  | 38 | 0.3 |
|  | 40 | 0.3 |
| CA_28A-41A-42A10 | 28 | 0.5 |
|  | 41 | 0.31 |
|  | 42 | 0.81 |
| CA_28A-41C-42C | 28 | 0.5 |
|  | 41 | 0.39 |
|  | 42 | 0.89 |
| CA_28A-41A-42C | 28 | 0.5 |
|  | 41 | 0.39 |
|  | 42 | 0.89 |
| CA_28A-41C-42A | 28 | 0.5 |
|  | 41 | 0.39 |
|  | 42 | 0.89 |
| CA_29A-30A-66A | 30 | 0.3 |
|  | 66 | 0.5 |
| CA_29A-46A-66A | 66 | 0.3 |
| CA_29A-66A-70A | 66 | 0.5 |
|  | 70 | 0.5 |
| CA_29A-66A-66A-70A | 66 | 0.5 |
|  | 70 | 0.5 |
| CA_29A-66A-70C | 66 | 0.5 |
|  | 70 | 0.5 |
| CA_29A-66A-66A-70C | 66 | 0.5 |
|  | 70 | 0.5 |
| CA_29A-66C-70A | 66 | 0.5 |
|  | 70 | 0.5 |
| CA_29A-66C-70C | 66 | 0.5 |
|  | 70 | 0.5 |
| CA_66A-70A-71A | 66 | 0.5 |
|  | 70 | 0.5 |
|  | 71 | 0.6 |
| CA_66C-70A-71A | 66 | 0.5 |
|  | 70 | 0.5 |
|  | 71 | 0.6 |
| CA_66A-70C-71A | 66 | 0.5 |
|  | 70 | 0.5 |
|  | 71 | 0.6 |
| CA_66A-66A-70A-71A | 66 | 0.5 |
|  | 70 | 0.5 |
|  | 71 | 0.6 |
| CA_66A-66A-70C-71A | 66 | 0.5 |
|  | 70 | 0.5 |
|  | 71 | 0.6 |
| CA_66C-70C-71A | 66 | 0.5 |
|  | 70 | 0.5 |
|  | 71 | 0.6 |
| NOTE 1: The above additional tolerances are only applicable for the E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations NOTE 2: The above additional tolerances also apply in non-aggregated operation for the supported E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations NOTE 3: Unless otherwise specified, in case the UE supports more than one of the above 3DL inter-band carrier aggregation configurations and a E-UTRA operating band belongs to more than one 3DL inter-band carrier aggregation configurations then: - When the E-UTRA operating band frequency range is ≤ 1GHz and the tolerances are the same, the value applies to the band. If the tolerances are different, the applicable additional 3DL tolerance is FFS. In case there is a harmonic relation between low band UL and high band DL, then the maximum tolerance among the different supported 3DL carrier aggregation configurations involving such band shall be applied - When the E-UTRA operating band frequency range is >1GHz, the applicable additional 3DL tolerance shall be the maximum tolerance above that applies for that operating band among the supported 3DL CA configurations NOTE 4: The above additional tolerances applicable for the E-UTRA operating bands that belong to the supported highest order inter-band carrier aggregation configuration, also applies to the same E-UTRA operating bands that belong to a supported lower order CA configuration. NOTE 5: The requirement is specified for the frequency range of 2545-2690MHz. NOTE 6: The requirement is specified for the frequency range of 2496-2545MHz. NOTE 7: For UE supporting E-UTRA band 65 and CA configurations including Band 1, the Band 65 ΔTIB,c is the max(Band 65 ΔTIB,c , Band 1 ΔTIB,c) NOTE 8: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 1 or Band 42. NOTE 9: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx on Band 41 and Band 42. NOTE 10: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx among TDD bands. NOTE 11: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 1 or Band 8 NOTE 12: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 3 or Band 8. NOTE 13: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx among TDD bands. NOTE 14: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx among TDD bands. NOTE 15: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 8 or Band 41. NOTE 16: For UE supporting E-UTRA band 42, 43 or 48 and CA configurations including Band 42, 43 or 48, the applicable ΔTIB,c in Band 42, 43, or 48 is the max(Band 42 ΔTIB,c , Band 43 ΔTIB,c, Band 48 ΔTIB,c). NOTE 17: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 8 or Band 11. NOTE 18: The values in the table reflect what can be achieved with the present state of the art technology. They shall be reconsidered when the state of the art technology progresses. NOTE19: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRAN band and without simultaneous Rx/Tx on band 39 and band 41 |  |  |

Table 6.2.5-4: ΔTIB,c (four bands)

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1A-3A-5A-7A | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 5 | 0.3 |
|  | 7 | 0.6 |
| CA_1A-3A-5A-7A-7A | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 5 | 0.3 |
|  | 7 | 0.6 |
| CA_1A-3A-5A-40A | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 5 | 0.3 |
|  | 40 | 0.5 |
| CA_1A-3A-5A-41A | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 5 | 0.39 |
|  | 41 | 0.57 |
|  |  | 0.88 |
| CA_1A-3A-7A-7A-26A | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 26 | 0.3 |
| CA_1A-3A-7A-8A | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 8 | 0.6 |
| CA_1A-3C-7A-8A | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 8 | 0.6 |
| CA_1A-3A-7A-20A | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 20 | 0.3 |
| CA_1A-3C-7A-20A | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 20 | 0.3 |
| CA_1A-3A-7A-26A | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 26 | 0.3 |
| CA_1A-3A-7A-28A | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 28 | 0.6 |
| CA_1A-3A-7C-28A | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 28 | 0.6 |
| CA_1A-3A-7A-40A | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.8 |
|  | 40 | 0.9 |
| CA_1A-3A-7A-40C | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.8 |
|  | 40 | 0.9 |
| CA_1A-3A-7A-42A | 1 | 0.7 |
|  | 3 | 0.7 |
|  | 7 | 0.7 |
|  | 42 | 0.8 |
| CA_1A-3A-8A-11A | 1 | 0.3 |
|  | 3 | 0.8 |
|  | 8 | 0.3 |
|  | 11 | 0.9 |
| CA_1A-3A-8A-28A10 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 8 | 0.6 |
|  | 28 | 0.6 |
| CA_1A-3A-11A-28A | 1 | 0.3 |
|  | 3 | 0.8 |
|  | 11 | 0.9 |
|  | 28 | 0.6 |
| CA_1A-3A-8A-40A | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 8 | 0.3 |
|  | 40 | 0.5 |
| CA_1A-3A-19A-21A | 1 | 0.3 |
|  | 3 | 0.8 |
|  | 19 | 0.3 |
|  | 21 | 0.9 |
| CA_1A-3A-19A-42A | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
| CA_1A-3A-3A-19A-21A | 1 | 0.3 |
|  | 3 | 0.8 |
|  | 19 | 0.3 |
|  | 21 | 0.9 |
| CA_1A-3A-19A-42C | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
| CA_1A-3A-20A-28A | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 20 | 0.6 |
|  | 28 | 0.6 |
| CA_1A-3A-20A-42A | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 20 | 0.3 |
|  | 42 | 0.8 |
| CA_1A-3A-21A-28A | 1 | 0.3 |
|  | 3 | 0.8 |
|  | 21 | 0.9 |
|  | 28 | 0.6 |
| CA_1A-3A-21A-42A | 1 | 0.6 |
|  | 3 | 0.8 |
|  | 21 | 0.9 |
|  | 42 | 0.8 |
| CA_1A-3A-21A-42C | 1 | 0.6 |
|  | 3 | 0.8 |
|  | 21 | 0.9 |
|  | 42 | 0.8 |
| CA_1A-3A-28A-42A | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 28 | 0.6 |
|  | 42 | 0.8 |
| CA_1A-3A-28A-42C | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 28 | 0.6 |
|  | 42 | 0.8 |
| CA_1A-7A-8A-20A | 1 | 0.5 |
|  | 7 | 0.6 |
|  | 8 | 0.6 |
|  | 20 | 0.6 |
| CA_1A-21A-28A-42A | 1 | 0.3 |
|  | 21 | 0.4 |
|  | 28 | 0.6 |
|  | 42 | 0.8 |
| CA_1A-5A-7A-46A | 1 | 0.5 |
|  | 5 | 0.3 |
|  | 7 | 0.6 |
| CA_1A-5A-7A-46C | 1 | 0.5 |
|  | 5 | 0.3 |
|  | 7 | 0.6 |
| CA_1A-7A-20A-28A | 1 | 0.5 |
|  | 7 | 0.6 |
|  | 20 | 0.6 |
|  | 28 | 0.6 |
| CA_1A-7A-20A-32A | 1 | 0.7 |
|  | 7 | 0.7 |
|  | 20 | 0.3 |
| CA_1A-7A-20A-42A | 1 | 0.6 |
|  | 7 | 0.7 |
|  | 20 | 0.4 |
|  | 42 | 0.8 |
| CA_1A-8A-11A-28A11 | 1 | 0.3 |
|  | 8 | 0.6 |
|  | 11 | 0.4 |
|  | 28 | 0.6 |
| CA_1A-19A-21A-42A | 1 | 0.3 |
|  | 19 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
| CA_1A-19A-21A-42C | 1 | 0.3 |
|  | 19 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
| CA_1A-21A-28A-42C | 1 | 0.3 |
|  | 21 | 0.4 |
|  | 28 | 0.6 |
|  | 42 | 0.8 |
| CA_2A-2A-5A-12A-66A | 2 | 0.5 |
|  | 5 | 0.8 |
|  | 12 | 0.8 |
|  | 66 | 0.5 |
| CA_2A-2A-5A-30A-66A | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
| CA_2A-2A-12A-30A-66A | 2 | 0.5 |
|  | 12 | 0.8 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
| CA_2A-2A-14A-30A-66A | 2 | 0.5 |
|  | 14 | 0.3 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
| CA_2A-4A-5A-12A | 2 | 0.5 |
|  | 4 | 0.5 |
|  | 5 | 0.8 |
|  | 12 | 0.8 |
| CA_2A-4A-5A-29A | 2 | 0.5 |
|  | 4 | 0.5 |
|  | 5 | 0.5 |
| CA_2A-4A-5A-30A | 2 | 0.5 |
|  | 4 | 0.5 |
|  | 5 | 0.3 |
|  | 30 | 0.3 |
| CA_2A-4A-5B-30A | 2 | 0.5 |
|  | 4 | 0.5 |
|  | 5 | 0.3 |
|  | 30 | 0.3 |
| CA_2A-4A-7A-12A | 2 | 0.5 |
|  | 4 | 0.5 |
|  | 7 | 0.5 |
|  | 12 | 0.8 |
| CA_2A-4A-12A-30A | 2 | 0.5 |
|  | 4 | 0.5 |
|  | 12 | 0.8 |
|  | 30 | 0.3 |
| CA_2A-14A-30A-66A | 2 | 0.5 |
|  | 14 | 0.3 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
| CA_2A-4A-29A-30A | 2 | 0.5 |
|  | 4 | 0.5 |
|  | 30 | 0.3 |
| CA_2A-5A-12A-66A | 2 | 0.5 |
|  | 5 | 0.8 |
|  | 12 | 0.8 |
|  | 66 | 0.5 |
| CA_2A-5A-30A-66A | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
| CA_2A-5A-30A-66A-66A | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
| CA_2A-5B-30A-66A | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
| CA_2A-12A-30A-66A | 2 | 0.5 |
|  | 12 | 0.8 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
| CA_2A-14A-30A-66A-66A | 2 | 0.5 |
|  | 14 | 0.3 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
| CA_3A-7A-8A-20A | 3 | 0.5 |
|  | 7 | 0.5 |
|  | 8 | 0.4 |
|  | 20 | 0.4 |
| CA_3A-7A-20A-28A | 3 | 0.5 |
|  | 7 | 0.5 |
|  | 20 | 0.6 |
|  | 28 | 0.5 |
| CA_3C-7A-20A-28A | 3 | 0.5 |
|  | 7 | 0.5 |
|  | 20 | 0.6 |
|  | 28 | 0.5 |
| CA_3A-7A-20A-32A | 3 | 0.7 |
|  | 7 | 0.7 |
|  | 20 | 0.3 |
| CA_3A-7A-20A-42A | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 20 | 0.3 |
|  | 42 | 0.8 |
| CA_3A-8A-11A-28A12 | 3 | 0.8 |
|  | 8 | 0.6 |
|  | 11 | 0.9 |
|  | 28 | 0.6 |
| CA_3A-19A-21A-42A | 3 | 0.8 |
|  | 19 | 0.3 |
|  | 21 | 0.9 |
|  | 42 | 0.8 |
| CA_3A-28A-41A-42A | 3 | 1 |
|  | 28 | 0.5 |
|  | 41 | 0.37/0.88 |
|  | 42 | 0.8 |
| CA_3A-28A-41A-42C | 3 | 1 |
|  | 28 | 0.5 |
|  | 41 | 0.37/0.88 |
|  | 42 | 0.8 |
| CA_3A-28A-41C-42A | 3 | 1 |
|  | 28 | 0.5 |
|  | 41 | 0.37/0.88 |
|  | 42 | 0.8 |
| NOTE 1: The above additional tolerances are only applicable for the E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations. NOTE 2: The above additional tolerances also apply in non-aggregated operation for the supported E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations. NOTE 3: Tolerances for a UE supporting multiple 4DL inter-band CA configurations are FFS. NOTE 4: The above additional tolerances applicable for the E-UTRA operating bands that belong to the supported highest order inter-band carrier aggregation configuration, also applies to the same E-UTRA operating bands that belong to a supported lower order CA configuration. NOTE 5: For UE supporting E-UTRA band 65 and CA configurations including Band 1, the Band 65 ΔTIB,c is the max(Band 65 ΔTIB,c , Band 1 ΔTIB,c). NOTE 6: For UE supporting E-UTRA band 42, 43 or 48 and CA configurations including Band 42, 43 or 48, the applicable ΔTIB,c in Band 42, 43, or 48 is the max(Band 42 ΔTIB,c , Band 43 ΔTIB,c, Band 48 ΔTIB,c). NOTE 7: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 8: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. NOTE 9: The values in the table reflect what can be achieved with the present state of the art technology. They shall be reconsidered when the state of the art technology progresses. NOTE 10: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 1, Band 3 or Band 8. NOTE 11: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 1, Band 8 or Band 11. NOTE 12: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 3, Band 8 or Band 11. |  |  |

Table 6.2.5-5: ΔTIB,c (five bands)

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1A-3A-7A-20A-28A | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 20 | 0.6 |
|  | 28 | 0.6 |
| CA_1A-3A-7A-20A-42A | 1 | 0.7 |
|  | 3 | 0.7 |
|  | 7 | 0.7 |
|  | 20 | 0.3 |
|  | 42 | 0.8 |
| CA_1A-3A-8A-11A-28A | 1 | 0.3 |
|  | 3 | 0.8 |
|  | 8 | 0.6 |
|  | 11 | 0.9 |
|  | 28 | 0.6 |
| NOTE 1: The above additional tolerances are only applicable for the E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations. NOTE 2: The above additional tolerances also apply in non-aggregated operation for the supported E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations. NOTE 3: Tolerances for a UE supporting multiple 5DL inter-band CA configurations are FFS. NOTE 4: The above additional tolerances applicable for the E-UTRA operating bands that belong to the supported highest order inter-band carrier aggregation configuration, also applies to the same E-UTRA operating bands that belong to a supported lower order CA configuration. |  |  |

NOTE:	The above additional tolerances do not apply to supported UTRA operating bands with frequency range below 1 GHz that correspond to the E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations when such bands are belonging only to band combination(s) where one band is <1GHz and other bands are >1.7GHz and there is no harmonic relationship between the low band UL and high band DL. Otherwise the above additional tolerances also apply to supported UTRA operating bands that correspond to the E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations.

### ----------------------------------------End of Change 2-------------------------------