3GPP TSG-WG RAN4 Meeting #86								R4-1802822

Athens, Greece, 26th February – 2nd March 2018

Title:	[draft] LS reply on OFDM symbol generation

Release:	Rel-15

Work Item:	NR_newRAT

Source:	RAN4

To:	RAN1

Cc:

Contact Person:

#### Name:	Hisashi Onozawa

E-mail Address:	hisashi.onozawa@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN4 thanks RAN1 for the LS on OFDM symbol generation (R1-1801279/ R4-1801415). For the frequency bands that support a channel raster of 100 kHz, RAN4 has agreed the default value of the frequency offset associated with the value M is 5kHz. It is 10kHz for the frequency ranges supporting 30kHz SS Block SCS, i.e., 824-849 MHz and 1710-1850 MHz.

Accordingly, GSCN parameters for the global frequency raster is revised in the following.

# GSCN parameters for the global frequency raster

| Frequency range | SS block frequency position SSREF | GSCN | Range of GSCN |
| --- | --- | --- | --- |
| 0 – 2700 MHz | N * 900kHz + M * Δsync kHz N=1:3000, M=-1:1 | 3N + M - 1 | 1 – 8999 |
| 2400 – 24250 MHz | 2400 MHz + N * 1.44 MHz N= 0:15173 | 9000 + N | 9000 – 24173 |
| 24250 – 100000 MHz | 24250.08 MHz + N * 17.28 MHz N= 0:4383 | 24174+ N | 24174– 28557 |
| Note 1: Δsync = 10 for Frequency ranges 869-894 MHz and 2110-2200 MHz. Otherwise Δsync = 5. |  |  |  |

2. Actions:

To RAN1:

ACTION: 	RAN4 asks RAN1 to progress on the OFDM symbol generation taking the above RAN4 decision into account.

3. Date of Next TSG-RAN WG1 Meetings:

RAN4#86bis	16th – 20th April 2018		Melbourne, Australia.

RAN4#87	21st – 25th May 2018 		Busan, Korea (Republic of)