3GPP TSG-RAN WG4 #86	                                                  R4-1802868

Athens, Greece 26 February – 2 March 2018

Source:	Sony

Title:	UE Spherical coverage at mmWave 28GHz

Agenda item:	7.4.9.3

Document for:	Discussion

# Background

The impact on spherical coverage of a realistic design has been discussed for several meetings. In RAN4-AH-1801 in San Diego detailed simulation assumptions were agreed in WF on spherical coverage [1]. This agreement was an update on a similar WF [2] from RAN4 #85 in Reno. Another WF on power class was agreed in [3] addressing the peak EIRP.

This discussion paper is an update of [4]. In [4] the impact of glass and plastic back cover on spherical coverage was studied for dipole edge antennas and patch antennas using a simplified handheld UE layout model. In this paper the study is extended with a more complete handheld UE (smartphone) model. More cases (assumptions) are studied, however, this time only including patch antennas.

# Simulation set-up

Spherical coverage is simulated for all 7 assumptions in [1]. These assumptions are copied in Table 1. In addition assumption 6* and assumption 7* including “Partial” displays are added. Partial display, in our simulation, means there is 5mm gap in top (and bottom) and thus one antenna array is placed in the front and the two others in the back compared to assumptions 6 and 7 where there are no room for antennas in the front and thus all 3 antennas are placed on the backside. This is schematically shown in Figure 1. The simulations are done for a single frequency but valid for the complete frequency band since only relative results ( EIRP @ %-tile) are discussed.

|  |  | Assumptions1 | Assumptions2 | Assumptions3 | Assumptions4 | Assumptions5 | Assumptions6 | Assumptions7 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency range |  | n257 | n257 | n257 | n257 | N257 | n257 | N257 |  |
| # of antenna in an antenna module/set (# of patches, # of dipoles, etc.) |  | - | - | - | - | - | - | - | Depends on the current implementation |
| # of antenna module/set in total |  | 1 | 1 | 2 | 2 | 2 | 3 | 3 |  |
| Finite UV test points | Y/N | Y | Y | Y | Y | Y | Y | Y | Finite test point shall be the baseline |
| Beam phase shifter controller | degree | 45 | 45 | 45 | 45 | 45 | 45 | 45 | Finite beam shall be the baseline |
| Antenna type (patch, dipole, or both) |  | - | - | - | - | - | - | - | Depends on the current implementation |
| Antenna module/set location (front, back, top-side, left-side, right-side, bottom-side) |  | Top / Bottom | Top / Bottom | Top & Bottom | Top & Bottom | Top & Bottom | Left & Right & Bottom | Left & Right & Bottom | Combination of the lists are not precluded. |
| Front cover (Plastic, Glass, Ceramic, Metal) |  | Glass | Glass | Glass | Glass | Glass | Glass | Glass | This information is meaningful only if it’s the same with the material which covers antennas. |
| Back cover (Plastic, Glass, Ceramic, Metal) |  | Glass | Plastic | Glass | Glass | Plastic | Glass | Plastic |  |
| Side cover / Frame (Plastic, Glass, Ceramic, Metal) |  | Metal | Plastic | Metal | Metal | Plastic | Metal | Plastic |  |
| Device size (WxHxD) | cm3 | 66.6 | 66.6 | 66.6 | 66.6 | 66.6 | 66.6 | 66.6 | This is for information |
| Display panel – Full (Y) or Partial (N) | Y/N | Y | Y | Y | N | Y | Y | Y |  |
| Bezel Margin | mm | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 | Module can’t be placed outer edge of UE to secure mechanical reliability |

Table 1. Simulations assumption from “WF on EIRP CDF for spherical coverage” [1].

Figure 1. Schematic picture of Assumption 6* (and 7*)

## UE layout model

In [4] a simplified model of a handheld UE (smartphone) was used. In this simulation CAD-files of an existing smartphone have been extracted into the full wave electromagnetic simulator. The display size has been modified to accommodate the different assumptions. In all simulations the antennas are general patches on Rogers RO4003C™ laminate. A summary of the layer properties according to [1] is given in Table 2.

| Layer Properties |  |  |  |  |
| --- | --- | --- | --- | --- |
| Name | Material | Thickness | Ɛr | tan δ |
| Antenna substrate Dielectrics | Rogers RO4003C™ | 0.3mm |  |  |
| Antenna Substrate Metals | copper | - | conductivity |  |
| Gap 1 | Air/Plastic/? | ~2-3mm |  |  |
| Gap 2 | Air/Plastic/? | 2.8mm |  |  |
| Gap 3 | Air/Plastic/? | G3 |  |  |
| Gap 4 | Air/Plastic/? | 1.5mm | Air |  |
| Edge material 1 | Glass/Plastic/Metal | W1 | Al |  |
| Edge material 2 | Glass/Plastic/Metal | 0.77mm | 6.98 | 0.03 |
| Edge material 3 | Glass/Plastic/Metal | W3 | Al |  |
| Edge material 4 | Glass/Plastic/Metal | 0.77mm | 6.98 | 0.03 |

Table 2. Summary of layer properties used in the simulations

# Result

## Impact of Back Cover Material – Glass vs. Plastic

Glass versus plastic back cover material is shown in Figure 2 in the 3-antenna array example (assumption 6 vs. assumption 7). Note that all 3 antennas are mounted in the back.

It can be seen that the peak value is only affected marginally but the lower percentiles are degraded substantially by the high loss glass material compared to plastic. The same trend is seen for other number of antennas (e.g. assumption 1 vs. 2 or assumption 3 vs. 5)

Observation 1: Implementation loss due to high-loss glass is 1 – 2 dB higher compared to plastic cover material.

Figure 2. Impact of Back Cover Material.

## Number of Antenna Arrays

The number of antenna arrays are studied in Figure 3 for both plastic and glass back cover. Note that all antenna arrays are in the back in this comparison. The following observations are done:

Observation 2: There is no significant difference in free space between 2 and 3 arrays when facing in the same direction. Single antenna has ~1dB worse spherical performance.

Observation 3: The same trend is seen regardless of plastic or glass cover.

Observation 4: Hand and body blockage could possibly motivate higher number of antenna arrays.

Figure 3. Number of Antenna Arrays. a. Plastic back cover. b. Glass back cover.

## Single-side Antennas Arrays vs. 2-side Arrays

Single-side antennas arrays (backside only) versus antenna arrays on both front and back is studied in Figure 4 leading to:

Observation 5: There is a significant improvement in spherical coverage with front + back antenna arrays vs. back-side arrays only.

Figure 4. Single-side antennas arrays versus 2-side arrays

## Summary

All simulated assumptions are summarized in Figure 5 and Table 3. Note that  EIRP @ %-tile in Table 3 refers to the 99.9%-tile point which is dependent on assumption (~1dB difference).

Figure 5. Spherical coverage as CDF simulated on a realistic handheld UE (smartphone) layout.

| Assumption | - | 3GPP 1 | 3GPP 2 | 3GPP 3 | 3GPP 4 | 3GPP 5 | 3GPP 6 | 6* | 3GPP 7 | 7* |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Display | Partial / Full | F | F | F | P | F | F | P | F | P |
| No antenna arrays | Back side | 1 | 1 | 2 | 1 | 2 | 3 | 2 | 3 | 2 |
|  | Front side |  |  |  | 1 |  |  | 1 |  | 1 |
| Side cover (cover material near antennas) | Metal / Plastic | M | M | M | M | M | M | M | M | M |
| Back cover | Glass / Plastic | G | P | G | G | P | G | G | P | P |
| Front cover | Glass / Plastic | G | G | G | G | G | G | G | G | G |
| EIRP @ 100%%-tile point | dBm |  |  |  |  |  |  |  |  |  |
| EIRP @ 50% | dB | 13 | 10 | 13 | 9 | 10 | 12 | 7 | 10 | 7 |
| EIRP @ 40% | dB | 15 | 12 | 14 | 10 | 11 | 14 | 9 | 11 | 8 |
| EIRP @ 30% | dB | 16 | 14 | 16 | 12 | 13 | 16 | 10 | 13 | 9 |
| EIRP @ 20% | dB | 18 | 17 | 18 | 14 | 15 | 17 | 11 | 14 | 10 |

Table 3. Summary of simulated spherical coverage for a realistic handheld UE (smartphone) layout.

# Conclusion

The spherical coverage of a handheld UE was simulated using CAD-files from an existing smartphone modified with patch antenna arrays for 28GHz. The following observations were made:

Observation 1: 	Implementation loss due to high-loss glass is 1 – 2 dB higher compared to plastic cover material.

Observation 2: 	There is no significant difference in free space between 2 and 3 arrays when facing in the same direction. Single antenna has ~1dB worse spherical performance.

Observation 3: 	The same trend is seen regardless of plastic or glass cover.

Observation 4: 	Hand and body blockage could possibly motivate higher number of antenna arrays.

Observation 5: 	There is a significant improvement in spherical coverage with front + back antenna arrays vs. back-side arrays only.

# References

- R4-1801202	“WF on EIRP CDF for spherical coverage”, Samsung MediaTek

- R4-1714355	“WF on spherical coverage in FR2”, Samsung, Apple, LGE, Intel, Oppo, Xiaomi, Vivo, MediaTek

- R4-1801106	“WF peak EIRP requirement for mmW”, Qualcomm

- R4-1800888 	“UE Spherical coverage at mmWave 28GHz”, Sony, Ericsson

Refer to separate discussion